Purchase Description (PD)

FOR

Joint Light Tactical Vehicle (JLTV)

Family of Vehicles

VERSION 2.0

21 January 2008

# 1 SCOPE.

This ATPD identifies the physical, performance and inspection requirements for the Joint Light Tactical Vehicle (JLTV) Family of Vehicles (FoV) and the companion trailers. The ATPD establishes these requirements by identifying the following:

a. Physical Characteristics

b. Performance Requirements

c. Test Requirements

## 1.1 General Description.

The JLTV FoV is composed of three Payload Category vehicles, A, B, and C. The performance, force protection and physical characteristics of these Payload Category vehicles have been tailored to fulfill mission roles they will be required to execute.  Each of the Payload Categories will have the capability of towing JLTV Companion Trailers as well as specified legacy trailers. The Payload Category Vehicles are defined as:

a. Payload Category A (JLTV-A): The JLTV-A will serve Battlespace Awareness (BA) mission roles by providing protected, networked mobility for USMC and USA general command and control purposes.

b. Payload Category B (JLTV-B): The JLTV-B will serve Force Application (FA) mission roles by providing protected, sustained and networked tactical ground mobility for mounted infantry/combat arms forces.

c. Payload Category C (JLTV-C): The JLTV-C will serve Focused Logistics (FL) mission roles by providing transport of wounded personnel, general cargo, ammunition and shelters.

d. Companion Trailers (JLTV-T): The companion trailers will provide addition payload carrying capacity commensurate with the specific Payload Category vehicles.

## 1.2

The ATPD is divided into a Main Body supplemented by Annexes. The Main Body of the ATPD defines overarching requirements that are applicable to the JLTV FoV and companion trailers.  Specific Payload Category vehicle requirements and classified requirements are contained in the annexes. The outline of the ATPD is as follows:

a. Annex A - Payload Category A (JLTV-A)

b. Annex B - Payload Category B (JLTV-B)

c. Annex C - Payload Category C (JLTV-C)

d. Annex D - Companion Trailer (JLTV -T)

e. Annex E - Force Protection (Classified)

f. Annex F - Signature Management (Classified)

g. Annex G - Export Controlled Annex (FOUO)

h. Annex H - Operational Mode Summary/Mission Profile (OMS/MP)

i. Annex I - Reserved

j. Annex J - Engineering Drawings

k. Annex K – Item Quantities

# 2 APPLICABLE DOCUMENTS

## 2.1 Government documents.

The following specifications, standards, and handbooks form a part of this document to the extent specified herein. Unless otherwise specified, the issue of these documents are those listed in the issue of the Department of Defense Index of Specifications and Standards (DODISS) and in effect on the date of Request for Proposal. 

MILITARY SPECIFICATIONS

DEPARTMENT OF DEFENSE:

| A-A-50271 | Plate, Identification |
| --- | --- |
| A-A-52418 | Light, Warning, Vehicular: Rotating, Unit, 14 and 28 Volt DC |
| A-A-52432 | Mirror Assembly, Rearview: Automotive Exterior Mounting |
| A-A-52474 | Electro coating Primer |
| A-A-52507 | Chain Assembly and Cross Chain, Tire: For Military Vehicles |
| A-A-52513 | Bracket Assembly, Liquid Container, Five Gallon |
| A-A-52525 | Horns and Buzzers, Air- and Electrically-Actuated |
| A-A-52557 | Fuel Oil, Diesel; For Posts, Camps and Stations |
| A-A-52624 | Antifreeze, Multi-Engine Type |
| A-A-59326 | Coupling Halves, Quick-Disconnect, Cam-Locking Type |
| A-A-59487 | Padlock (Key Operated) |
| MIL-PRF-2104 | Lubricating Oil, Internal Combustion Engine, Combat/Tactical Service |
| MIL-PRF-2105 | Lubricating Oil, Gear, Multipurpose (Metric) |
| MIL-PRF-10924 | Grease, Automotive And Artillery |
| MIL-PRF-20696 | Cloth, Waterproof, Weather Resistant |
| MIL-S-40626 | Sign Kit, Vehicle Class |
| MIL-PRF-46167 | Lubricating Oil, Internal Combustion Engine, Arctic |
| MIL-PRF-52308 | Filter-Coalescer Element, Fluid Pressure |
| MIL-C-53072 | Chemical Agent Resistant Coating (CARC) System Application Procedures and Quality Control Inspection |
| MIL-P-53084 | Primer, Cathodic Electrodeposition, Chemical Agent Resistant |
| MIL-PRF-62048 | Air Cleaners, Automotive: Heavy Duty, Dry-Type (For Internal Combustion Engines) (Metric) |
| MIL-DTL-83133 | Turbine Fuels, Aviation, Kerosene Types, NATO F-34 (JP-8), NATO F-35, and JP-8+100 |
| MIL-V-81940 | Valve, Sampling and Bleed, Hydraulic, Type II Systems |

STANDARDS

FEDERAL

| FED-STD-595 | Colors Used in Government Procurement |
| --- | --- |

DEPARTMENT OF DEFENSE

| MIL-STD-129 | Standard Practice For Military Marking |
| --- | --- |
| MIL-STD-209 | Lifting and Tie-down Provisions |
| MIL-STD-461 | Requirements for the Control of Electromagnetic Interference Characteristics of Subsystems and Equipment |
| MIL-STD-461E Interim Notice 4 | Measurement of Electromagnetic Interference Characteristics |
| MIL-STD-461E Interim Notice 5 | Measurement of Electromagnetic Interference Characteristics |
| MIL-STD-704 | Aircraft Electric Power Characteristics |
| MIL-STD-810 | Environmental Engineering Considerations and Laboratory Tests |
| MIL-STD-889 | Dissimilar Metals |
| MIL-STD-1275 | Characteristics of 28 Volt DC Electrical Systems in Military Vehicles |
| MIL-STD-1464 | Electromagnetic Environmental Effects Requirements for Systems |
| MIL-STD-1366 | Transportability Criteria |
| MIL-STD-1472 | Human Engineering |
| MIL-STD-1474 | Noise Limits |
| MIL-STD-2169 | High Altitude Electromagnetic Pulse |
| SAE J318 | ABS brake malfunction light |
| SAE J2497 | ABS brake intervehicular cable |
| MIL-DTL-12468 | Super Tropical Bleach |
| DODI-6055.11 | Electromagnetic Radiation Protection |

HANDBOOKS

DEPARTMENT OF DEFENSE

| MIL-HDBK-454 | General Guidelines for Electronic Equipment |
| --- | --- |
| MIL-HDBK-1791 | Designing for Internal Aerial Delivery in Fixed Wing Aircraft |

(Unless otherwise indicated, copies of the above specifications, standards, and handbooks are available from the Standardization Document Order Desk, 700 Robbins Avenue, Building 4D, Philadelphia PA 19111-5094.)

## 2.2 Other Government documents, drawings, and publications.

The following other Government documents, drawings, and publications form a part of this document to the extent specified herein. Unless otherwise specified, the issues are those cited in the solicitation.

Training and Doctrine Command Pamphlet 525-3-90/O&O(Change 2), The United States Army Objective Force Operational and Organizational Plan Maneuver Unit of Action, 30 June 2003.

TRADOC Pamphlet 525-4-0, US Army Concept for Maneuver Sustainment Operations in Support of the Objective Force (Draft), 23 Jan 03 U.S. Army White Paper: Concepts for the Objective Force

(Application for copies should be addressed to the U.S. Army Tank automotive and Armament Command, ATTN: AMSTA-LC-AH, Warren, MI 48397-5000)

C-130 Transportability of Army Vehicles, Military Traffic Management Command Transportation Engineering Agency, 11 Sept. 02, Joseph Cassidy.

US ARMY EDGEWOOD RESEARCH DEVELOPMENT AND ENGINEERING CENTER 
D5-15-8779 Interface for M-8 Alarm

Application for copies should be addressed to the: Technical Director, US Army Edgewood Research Development and Engineering Center, ATTN: SCBRD-RT/ASM, Aberdeen Proving Ground, MD 21010-5423)

US ARMY COMMUNICATIONS ELECTRONICS MATERIEL READINESS COMMAND

| A3013814 | SINCGARS AN/VRC-90 Radio Set |
| --- | --- |
| A3013842 | Antenna (AS-3684) |
| A3014039 | Power Cable |
| A3019214 | Mounting Base, Electrical Equipment For The MT6352/VRC-VEC |
| SCD189023 | Antenna Support Assembly |

(Application for copies should be addressed to the: US Army Communications and Electronics Materiel Readiness Command, Logistics Engineering Directorate, 12WD Bldg. 601 McAfee Center, Fort Monmouth, NJ 07703)

TECHNICAL BULLETIN (TB)

U. S. ARMY TANK-AUTOMOTIVE AND ARMAMENT COMMAND

| TB 43-0213 | Corrosion, Prevention and Control Including Rust proofing Procedures for Tactical Vehicles and Trailers |
| --- | --- |

(Application for copies should be addressed to the U.S. Army Tank automotive and Armament Command, ATTN: AMSTA-LC-AH, Warren, MI 48397-5000)

REGULATIONS, ARMY

| AR 70-38 | Research, Development, Test and Evaluation of Materiel for Extreme Climatic Conditions |
| --- | --- |

(Copies are available from the following website: http://www.usace.army.mil/inet/usace-docs/army-reg)

GOVERNMENT AGENCIES

CALIFORNIA AIR RESOURCES BOARD (CARB)

Tank Pressure and Vacuum Requirements

(Application for copies should be addressed to the: California Air Resources Board, 2020 L Street, Sacramento, CA 95814)

DEPARTMENT OF TRANSPORTATION (DOT)

Federal Motor Vehicle Safety Standards (FMVSS)

| 178.346 | Cargo Tank Motor Vehicle (DOT 406) |
| --- | --- |
| 571.101 | Controls and Displays |
| 571.102 | Transmission Shift Lever Sequence, Starter Interlock, and Transmission Braking Effect |
| 571.104 | Windshield Wiping and Washing Systems |
| 571.108 | Lamps, Reflective Devices, and Associated Equipment |
| 571.111 | Rearview Mirrors |
| 571.119 | New Pneumatic Tires for Vehicles other Than Passenger Cars |
| 571.120 | Tire Selection and Rims for Motor Vehicles Other Than Passenger Cars |
| 571.121 | Air Brake Systems |
| 571.124 | Accelerator Control Systems |
| 571.208 | Occupant Crash Protection |
| 571.209 | Seat Belt Assemblies |
| 571.210 | Seat Belt Assemblies Anchorage |
| 571.223 | Rear Impact Guards |
| 571.224 | Rear Impact Protection |

Federal Motor Carrier Safety Regulations (FMCSR)

| 393.27 | Wiring Specifications |
| --- | --- |
| 393.28 | Wiring to Be Protected |
| 393.29 | Grounds |
| 393.30 | Battery Installation |
| 393.31 | Overload Protection Devices |
| 393.32 | Detachable Electrical Connections |
| 393.33 | Installation Wiring, |
| 393.40 | Required Brake Systems |
| 393.41 | Parking Brake System |
| 393.42 | Brakes Required on All Wheels |
| 393.43 | Breakaway and Emergency Braking |
| 393.45 | Brake Tubing and Hose, Adequacy |
| 393.46 | Brake Tubing and Hose Connections |
| 393.47 | Brake Lining |
| 393.48 | Brakes to Be Operative |
| 393.49 | Single Valve to Operate All Brakes |
| 393.50 | Reservoirs Required |
| 393.51 | Warning Devices and Gauges |
| 393.52 | Brake Performance |
| 393.55 | Antilock Brake Systems |
| 393.65 | All Fuel Systems |
| 393.67 | Liquid Fuel Tanks |
| 393.70 | Coupling Devices and Towing Methods, Except for Driveaway-Towaway Operations |
| 393.83 | Exhaust Systems |
| 393.86 | Rear Impact Protection |
| 393.95 | Emergency Equipment on All Power Units |

(Application for copies should be addressed to the Dept of Transportation, Federal Highway Administrations, Washington, DC 20591)

ENVIRONMENTAL PROTECTION AGENCY (EPA)

Control of Air Pollution from New Motor Vehicles and New Motor Vehicle Engines

Compliance with Interstate Motor Carrier Noise Emission Standards

(Application for copies should reference "Code of Federal Regulations 40 CAR and the Federal Register, and should be addressed to the Superintendent of Documents, US Government Printing Office, Washington, DC 20402)

NATIONAL FIRE PROTECTION AGENCY (NFPA)

| NFPA 407 | Standard for Aircraft Fuel Servicing (National Fire Codes, Vol. 7) |
| --- | --- |
| NFPA 70 | National Electrical Code |

(Application for copies should be addressed to the National Fire Protection Agency, One Batterymarch Park, PO Box 9101, Quincy, MA 02269-9101)

NORTH ATLANTIC TREATY ORGANIZATION (NATO) STANDARDIZATION AGREEMENT (STANAG)

| STANAG 2413 | Demountable Load Carrying Platforms (DLCP/Flatracks) |
| --- | --- |
| STANAG 4007 | Electrical Connectors Between Prime Movers, Trailers and Towed Artillery |
| STANAG 4074 | Auxiliary Power Unit Connections for Starting Tactical Land Vehicles |
| STANAG 4569 | Protection levels for occupants of logistic and light armored vehicles |
| QSTAG 244 Ed 3 | Nuclear Survivability Requirements For Military Equipment |

NORTH ATLANTIC TREATY ORGANIZATION (NATO) ALLIED VEHICLE TESTING PUBLICATION (AVTP)

| AVTP 03-30WT | Steering and Maneuverability |
| --- | --- |
| AVTP 03-160 W | Dynamic Stability |

(Applicable NATO documents are those that are current at NATO Headquarters (Military Agency for Standardization , 1110 Brussels).  Copies are available from Global Engineering, Inc., 15 Inverness Way East, Englewood, CO  80112).

OCCUPATIONAL SAFETY & HEALTH ADMINISTRATION (OSHA)

| Title 29, CFR, Part 1910.1000 | Air Contaminants |
| --- | --- |

(Application for copies should be addressed to the American Conference of Government Industrial Hygienists (ACGIH), 1330 Kemper Meadow Drive, Cincinnati, OH 45240).

## 2.3 Non-Government Publications.

The following documents form a part of this document to the extent specified herein.  Unless otherwise specified, the issues of the documents which are DoD adopted are those listed in the issue of the DoDISS cited in the solicitation. Unless otherwise specified, the issues of documents not listed in the DoDISS are the issues of the documents cited in the solicitation (see 6.2).

AMERICAN PETROLEUM INSTITUTE (API)

| API STD 1529 | Aviation Fueling Hose (DOD Adopted) |
| --- | --- |
| API SPEC 1581 | Specifications and Qualification Procedures for Aviation Jet Fuel Filter/Separators |

(Application for copies should be made to the: American Petroleum Institute, 1220 L St NW, Washington, DC  200005 )

AMERICAN SOCIETY OF MECHANICAL ENGINEERS (ASME)

ASME B30.22 Articulating Boom Cranes 01JAN00

(Applications for copies should be addressed to the: American Society of Mechanical Engineers, 345 East 47th Street, New York, NY  10017)

AMERICAN SOCIETY FOR TESTING & MATERIALS (ASTM).

D522 	Mandrel Bend Test of Attached Organic Coatings (DOD Adopted) 01JAN01

D1171 	Rubber Deterioration - Surface Ozone Cracking Outdoors or Chamber (Triangular Specimens) (DOD Adopted)

(Applications for copies should be addressed to the: American Society for Testing & Materials 100 Bar Harbor Drive, West Conshohocken, PA 19428-2959)

GENERAL MOTORS (GM)

GM 9540P Accelerated Corrosion Test

(Application for copies should be addressed to Global Engineering, 7730 Carondelet Ave., Suite 407, St. Louis, MO  63105)

INTERNATIONAL ORGANIZATION OF STANDARDIZATION (ISO)

668	Series 1 Freight Containers - Classification, Dimensions and Ratings

(Application for copies should be addressed to International Organization of Standardization (ISO), Case Postale 56, Geneva, Switzerland  CH-1211)

SOCIETY OF AUTOMOTIVE ENGINEERS, INC. (SAE)

| AS8090 | Mobility, Towed Aerospace Ground Equipment, General Requirements for (DOD Adopted) |
| --- | --- |
| J163 | Low Tension Wiring and Cable Terminals and Splice Clips (DOD Adopted) |
| J198 | Windshield Wiper Systems- Trucks, Buses, and Multipurpose Vehicles (DOD Adopted) |
| J318 | Automotive Air Brake Line Couplers (Gladhands) |
| J336 | Sound Level for Truck Cab Interior (DOD Adopted) |
| J343 | Test and Test Procedures for SAE 100R Series Hydraulic Hose and Hose Assemblies |
| J366 | Exterior Sound Level for Heavy Trucks and Buses (DOD Adopted) |
| J381 | Windshield Defrosting Systems Test Procedures -Trucks, Buses, and Multipurpose Vehicles (DOD Adopted) |
| J382 | Windshield Defrosting Systems Performance Requirements Trucks, Buses, and Multipurpose Vehicles (DOD Adopted) |
| J516 | Hydraulic Hose Fittings |
| J517 | Hydraulic Hose |
| J534 | Lubrication Fittings (DOD Adopted) |
| J560 | Seven Conductor Electrical Connector for Truck-Trailer Jumper Cable (DOD Adopted) |
| J682 | Rear Wheel Splash and Stone Throw Protection (DOD Adopted) |
| J683 | Tire Chain Clearance-Trucks, Buses (Except Suburban, Intercity, and Transit Buses), and Combinations of Vehicle (DOD Adopted) |
| J697 | Safety Chain of Full Trailers or Converter Dollies (DOD Adopted) |
| J701 | Truck Tractor Semitrailer Interchange Coupling Dimensions (DOD Adopted) |
| J706 | Rating of Winches (DOD Adopted) |
| J848 | Fifth Wheel King Pin, Heavy Duty - Commercial Trailers and Semitrailers (DOD Adopted) |
| J849 | Connection and Accessory Locations for Towing Multiple Trailers (DOD Adopted) |
| J994 | Alarm - Backup - Electric Laboratory Performance Testing, Standard (DOD Adopted) |
| J1100 | Motor Vehicle Dimensions (DOD Adopted) |
| J1292 | Automobile, Truck, Truck-Tractor, Trailers, and Motor Coach Wiring (DOD Adopted) |
| J1436 (R) | Requirements for Engine Cooling System Filling, De-aeration, and Drawdown Tests, Information Report |
| J1587 | Joint SAE/TMC Electronic Data Interchange between Microcomputer Systems in Heavy Duty Vehicle Applications |
| J1708 | Serial Data Communications between Microcomputer Systems in Heavy-Duty Vehicle Applications 13 |
| J1850 | Class B Data Communications Network Interface |
| J1939 | Series: J1939-11 Physical Layer - 250K bits/s, Shielded Twisted Pair |
| J1939-13 | Off-Board Diagnostic Connector |
| J1939-21 | Data Link Layer |
| J1939-31 | Network Layer |
| J1939-71 | Vehicle Application Layer |
| J1939-73 | Application Layer - Diagnostics |
| J1939-81 | Recommended Practice for Serial Control and Communications Vehicle Network - Part 81 - Network Management |
| J1992 | Wheels/Rims - Military Vehicles - Test Procedures and Performance Requirements |
| J2014 | Pneumatic Tires for Military Tactical Wheeled Vehicles |
| J2711 | Recommended Practice for Measuring Fuel Economy and Emissions of Hybrid-Electric and Conventional Heavy-Duty Vehicles |

(Applications for copies should be addressed to the: Society of Automotive Engineers, Inc., 400 Commonwealth Drive, Warrendale, PA 15096)

TIRE AND RIM ASSOCIATION (TRA) INCORPORATED

European Committee for Standardization EN-12999 Crane-Loader cranes

TRA 1-Year Book

(Application for copies should be addressed to the: TRA Inc., 175 Montrose West Avenue, Suite 150, Copley, OH  44321)

## 2.4 Order of precedence.

In the event of a conflict between the text of this specification and the references cited herein, requirements shall be followed by the below listed order of precedence:

1.  Contract Document and associated contract clauses.

2.  JLTV Performance Specification, dated 7 December 2007.

3.  Government Standards, specifications or handbooks.

4.  Non-government standards, specifications or handbooks.

### 2.4.1 Compliance with Laws & Regulations.

The requirements and specifications contained in the above documents shall not be interpreted as a waiver or allowance to supersede any law or regulation unless a specific exemption has been obtained.

| JLTV REQUIREMENTS and VERIFICATION MATRIX |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PD ID | JLTV FoV Requirements | I | T | C | A | Section 4 - Verification | Tier |
| PD874 | # 3 VEHICLE REQUIREMENTS. |  |  |  |  | 4 Requirements Verification |  |
| PD875 | The JLTV is defined as a System of Systems to include the truck chassis, the Companion Trailer, and applicable sub-components listed hereunder. All vehicle variants shall meet the general requirements of section 3 of this specification. Payload Category and Companion Trailer specific requirements are contained in the respective vehicle specific annex for each JLTV sub-configuration. Each requirement within the FoV and Annexes has a corresponding ranking, referred to as a "Tier", ranging from 1 to 3. No Tier 1 or Tier 2 requirement may be traded. Proposed trades to Tier 3 requirements should be informed by the OMS/MP (see ATPD Annex H), and shall be managed through the JLTV trade study process per the Statement of Work (SOW). If a conflict arises between Section 3 of this specification and the vehicle specific annex, the callout in the vehicle specific annex shall take precedence. If not otherwise specified, all requirements are threshold values (T). Objective values, which are desired capabilities, are labeled with an (O). |  |  |  |  | Compliance with the Section 3 requirements shall be verified through one or more of the following methods, as specified in the JLTV Requirements and Verification Matrix and described herein: Inspection, Test, Certification, Analysis. Verification requiring Inspection, Test, and/or Certification shall pertain only to those sub-configurations being procured. Verification requiring Analysis shall pertain to all sub-configurations. Inspection (I). Inspections constitute those activities conducted by the contractor or government inspector for the purpose of evaluating vehicle attributes and characteristics against the JLTV Section 3 requirements. Verification shall be accomplished by visual or physical examination of the end item or its subsystems/components, and may include but are not limited to recording measurements, evaluating physical characteristics and interfaces with respect to form, fit, and function, and reviewing descriptive documentation. Test (T). Testing shall be conducted by the Government, and shall be accomplished through the systematic physical operation of the JLTV, its subsystems or components, under appropriate and specified conditions, with or without instrumentation, and the collection, analysis, and evaluation of resulting data. Certification (C). Certifications are defined as contractor-furnished documents certifying compliance with the specific Section 3 requirement criteria. Certifications shall be signed by the contractor’s certifying official or responsible party, and shall include all supporting performance, inspection, and analytical data, as applicable. As directed by the Government, certifications may be used in lieu of additional verification methods. Analysis (A). Analyses shall be conducted by the contractor and shall consist of technical or mathematical evaluations, mathematical models, simulations, algorithms, charts, diagrams, representative data, or other appropriate means to demonstrate compliance with the Section 3 requirement. |  |
| PD876 | ## 3.1 Physical Requirements |  |  |  |  | 4.1 |  |
| PD877 | ### 3.1.1 Curb Weight (CW). | X | X |  |  | 4.1.1 |  |
| PD878 | Curb weight of the JLTV shall be defined as the empty vehicle weight with a full fuel tank, all fluids, lubricants, coolant, A structure inherent armor , Basic Issue Items (BII), On-vehicle Maintenance (OVM) and weapons mounts and spare tire. | X | X |  |  | The vehicle shall be inspected for compliance with the configuration called out in Section 3, and shall be weighed with verified, calibrated scales and recorded as CW. |  |
| PD879 | ### 3.1.2 Gross Vehicle Weight (GVW). | X | X |  |  | 4.1.2 |  |
| PD880 | GVW is defined as CW plus payload, plus B armor kit. | X | X |  |  | The vehicle shall be inspected for compliance with the configuration called out in Section 3, and shall be weighed with verified, calibrated scales and recorded as GVW. |  |
| PD881 | ### 3.1.3 Gross Combined Vehicle Weight (GCVW). | X | X |  |  | 4.1.3 |  |
| PD882 | GCVW shall be defined as the gross vehicle weight of the JLTV (excluding tongue/pintle weight) plus the GVW of the towed load, when coupled to the truck. For planning purposes and mobility calculations, it shall be assumed that 10% of the trailer GVW is trailer tongue weight that is carried by the JLTV prime mover. All characteristics requiring evaluation at GCVW shall be performed using the JLTV Companion Trailer (CT) at GVW. | X | X |  |  | The vehicle shall be inspected for compliance with the configuration called out in Section 3, and shall be weighed with verified, calibrated scales and recorded as GCVW. |  |
| PD883 | ### 3.1.4 Payload. |  |  |  |  | 4.1.4 |  |
| PD884 | Payload shall be defined as occupants and their personal equipment with individual weapon, sustainment items for mission lengths called out in the vehicle specific annex, mission essential equipment, cargo, and the tongue/pintle weight of the towed load (if any). B armor kit is not considered part of payload. The planning factor per soldier with equipment is 363 pounds each. |  |  |  |  |  |  |
| PD885 | #### 3.1.4.1 Payload for Payload Category Vehicles. |  |  |  |  | 4.1.4.1 |  |
| PD886 | See vehicle specific annex. |  |  |  |  | See vehicle specific annex. |  |
| PD7339 | #### 3.1.4.2 Self Sustainment Payload. |  |  |  | X | 4.1.4.2 |  |
| PD7340 | The JLTV FoV (with accompanying trailer) shall be capable of carrying 3 days (T), 7 days (O) of self sustainment payload. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. |  |
| PD7341 | ##### 3.1.4.2.1 On Vehicle Self-Sustainment Payload. |  |  |  | X | 4.1.4.2.1 |  |
| PD7342 | The JLTV FoV shall be capable of carrying at least one (1) day of self sustainment payload on the vehicle as defined below. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. |  |
| PD7343 | ###### 3.1.4.2.1.1 One Day of Self Sustainment Payload. |  |  |  |  | 4.1.4.2.1.1 |  |
| PD7345 | Table 1. One Day of Self Sustainment Payload. |  |  |  |  |  |  |
| PD7344 |  |  |  |  |  |  |  |
| PD3930 | ### 3.1.5 Essential Combat Configuration (ECC) | X | X |  |  | 4.1.5 |  |
| PD3931 | Essential Combat Configuration shall be defined as curb weight of the vehicle but at ¾ tank of fuel plus 2 crew members with personal gear (363 lbs per person for planning purposes), plus sub-configuration specific communication equipment, one days supply of food, water and ammunition and the primary weapon system, such that the vehicle is capable of immediate employment upon debarkation. "Immediate" for this purpose is defined as the time required to mount and load the weapon, 3 minutes or less excluding the time to mount the Remote Weapons Station. | X | X |  |  | To determine compliance with Essential Combat Configuration, the vehicle shall be payloaded as stated and the primary weapon is mounted and loaded. When the primary weapon is mounted and loaded in 3 minutes or less the requirement will be considered met. | 2 |
| PD6836 | ### 3.1.6 B armor Kit. |  |  |  |  | 4.1.6 |  |
| PD6837 | The B armor kit shall be defined as the Gunners Protection Kit (GPK) and the add on armor above the A structure inherent armor. |  |  |  |  |  |  |
| PD895 | ### 3.1.7 Operating Environment. |  | X |  | X | 4.1.7 |  |
| PD896 | #### 3.1.7.1 Operating Parameters. |  | X |  | X | 4.1.7.1 |  |
| PD897 | The JLTV shall |  |  |  |  |  | 3 |
| PD898 | 1) Start and operate in all altitudes from -500 feet (-152.4 meters) through 12,000 feet (3658 meters) |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD899 | 2) Maintain full mission capability in temperatures from -50°F to 130°F (-46° C to 54° C). |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3940 | 3) Start and operate on JP-8 and DF-2 Fuels independently. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD900 | ##### 3.1.7.1.1 Start |  | X |  |  | 4.1.7.1.1 |  |
| PD901 | The JLTV shall Start in: |  |  |  |  |  | 3 |
| PD902 | 1) -25°F to 130°F (-32° C to 54° C) within one minute from the initiation of cranking with no external aids, kits, or prior warming of the batteries. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD903 | 2) -50°F to -26°F (-46° C to -32.5° C) within two minutes or less from the initiation of cranking assisted by external aids or arctic heater kits which will not be a permanent part of the non-artic fleet. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3942 | a) The engine must be able to be started within thirty (30) minutes from the initiation of the engine arctic kit-aided start sequence. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6544 | ###### 3.1.7.1.1.1 Automatic Starting Aid. |  |  |  | X | 4.1.7.1.1.1 |  |
| PD3529 | If ether is required to assist engine start in cold weather, the vehicle shall be equipped with a fully-automatic starting fluid system that will inject precisely controlled amount of vaporized starting fluid into the engine's air intake system to prevent engine damage. The automatic starting aid system will only operate when enabled by the driver. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD911 | #### 3.1.7.2 Storage Temperatures. |  |  |  | X | 4.1.7.2 |  |
| PD912 | The JLTV and it's Companion Trailer shall be capable of being placed outdoor in long term storage, up to six months, at temperatures ranging between -60° F and 160° F (-51° C to 71° C) , in humid storage conditions IAW AR 70-38 table 2-1 and in salt-fog conditions per MCO 4790.18B 16 Jul 04 / TM 4795-34-2 / TM4795-12-1 without degradation. |  |  |  | X | Analysis shall be conducted to demonstrate compliance with storage conditions. | 3 |
| PD939 | #### 3.1.7.3 Crew Visibility. |  |  |  |  | 4.1.7.3 |  |
| PD7276 | ##### 3.1.7.3.1 |  |  |  |  | 4.1.7.3.1 |  |
| PD940 | The vehicle crew compartment shall allow the driver and passengers to collectively maintain 360 degree visibility around the vehicle, with and without B armor kit. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7277 | ##### 3.1.7.3.2 |  | X |  |  | 4.1.7.3.2 |  |
| PD7278 | The JLTV Companion Trailer (JLTV-T) shall be visible from the driver's position when tracking directly behind the prime mover. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. |  |
| PD941 | ## 3.2 PERFORMANCE CHARACTERISTICS |  |  |  |  | 4.2 |  |
| PD942 | Unless otherwise specified, all performance requirements shall be met with the vehicle at Gross Vehicle Weight (GVW). If Gross Combined Vehicle Weight (GCVW) is specified, the CT (Companion Trailer with uniformly distributed payload whose CG is 24 inches above the cargo bed) shall be the trailer to be used for mobility calculations and/or modeling. All performance requirements shall be met while operating on JP-8 fuel per MIL-DTL-83133 and at full hotel load. All Force Protection and Mobility requirements shall be met at one ride height. |  |  |  |  |  | 2 |
| PD943 | ### 3.2.1 MOBILITY. |  |  |  |  | 4.2.1 |  |
| PD945 | #### 3.2.1.1 Mission Profile. |  |  |  |  | 4.2.1.1 |  |
| PD946 | JLTV Tactical Mobility is defined as 60 percent improved roads (Primary and Secondary) and 40 percent-unimproved roads (trails) and cross-country. Cross-country includes beaches, forests, grasslands, tropical jungles, mountains, and deserts throughout all seasonal conditions. |  |  |  |  |  |  |
| PD7207 | Table 2. JLTV Tactical Mobility |  |  |  |  |  |  |
| PD7346 |  |  |  |  |  |  |  |
| PD977 | * Root Mean Squared (RMS) is a measure of surface and terrain roughness used to evaluate trafficability |  |  |  |  |  |  |
| PD978 | #### 3.2.1.2 Acceleration Dash Speed. |  | X |  |  | 4.2.1.2 |  |
| PD979 | The JLTV at GVW shall be capable of accelerating on dry, level hard terrain: |  |  |  |  |  | 3 |
| PD980 | 1) From 0 to 30 mph (48.3 kph) within 9.4 seconds (T)/7 seconds (O) |  | X |  |  | Test shall be conducted IAW TOP 2-2-602. | 3 |
| PD981 | 2) From 0 to 50 mph (80.5 kph) within 26.1 seconds. |  | X |  |  | Test shall be conducted IAW TOP 2-2-602. | 3 |
| PD982 | #### 3.2.1.3 Speed |  | X |  |  | 4.2.1.3 |  |
| PD983 | The JLTV variants shall meet the following speed requirements: |  |  |  |  |  |  |
| PD984 | ##### 3.2.1.3.1 Forward Speed. |  | X |  |  | 4.2.1.3.1 |  |
| PD985 | The JLTV at GVW shall be capable of maintaining a minimum speed of 70 mph (112.7 kph) (T), 90 mph (144.8 kph) (O) in the forward direction on a dry, level, hard surface road. |  | X |  |  | Test Shall be conducted IAW TOP 2-2-602. | 2 |
| PD986 | ##### 3.2.1.3.2 Reverse speed. |  | X |  |  | 4.2.1.3.2 |  |
| PD987 | The JLTV at GVW shall be capable of operating in reverse at a speed of 10 mph (16.1 kph) (T) and 20 mph (32.2 kph) (O) on a dry, level, hard surface road. |  | X |  |  | Test Shall be conducted IAW TOP 2-2-602. | 3 |
| PD988 | ##### 3.2.1.3.3 Speed on grade. |  |  |  |  | 4.2.1.3.3 |  |
| PD989 | See vehicle specific annex. |  |  |  |  | See vehicle specific annex. | 2 |
| PD990 | #### 3.2.1.4 Lateral Stability. |  | X |  |  | 4.2.1.4 |  |
| PD991 | For the JLTV the lateral roll stability as determined from a steady-state circular turn at the vehicle’s tightest turning radius, as defined in the vehicle specific annex, on a paved surface shall meet or exceed a lateral acceleration of 0.5 g's (T), 0.6 g’s (O) without wheel-lift off. The JLTV shall meet the stability requirements at Curb weight, CW with B armor kit, at GVW, at GVW minus B armor kit, at GCVW and at GCVW minus B armor kit. |  | X |  |  | Test shall be conducted IAW AVTP 03-160W | 3 |
| PD992 | #### 3.2.1.5 Approach & Departure Angles. | X | X |  |  | 4.2.1.5 |  |
| PD993 | ##### 3.2.1.5.1 | X | X |  |  | 4.2.1.5.1 |  |
| PD994 | The JLTV angle of approach shall not be less than 60 degrees without winch. Protrusion of the tow eyes into the angle of approach plane is permitted. | X | X |  |  | The JLTV shall be inspected and tested to verify compliance with the Section 3 requirement. Measurement shall be conducted IAW SAE J1100. | 3 |
| PD995 | ##### 3.2.1.5.2 | X | X |  |  | 4.2.1.5.2 |  |
| PD996 | The JLTV angle of approach shall not be less than 45 degrees with winch. Protrusion of the tow eyes into the angle of approach plane is permitted. | X | X |  |  | The JLTV shall be inspected and tested to verify compliance with the Section 3 requirement. Measurement shall be conducted IAW SAE J1100. | 3 |
| PD997 | ##### 3.2.1.5.3 | X | X |  |  | 4.2.1.5.3 |  |
| PD998 | The JLTV, at GVW and GCVW, angle of departure shall not be less then 45 degrees in the area of the pintle hook. Protrusion of the tow eyes into the angle of approach plane is permitted. | X | X |  |  | The JLTV shall be inspected and tested to verify compliance with the Section 3 requirement. Measurement shall be conducted IAW SAE J1100. | 3 |
| PD1001 | #### 3.2.1.6 Brakes. | X | X | X |  | 4.2.1.6 |  |
| PD1002 | ##### 3.2.1.6.1 Service Brakes |  | X | X |  | 4.2.1.6.1 |  |
| PD1003 | Service brakes shall comply with the requirements of FMVSS 121 or FMVSS 105. |  |  | X |  | Certification shall be issued by the contractor stating the brakes comply with FMVSS 121 or FMVSS 105. | 3 |
| PD1006 | ###### 3.2.1.6.1.1 |  | X |  |  | 4.2.1.6.1.1 |  |
| PD1007 | The service brakes shall hold the vehicle at GVW on a dry hard surface, 60% grade (30.96 degrees) pointing either uphill or downhill. |  | X |  |  | Test shall be conducted IAW TOP 2-2-608. | 3 |
| PD3919 | ###### 3.2.1.6.1.2 |  | X |  |  | 4.2.1.6.1.2 |  |
| PD3921 | The service brakes shall hold the vehicle at GCVW on a dry hard surface, 40% grade (21.8 degrees) pointing either uphill or downhill. |  | X |  |  | Test shall be conducted IAW TOP 2-2-608. | 3 |
| PD6857 | ###### 3.2.1.6.1.3 Stopping Distance |  | X |  |  | 4.2.1.6.1.3 |  |
| PD6858 | The service brakes shall stop the JLTV at GVW from a speed of 20 mph (T), 30 mph (O) in 25 ft or less, when tested in accordance with FMVSS 105 and 121. Brake pedal force shall not exceed that which can be applied by the driver as defined in MIL-STD 1472. |  | X |  |  | Test shall be conducted IAW TOP 2-2-608. | 3 |
| PD6915 | ###### 3.2.1.6.1.4 |  | X |  |  | 4.2.1.6.1.4 |  |
| PD6916 | The JLTV at GCVW shall be capable of maintaining speeds downhill equal to the uphill capability of the vehicle up to a 15% slope. |  | X |  |  | Test shall be conducted IAW TOPs 2-2-604 and 2-2-608 | 3 |
| PD3377 | 3.2.1.6.1.4.1 |  | X |  |  | 4.2.1.6.1.4.1 |  |
| PD3378 | If engine assisted braking is provided then this system shall have the ability to be turned off when not needed. |  | X |  |  | The engine assisted brake if used will be tested for the ability be turned off if not needed | 3 |
| PD1008 | ##### 3.2.1.6.2 Parking Brakes. | X | X | X |  | 4.2.1.6.2 |  |
| PD3922 | ###### 3.2.1.6.2.1 |  | X |  |  | 4.2.1.6.2.1 |  |
| PD3924 | At GVW, the JLTV shall be capable of holding in either direction on a 40% (21.8 degrees) longitudinal slope (T) using the JLTV parking brake with the engine off and the transmission in neutral. Grade surface shall be dry, hard and free from loose material. |  | X |  |  | Test shall be conducted IAW TOP 2-2-608. | 3 |
| PD3923 | ###### 3.2.1.6.2.2 |  | X |  |  | 4.2.1.6.2.2 |  |
| PD1009 | At GCVW, the JLTV shall be capable of holding in either direction on a 40% (21.8 degrees) longitudinal slope (T) using the JLTV parking brake with the engine off and the transmission in neutral. Grade surface shall be dry, hard and free from loose material. |  | X |  |  | Test shall be conducted IAW TOP 2-2-608. | 3 |
| PD1010 | ###### 3.2.1.6.2.3 |  |  | X |  | 4.2.1.6.2.3 |  |
| PD1011 | Parking brake shall conform to FMCSR 393.41 |  |  | X |  | Certification shall be issued stating the brakes comply with FMCSR 393.41. | 3 |
| PD1012 | ###### 3.2.1.6.2.4 | X | X |  |  | 4.2.1.6.2.4 |  |
| PD1013 | An indicator shall be provided to alert the crew when the parking brake is engaged. | X | X |  |  | The JLTV shall be inspected and tested to verify compliance with the Section 3 requirement. | 3 |
| PD6818 | ###### 3.2.1.6.2.5 |  | X |  |  | 4.2.1.6.2.5 |  |
| PD6819 | The vehicle parking brake shall have a manual release in the event of an electrical, hydraulic, or pneumatic system failure. |  | X |  |  | The parking brake shall be tested for manual release. | 3 |
| PD1014 | ##### 3.2.1.6.3 Brake Configuration. | X | X | X |  | 4.2.1.6.3 |  |
| PD1015 | Brakes shall conform to Federal Motor Carriers Safety Regulations (FMCSR) 393.40 through 393.42 (b), 393.43, and 393.45 through 393.52, and 393.55 in the tactical environment. |  |  | X |  | Certification shall be issued stating the brakes comply with Section 3 requirements. | 3 |
| PD1018 | ###### 3.2.1.6.3.1 | X | X |  |  | 4.2.1.6.3.1 |  |
| PD1019 | Brake system shall be designed to minimize exposure to "off-road hazards" and maximize ground clearance. | X | X |  |  | The JLTV shall be inspected and tested to verify compliance with the Section 3 requirement. | 3 |
| PD1022 | ###### 3.2.1.6.3.2 Brakes for Pintle/Lunette Towed Trailers. |  | X | X |  | 4.2.1.6.3.2 |  |
| PD1023 | Air brake glad hand couplers with cover assemblies and cover securing chains shall be provided at the front (if truck complies with FMVSS 121) and rear of the truck per SAE J849 to interface with pintle/lunette towed trailers. |  | X | X |  | The glad hands shall be certified and checked for presence, location, function and marking and shall not leak. Glad hands, when hooked up to the towed vehicle’s brake air supply shall function the towed vehicle’s brake system. | 3 |
| PD1026 | ###### 3.2.1.6.3.3 Trailer Brake Control System. |  | X | X |  | 4.2.1.6.3.3 |  |
| PD1027 | A complete trailer brake control system shall be furnished that includes the following: |  | X |  |  | The vehicle shall be tested to verify that the stated conditions are met. | 3 |
| PD1028 | a. Clear identification (color code and tags) of both emergency and service line connectors. |  | X |  |  | The vehicle shall be tested to verify that the stated conditions are met. | 3 |
| PD1029 | b. Coincident control of trailer brakes with truck foot brake control. |  | X |  |  | The vehicle shall be tested to verify that the stated conditions are met. | 3 |
| PD1030 | c. Independent hand control for trailer brakes. |  | X |  |  | The vehicle shall be tested to verify that the stated conditions are met. | 3 |
| PD1032 | d. Trailer stoplight operative with both the foot brake and the hand control for trailer brakes. |  | X |  |  | The vehicle shall be tested to verify that the stated conditions are met. | 3 |
| PD1033 | e. Two connecting coil-type air hoses (color-coded) equipped with coiled spring hose guards and "glad hand" quick connector on trailer end of hoses. |  | X |  |  | The vehicle shall be tested to verify that the stated conditions are met. | 3 |
| PD1034 | ##### 3.2.1.6.4 Antilock Braking System (ABS). |  |  | X |  | 4.2.1.6.4 |  |
| PD1038 | ###### 3.2.1.6.4.1 |  |  | X |  | 4.2.1.6.4.1 |  |
| PD1039 | The ABS system shall have built-in test and report thru the vehicle diagnostic system. |  |  | X |  | The contractor shall certify that ABS is monitored by the diagnostic system. | 2 |
| PD1042 | ###### 3.2.1.6.4.2 ABS Electronic Control Unit. |  |  | X |  | 4.2.1.6.4.2 |  |
| PD1043 | The Anti-lock Braking System (ABS) electronic control unit (ECU) shall include wiring provisions to transmit the trailer ABS malfunction signal to the prime mover cab. PLC (Power Line Carrier - SAEJ 2497) communication technology shall be used to comply with the trailer ABS malfunction requirement of FMVSS 121. |  |  | X |  | The contractor shall certify that the wiring provisions are capable of transmitting the ABS malfunction to the cab and meets the ABS malfunction requirement of FMVSS 121. | 2 |
| PD1044 | ##### 3.2.1.6.5 Brake Wear Indicator. |  | X |  |  | 4.2.1.6.5 |  |
| PD1045 | The JLTV shall be equipped with a means to inform the operator, without removal of tire/wheel and without any special tools, when the brake pads are approaching the end of their service life. |  | X |  |  | The brake wear indicator will be verified during RAM testing. | 3 |
| PD1046 | #### 3.2.1.7 Maneuverability over Terrain obstacles. |  | X |  | X | 4.2.1.7 |  |
| PD1047 | ##### 3.2.1.7.1 NATO Reference Mobility Model (NRMM) |  | X |  | X | 4.2.1.7.1 |  |
| PD1048 | The vehicle (at GVW) shall achieve the NATO Reference Mobility Model (NRMM) requirements as called out in the vehicle specific annex. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. |  |
| PD1049 | ##### 3.2.1.7.2 Vehicle Cone Index |  | X |  | X | 4.2.1.7.2 |  |
| PD1050 | The JLTV single pass vehicle cone index (VCI [VCI1]) for mud/sand/snow shall have a value no greater than 25.0 (T) 20.0 (O) at GVW, with the tire pressure set at an appropriate reduced level, and using a corresponding increased section width. |  | X |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. The test shall be conducted IAW TOP 2-2-619. | 2 |
| PD1051 | #### 3.2.1.8 Ride Quality. |  | X |  | X | 4.2.1.8 |  |
| PD1052 | The JLTV shall meet the ride quality requirements at CW, GVW, GVW minus B armor kit, GCVW, and GCVW minus B armor kit. |  |  |  |  |  | 3 |
| PD1053 | ##### 3.2.1.8.1 |  | X |  | X | 4.2.1.8.1 |  |
| PD1060 | The JLTV shall be robust enough to accommodate future payload growth of at least 25%(O). |  | X |  | X | The contractor, if they can meet the overload requirement, shall certify to that effect. If the contractor certifies to meeting the overload requirement, the vehicle will be tested IAW TOP 1-1-014 | 3 |
| PD1061 | ##### 3.2.1.8.2 Ride Limiting Speeds. |  | X |  | X | 4.2.1.8.2 |  |
| PD1062 | The JLTV shall attain no more than 6 watts average vertical absorbed power, as measured at the base of all occupant seats of the JLTV, while negotiating the following Root Mean Square (RMS) ride courses at speeds listed below, with the tires at cross-country tire pressure. |  | X |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. The test shall be conducted IAW TOP 1-1-014. | 3 |
| PD1063 | Table 3. 6-Watt Speeds |  |  |  |  |  | 3 |
| PD7347 |  |  |  |  |  |  |  |
| PD7348 |  |  |  |  |  |  |  |
| PD7349 |  |  |  |  |  |  |  |
| PD7350 |  |  |  |  |  |  |  |
| PD7351 |  |  |  |  |  |  |  |
| PD1081 | ##### 3.2.1.8.3 Vertical Acceleration. |  | X |  | X | 4.2.1.8.3 |  |
| PD1082 | The JLTV shall sustain no more than 2.5-G peak vertical acceleration, as measured at each occupant location while negotiating a non-deformable, half-round obstacle at the rated speed as listed below with the tires at cross-country tire pressure. |  | X |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. The test shall be conducted IAW TOP 2-2-808. | 3 |
| PD7208 | Table 4. Vertical Acceleration |  |  |  |  |  |  |
| PD7352 |  |  |  |  |  |  |  |
| PD7353 |  |  |  |  |  |  |  |
| PD7354 |  |  |  |  |  |  |  |
| PD7355 |  |  |  |  |  |  |  |
| PD7356 |  |  |  |  |  |  |  |
| PD1096 | ##### 3.2.1.8.4 Vehicular Vibration. |  | X |  | X | 4.2.1.8.4 |  |
| PD1097 | The JLTV shall be designed to control the transmission of whole-body vibration to levels that permit safe and effective operation and maintenance per MIL-STD-1472. Evaluation of military vehicle vibration and its possible effects on health, comfort, perception and motion sickness shall conform to ISO 2631-1 and ISO 2631-5. |  | X |  | X | Analysis and testing will be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD1108 | #### 3.2.1.9 Grade & Slope Operations. |  | X |  |  | 4.2.1.9 |  |
| PD3926 | The JLTV shall be capable of meeting the below Grade & Slope operations in the following states: |  | X |  |  |  | 3 |
| PD3927 | 1) With the fuel tank at 100% full. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3928 | 2) With the fuel tank at 10% of the useable left. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1109 | ##### 3.2.1.9.1 Grade Operations. |  | X |  |  | 4.2.1.9.1 |  |
| PD1110 | The JLTV shall be capable of: |  | X |  |  |  |  |
| PD1111 | 1) Ascending/descending, starting, and stopping on dry, hard-surfaced longitudinal slopes |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1112 | 2) The vehicle engine shall be capable of being turned off and restarted while on the slope. The engine off times shall be of such long duration as to assure that there shall be no loss of fluids or other malfunction while parked on the slope |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1113 | ###### 3.2.1.9.1.1 60% Grade (30.96 degrees). |  | X |  |  | 4.2.1.9.1.1 |  |
| PD1114 | The JLTV at GVW shall be capable of slope operations up to and including 60% (30.96 degrees) at GVW (T), GCVW (O). |  | X |  |  | Test shall be conducted IAW 2-2-610. | 3 |
| PD1115 | ###### 3.2.1.9.1.2 40% Grade (21.8 degrees). |  | X |  |  | 4.2.1.9.1.2 |  |
| PD1116 | The JLTV at GCVW shall be capable of slope operations up to and including 40% (21.8 degrees) (T), 60% (30.96 degrees) (O). |  | X |  |  | Test shall be conducted IAW 2-2-610. | 3 |
| PD1118 | ##### 3.2.1.9.2 Side Slopes Operations. |  | X |  |  | 4.2.1.9.2 |  |
| PD1119 | The JLTV shall be capable of laterally traversing, in forward (at GVW and GCVW) and reverse (at GVW), side slopes up to and including 40% with no degradation in driver control (at GVW). Side slope operation shall be performed with either side of the vehicle facing up slope and without engine malfunction or loss of vehicle fluids. |  | X |  |  | Test shall be conducted IAW 2-2-610. | 3 |
| PD1122 | #### 3.2.1.10 Tires. | X | X | X | X | 4.2.1.10 |  |
| PD1123 | ##### 3.2.1.10.1 Rims & Tires. | X | X | X | X | 4.2.1.10.1 |  |
| PD1124 | Rims and tires shall meet the requirements of SAE J2014 and conform to Federal Motor Vehicles Safety Standards (FMVSS) 571.119 and 571.120. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD1125 | ###### 3.2.1.10.1.1 | X |  | X |  | 4.2.1.10.1.1 |  |
| PD1126 | Vehicle and trailer tires shall be a tubeless radial design with hub piloted wheels. Beadlock for CTIS system shall be provided if prime mover is equipped with CTIS. | X |  | X |  | Certification shall be provided that the tires and rims are in compliance with the Section 3 requirement. | 3 |
| PD7055 | 3.2.1.10.1.1.1 | X |  |  |  | 4.2.1.10.1.1.1 |  |
| PD7056 | The JLTV shall not contain a split rim design. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD1127 | ###### 3.2.1.10.1.2 |  |  | X |  | 4.2.1.10.1.2 |  |
| PD1128 | All tire and rim ratings shall conform to the Tire and Rim Association (TRA) 1 or the European Tire and Rim Technical Organization (ETRTO) Standards Manual for the GCVW and maximum speed of the vehicle. |  |  | X |  | Certification shall be provided to verify that tires and rims are TRA1 or ETRTO compliant. | 3 |
| PD1129 | ###### 3.2.1.10.1.3 | X |  |  |  | 4.2.1.10.1.3 |  |
| PD1130 | The JLTV shall use a single tire tread design within a payload category (T), across the FoV (O). | X |  |  |  | Tires shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD1131 | ###### 3.2.1.10.1.4 | X |  |  |  | 4.2.1.10.1.4 |  |
| PD1132 | All wheel/tire assemblies shall be interchangeable within each payload category and its trailer (T), the FoV and all trailer types (O). | X |  |  |  | Inspection shall be conducted to verify interchangeability of wheels between payload category and trailers. | 3 |
| PD1133 | ###### 3.2.1.10.1.5 |  | X |  | X | 4.2.1.10.1.5 |  |
| PD1134 | Tires shall have a minimum tread life of 18,000 miles over JLTV OMS/MP terrain, without re-treading. |  | X |  | X | The JLTV shall be tested and analyzed to verify compliance with the Section 3 requirement. | 3 |
| PD1135 | ##### 3.2.1.10.2 Spare Tire/Wheel Assembly, Carrier and Hoist. | X | X |  |  | 4.2.1.10.2 |  |
| PD1136 | A spare wheel/tire assembly (identical to vehicle set) shall be provided for each vehicle. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD1137 | ###### 3.2.1.10.2.1 | X | X |  |  | 4.2.1.10.2.1 |  |
| PD1138 | A device, capable of operating independent of vehicle power, shall be provided on each vehicle to facilitate spare tire loading and unloading from stowed position by two (T), one (O) occupant. | X | X |  |  | Inspection shall be conducted to demonstrate compliance with the Section 3 requirement. Testing shall be conducted to verify compliance. | 3 |
| PD1139 | ##### 3.2.1.10.3 Tire Changing Equipment. | X | X |  |  | 4.2.1.10.3 |  |
| PD1140 | A two person JLTV crew shall be capable of completing a field tire change on the prime mover and companion trailer, using only BII, within 30 minutes per tire (T), 15 minutes per tire (O), while the vehicle is on a flat, hard surface. | X | X |  |  | Inspection shall be conducted to demonstrate compliance with the Section 3 requirement. Testing shall be conducted to verify compliance. | 3 |
| PD7268 | ###### 3.2.1.10.3.1 |  | X |  |  | 4.2.1.10.3.1 |  |
| PD7269 | A single crew member shall be capable of completing a field tire change on the prime mover and companion trailer, using only BII, within 30 minutes per tire (O). |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1141 | ##### 3.2.1.10.4 Run-Flat Capability. | X | X | X |  | 4.2.1.10.4 |  |
| PD6899 | ###### 3.2.1.10.4.1 |  | X |  |  | 4.2.1.10.4.1 |  |
| PD1142 | The run-flat capability shall permit driving over the OMS/MP terrain after loss of air pressure in any two tires (T), all tires (O) for the vehicle, or one tire (T), all tires (O) for the trailer. Reduction in speed is allowable while utilizing the run-flat device. |  | X |  |  | Testing shall be conducted IAW TOP 2-2-704 to verify compliance with the Section 3 requirement. | 2 |
| PD7297 | 3.2.1.10.4.1.1 |  | X | X |  | 4.2.1.10.4.1.1 |  |
| PD1144 | The run-flat capability shall be performed without speed reduction (O). |  | X | X |  | If the contractor certifies that the objective requirement can be met, then the JLTV shall be tested to verify compliance. | 3 |
| PD7296 | 3.2.1.10.4.1.2 |  | X |  |  | 4.2.1.10.4.1.2 |  |
| PD1143 | The run-flat capability shall permit the JLTV to travel a distance of at least 30 miles (T), 60 miles (O). |  | X |  |  | Testing shall be conducted IAW TOP 2-2-704 to verify compliance with the Section 3 requirement. | 3 |
| PD6900 | ###### 3.2.1.10.4.2 | X | X |  |  | 4.2.1.10.4.2 |  |
| PD6901 | The run-flat device shall not damage the vehicle's tires, wheels or suspension components when run at highway, off-road or cross-country inflation levels. | X | X |  |  | Inspection and Testing shall be conducted to verify that any RAM failures were not caused by a run-flat device. | 3 |
| PD1145 | ##### 3.2.1.10.5 Central Tire Inflation System (CTIS). | X | X |  | X | 4.2.1.10.5 |  |
| PD6896 | If CTIS is implemented, it shall meet the following requirements: |  |  |  |  |  |  |
| PD6897 | ###### 3.2.1.10.5.1 |  | X |  |  | 4.2.1.10.5.1 |  |
| PD1151 | The CTIS system shall allow the driver to adjust all JLTV (T) and Companion Trailer (O) tires to any one of four preset tire pressures (highway, cross country, mud/snow/sand, and emergency). |  | X |  |  | Testing shall be conducted IAW TOP 2-2-704 to verify compliance with the Section 3 requirement. | 3 |
| PD3979 | ###### 3.2.1.10.5.2 CTIS Isolation |  | X |  |  | 4.2.1.10.5.2 |  |
| PD1155 | The system shall provide for the automatic isolation of any or all tires from the CTIS in the event of CTIS or tire failure for any reason. |  | X |  |  | Testing shall be conducted IAW TOP 2-2-704 to verify compliance with the Section 3 requirement. | 3 |
| PD3981 | ###### 3.2.1.10.5.3 Manual Tire Inflation/Deflation | X |  |  |  | 4.2.1.10.5.3 |  |
| PD1157 | Valves for manual inflation shall be readily accessible. | X |  |  |  | Valves shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD1158 | ###### 3.2.1.10.5.4 Air-Priority System. |  | X |  |  | 4.2.1.10.5.4 |  |
| PD1159 | The CTIS shall incorporate sufficient safeguards to assure that air pressure necessary to continue safe operation of the JLTV System shall be available at all times during activation of CTIS or in the event of a CTIS failure. Use of brakes is the minimum requirement for safe operation. |  | X |  |  | Testing shall be conducted IAW TOP 2-2-704 to verify compliance with the Section 3 requirement. | 3 |
| PD3977 | ###### 3.2.1.10.5.5 Tire pressure control system |  |  |  |  | 4.2.1.10.5.5 |  |
| PD1164 | 3.2.1.10.5.5.1 Maintenance of Tire Pressure. |  |  |  |  | 4.2.1.10.5.5.1 |  |
| PD1165 | 3.2.1.10.5.5.1.1 | X |  |  |  | 4.2.1.10.5.5.1.1 |  |
| PD1166 | With the CTIS in operation, tire pressure shall be checked and adjusted at intervals necessary to assure that no more than 3 psi variation exists between the selected pressure and actual pressure except during the inflation/deflation operation caused by the selection of a new tire pressure or a catastrophic tire failure. | X |  |  |  | CTIS system shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD1167 | 3.2.1.10.5.5.1.2 | X |  |  |  | 4.2.1.10.5.5.1.2 |  |
| PD1168 | With the CTIS not in operation and the vehicle engine not running after 24 hours, the tire pressure shall not drop below 97% of the pressure setting which existed before the vehicle was stopped. No action shall be required of crew personnel beyond normal shutdown to meet this requirement. | X |  |  |  | CTIS system shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD1160 | 3.2.1.10.5.5.2 Speed-at-Pressure Control Warning. |  |  |  | X | 4.2.1.10.5.5.2 |  |
| PD1161 | The CTIS shall include sensing of the vehicle speed and comparing indicated speed to the maximum allowable speed for each tire pressure setting (highway, cross country, mud/snow/sand, emergency). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1162 | 3.2.1.10.5.5.2.1 |  | X |  |  | 4.2.1.10.5.5.2.1 |  |
| PD1163 | An indicator shall activate to warn the driver of excessive speed-at-pressure conditions until the CTIS has automatically adjusted each tire to the appropriate pressure. |  | X |  |  | Testing shall be conducted IAW TOP 2-2-704 to verify compliance with the Section 3 requirement. | 3 |
| PD1169 | ###### 3.2.1.10.5.6 Time to Inflation/Deflation. |  | X |  |  | 4.2.1.10.5.6 |  |
| PD1170 | 3.2.1.10.5.6.1 Deflation |  | X |  |  | 4.2.1.10.5.6.1 |  |
| PD1171 | The CTIS shall be capable of deflating all JLTV prime mover tires at the same (T); deflating all JLTV prime mover and trailer tires at the same time (O) within the time constraints listed below, (minutes: seconds): |  | X |  |  | Testing shall be conducted IAW TOP 2-2-704 to verify compliance with the Section 3 requirement. | 3 |
| PD1172 | Table 5. DEFLATION Times |  |  |  |  |  | 3 |
| PD7357 |  |  |  |  |  |  |  |
| PD7358 |  |  |  |  |  |  |  |
| PD7359 |  |  |  |  |  |  |  |
| PD7360 |  |  |  |  |  |  |  |
| PD1190 | 3.2.1.10.5.6.2 Inflation |  | X |  |  | 4.2.1.10.5.6.2 |  |
| PD1191 | The CTIS shall be capable of inflating all JLTV prime mover tires at the same (T); inflating all JLTV prime mover and trailer tires at the same time (O) within the time constraints listed below, (minutes: seconds): |  | X |  |  | Testing shall be conducted IAW TOP 2-2-704 to verify compliance with the Section 3 requirement. | 3 |
| PD1192 | Table 6. INFLATION Times |  |  |  |  |  | 3 |
| PD7361 |  |  |  |  |  |  |  |
| PD7362 |  |  |  |  |  |  |  |
| PD7363 |  |  |  |  |  |  |  |
| PD7364 |  |  |  |  |  |  |  |
| PD6898 | ###### 3.2.1.10.5.7 Operator Control Location | X |  |  |  | 4.2.1.10.5.7 |  |
| PD1153 | The operator control of the CTIS system shall be located so that the system may be activated while the driver continues to operate the JLTV. | X |  |  |  | CTIS system shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD1214 | #### 3.2.1.11 Turning Radius, Curb to Curb: |  | X |  |  | 4.2.1.11 |  |
| PD1215 | See vehicle specific annex. |  |  |  |  | See vehicle specific annex. | 3 |
| PD1219 | ##### 3.2.1.11.1 Steerable/Lockable Rear Axle. |  | X |  |  | 4.2.1.11.1 |  |
| PD1220 | If a steerable rear axle is used on the vehicles, transitions to and from the neutral steer position shall not adversely affect the handling of the vehicle. In the event of a rear steer system failure; the rear axles shall assume a neutral, locked position. |  | X |  |  | If a steerable rear axle is used the vehicle shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1216 | #### 3.2.1.12 Lane Changing. |  | X |  |  | 4.2.1.12 |  |
| PD3918 | ##### 3.2.1.12.1 |  | X |  |  | 4.2.1.12.1 |  |
| PD1217 | The vehicle (at GVW) shall be capable of making a NATO lane change in accordance with AVTP 03-160W at speeds up to 45 mph (72 kph) (T), 65 mph (105 kph) (O). |  | X |  |  | Testing shall be conducted IAW AVTP 03-160W to verify compliance with the Section 3 requirement. | 3 |
| PD3916 | ##### 3.2.1.12.2 |  | X |  |  | 4.2.1.12.2 |  |
| PD3917 | The vehicle (at GCVW) shall be capable of making a NATO lane change in accordance with AVTP 03-160W at speeds up to 40 mph (64 kph) (T); 55 mph (89 kph) (O). |  | X |  |  | Testing shall be conducted IAW AVTP 03-160W to verify compliance with the Section 3 requirement. | 3 |
| PD1262 | #### 3.2.1.13 Operational Range. |  | X |  |  | 4.2.1.13 |  |
| PD1263 | ##### 3.2.1.13.1 Range. |  | X |  |  | 4.2.1.13.1 |  |
| PD1264 | The JLTV shall be capable of traveling the distances shown in the table below at sustained speeds on a single tank of fuel. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7366 | Table 7. JLTV Range Requirements |  |  |  |  |  |  |
| PD7365 |  |  |  |  |  |  |  |
| PD1269 | #### 3.2.1.14 Standard Obstacles. |  | X |  |  | 4.2.1.14 |  |
| PD1270 | ##### 3.2.1.14.1 Vertical Step. |  | X |  |  | 4.2.1.14.1 |  |
| PD1271 | The JLTV at GVW shall be capable of stepping up and down a vertical step of 18 inches (T) or 24 inches (O) in forward and reverse on a straight on approach without preparation or modification of the vehicle. No portion of the vehicle other than the tires shall contact the ground or the step. |  | X |  |  | Test shall be conducted IAW TOP 2-2-611 to verify compliance with the Section 3 requirement. | 3 |
| PD1272 | #### 3.2.1.15 Fording. | X | X |  | X | 4.2.1.15 |  |
| PD1273 | The JLTV, at GVW, shall ford a 30” (T) and 60” with kit (T) or 60” without kit (O) salt water obstacle without preparation, or other fording devices, in forward and reverse while maintaining contact with the ground. |  | X |  | X | Test shall be conducted IAW TOP 2-2-612 to verify compliance with the Section 3 requirement.. If attempting to meet the objective, an analysis shall be required. | 3 |
| PD1274 | ##### 3.2.1.15.1 |  | X |  |  | 4.2.1.15.1 |  |
| PD1275 | While fording, the engine shall be capable of being restarted when stopped for 10 minutes. |  | X |  |  | Test shall be conducted IAW TOP 2-2-612 to verify compliance with the Section 3 requirement.. | 3 |
| PD7070 | ##### 3.2.1.15.2 |  | X |  |  | 4.2.1.15.2 |  |
| PD7071 | During fording operations water contamination of engine, brake fluid, transmission, transfer transmission, power steering pump, bearing lubricants, fuel tank(s) and all differentials shall not exceed 2.0% by volume. All bearing seals shall restrict the leaking of lubricants from the bearings. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1276 | ##### 3.2.1.15.3 | X |  |  |  | 4.2.1.15.3 |  |
| PD1277 | Vented components shall be vented above the 30 inch fording line without fording kit and above the 60 inch fording line with kit. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD1278 | #### 3.2.1.16 Towing. | X | X | X |  | 4.2.1.16 |  |
| PD1293 | ##### 3.2.1.16.1 Towed Load Braking and Lighting. | X | X |  |  | 4.2.1.16.1 |  |
| PD1294 | The JLTV shall have the capability to actuate the towed vehicle's brakes and lights. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1295 | ###### 3.2.1.16.1.1 | X |  |  |  | 4.2.1.16.1.1 |  |
| PD1296 | The vehicle shall be equipped with all cables, receptacles and connectors necessary to operate 12 and 24 volt electrical components of the JLTV companion trailer and military trailers including blackout lights. | X |  |  |  | Inspection shall be used to verify compliance with this requirement. | 3 |
| PD1297 | ###### 3.2.1.16.1.2 | X |  |  |  | 4.2.1.16.1.2 |  |
| PD1298 | Interconnecting coil-type electrical cables (truck to trailer) shall be provided for 12-volt/7 pin and 24 volt/12 pin operation. | X |  |  |  | Inspection shall be used to verify compliance with this requirement. | 3 |
| PD1299 | 3.2.1.16.1.2.1 |  |  | X |  | 4.2.1.16.1.2.1 |  |
| PD1300 | The standard commercial (SAE J560) 12 volt/7 pin receptacles shall be mounted at the front and rear of the vehicle. A 24 volt/12 pin receptacle (STANAG 4007) with cover shall be mounted at the rear of the vehicle. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD1279 | ##### 3.2.1.16.2 Like Vehicle Towing. |  | X |  |  | 4.2.1.16.2 |  |
| PD1284 | The JLTV shall be capable of towing another JLTV in its payload category at GVW for 100 miles over flat secondary roads (T) and 100 miles over cross country terrain (O). All available on-board fuel including additional fuel cans, as well as any necessary attachments, tow bars or couplers, are permitted to meet this requirement. |  | X |  |  | Vehicle shall be tested for the ability to a)tow another like vehicle, b) be towed by another like vehicle. | 3 |
| PD1286 | ##### 3.2.1.16.3 Recovery/Towing. |  | X |  |  | 4.2.1.16.3 |  |
| PD1287 | The JLTV shall be capable of being recovered/lift and flat towed from both the front (at GCVW) and rear (at GVW) by 5-ton M939 series, MTVR, FMTV, and HEMTT wreckers, with no alteration required. |  | X |  |  | The vehicle shall be tested to ensure that it can be recovered/lift and flat towed as described in Section 3 | 3 |
| PD1288 | ##### 3.2.1.16.4 Towed Load Capability. |  |  |  |  | 4.2.1.16.4 |  |
| PD1289 | ###### 3.2.1.16.4.1 Companion Trailer. |  |  |  |  | 4.2.1.16.4.1 |  |
| PD1290 | The primary towed load for the JLTV is the Companion Trailer (CT) and is described in the trailer specific annex. |  |  |  |  | See trailer specific annex. |  |
| PD1291 | ###### 3.2.1.16.4.2 Backward Compatibility. |  |  |  |  | 4.2.1.16.4.2 |  |
| PD1292 | See vehicle specific annex for list of legacy trailers. |  |  |  |  | See vehicle specific annex. | 3 |
| PD1302 | ##### 3.2.1.16.5 Tow Eyes. | X | X |  |  | 4.2.1.16.5 |  |
| PD1303 | Front and rear tow eyes shall be provided on the JLTV. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD1304 | ###### 3.2.1.16.5.1 |  | X |  |  | 4.2.1.16.5.1 |  |
| PD1305 | The tow eyes shall be of a size such that the vehicle can be towed with the heavy-duty towbar described on drawing 12322663, which is referenced in MS500048. |  | X |  |  | The JLTV shall be tested to ensure that it can be towed using the heavy duty tow bar referenced in the Section 3 requirement. | 3 |
| PD7330 | ###### 3.2.1.16.5.2 |  | X |  |  | 4.2.1.16.5.2 |  |
| PD7331 | The tow lugs shall conform to STANAG 4019, and shall connect a towbar conforming to MS500048. |  | X |  |  | The JLTV shall be tested to ensure that it can be towed using the heavy duty tow bar referenced in the Section 3 requirement. | 3 |
| PD1308 | ##### 3.2.1.16.6 Pintle. | X | X |  |  | 4.2.1.16.6 |  |
| PD1309 | An adjustable pintle shall be provided which permits a single operator wearing MOPP level IV to latch/unlatch a trailer from the prime mover within 5 minutes. | X | X |  |  | The pintle will be tested to the requirements stated in Section 3. The pintle will also be inspected for the specified mounting height, operation, lubrication fittings, and evidence of lubrication. | 3 |
| PD6696 | ###### 3.2.1.16.6.1 |  | X |  |  | 4.2.1.16.6.1 |  |
| PD6689 | Hook-up shall be possible with the towed trailer’s tongue offset laterally from 7”(T) and 10”(O) from the centerline of the truck and from 7”(T) to 10”(O) aft of the normal towing position. |  | X |  |  | The pintle will be tested to the requirements stated in Section 3. | 3 |
| PD6697 | ###### 3.2.1.16.6.2 |  | X |  |  | 4.2.1.16.6.2 |  |
| PD6691 | Vehicle pintle shall be located at the rear and centered horizontally on the vehicle. |  | X |  |  | The pintle will be tested to the requirements stated in Section 3. | 3 |
| PD6699 | ###### 3.2.1.16.6.3 |  | X |  |  | 4.2.1.16.6.3 |  |
| PD6693 | The pintle assembly shall cause no interference when towing trailers and Howitzers in a full cramp steer (right or left) while traveling in the forward direction. |  | X |  |  | The pintle will be tested to the requirements stated in Section 3. The pintle will be inspected for the specified mounting height, operation, lubrication fittings, and evidence of lubrication. | 3 |
| PD6701 | ###### 3.2.1.16.6.4 |  |  | X |  | 4.2.1.16.6.4 |  |
| PD6695 | Provision for attachment of trailer safety chains shall be provided as per SAE J849 (per truck installation note) for single axle trailers. |  |  | X |  | The contractor shall certify that safety chains meet SAE J849, the certification data may be examined for compliance | 3 |
| PD3420 | #### 3.2.1.17 Collision Warning System (CWS). |  |  | X |  | 4.2.1.17 |  |
| PD3421 | A collision warning system (CWS) shall be integrated on the vehicle that shall provide the driver a visual and audible indication when the driver is approaching an unsafe stopping distance. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3422 | ##### 3.2.1.17.1 |  |  | X |  | 4.2.1.17.1 |  |
| PD3423 | The central processing unit of the CWS shall be able to compute closing speeds from 0.25 to 100 mph (minimum). |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3424 | ##### 3.2.1.17.2 |  |  | X |  | 4.2.1.17.2 |  |
| PD3425 | The front sensor of the CWS shall have a range of 1 to 350 feet (minimum). |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3426 | ##### 3.2.1.17.3 |  |  | X |  | 4.2.1.17.3 |  |
| PD3427 | Side blind spot sensor(s) shall be provided for blind spot detection. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3428 | ##### 3.2.1.17.4 |  |  | X |  | 4.2.1.17.4 |  |
| PD3429 | The CWS driver interface unit shall warn the driver both visually and audibly with tones when approaching potential hazardous situations. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3436 | ##### 3.2.1.17.5 |  |  | X |  | 4.2.1.17.5 |  |
| PD3437 | The audible alarm shall be controlled by a volume control. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3432 | ##### 3.2.1.17.6 |  |  | X |  | 4.2.1.17.6 |  |
| PD3433 | Vehicle operator shall be able to turn the entire CWS on and off. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD7211 | ##### 3.2.1.17.7 CWS Default. |  |  | X |  | 4.2.1.17.7 |  |
| PD7212 | The CWS shall default to "on" upon each vehicle start. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3434 | ##### 3.2.1.17.8 |  |  | X |  | 4.2.1.17.8 |  |
| PD3435 | Blackout controls shall automatically disable the indicator lights for the Collision Warning System. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3438 | #### 3.2.1.18 Electronic Stability Control System |  | X |  | X | 4.2.1.18 |  |
| PD3439 | The JLTV shall be equipped with an Electronic Stability Control System (ESC) that conforms to FMVSS 126. The ESC shall control all 4 wheels to minimize loss of vehicle control/accidents. The system shall have an on/off switch that will deactivate the system. Preference is for an integrated controller for the ABS/ESC modules. |  | X |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. The test shall be conducted IAW DOT TP-126-00. | 2 |
| PD7213 | ##### 3.2.1.18.1 ESC Default. |  | X |  | X | 4.2.1.18.1 |  |
| PD7214 | The ESC shall default to "on" upon each vehicle start. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. |  |
| PD1036 | ##### 3.2.1.18.2 |  |  |  | X | 4.2.1.18.2 |  |
| PD1037 | If the ABS becomes deactivated it shall not deactivate the ESC (O). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD6820 | #### 3.2.1.19 Road Departure Sensing System. |  | X |  |  | 4.2.1.19 |  |
| PD6821 | The JLTV shall have a system to sense when the vehicle is drifting towards the edge of the road/ditch and sound an audible warning to the driver. The system shall have an on/off switch(s) that deactivates the system. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6722 | #### 3.2.1.20 Vehicle Horn. |  | X |  |  | 4.2.1.20 |  |
| PD6723 | The JLTV shall be equipped with a horn that produces sound at 110 db to 120 db. The horn activation mechanism shall be within easy reach of the driver. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7041 | #### 3.2.1.21 Right Hand Drive. |  |  |  | X | 4.2.1.21 |  |
| PD7042 | The JLTV FoV shall be capable of being produced with either a Left Hand or Right Hand driving station (O). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1310 | ### 3.2.2 SURVIVABILITY. |  |  |  |  | 4.2.2 |  |
| PD1311 | The JLTV shall be designed with a base structure called an “A” structure and an appliqué "B" armor kit that provides higher armor protection levels. The A structure and B armor kit are stated as protection levels for the internal Crew Compartment (CC) (defined as all occupant space in the JLTV). JLTV shall also incorporate various additional survivability technologies to meet overall requirements. |  |  |  |  |  | 2 |
| PD1312 | #### 3.2.2.1 Armor Protection Maintenance. |  | X |  | X | 4.2.2.1 |  |
| PD1313 | All JLTV armor systems shall require no maintenance beyond standard Preventive Maintenance Checks and Services (PMCS) performed by the operator using on-board tools. |  | X |  | X | The JLTV shall be tested and analyzed to verify compliance with the Section 3 requirement. | 3 |
| PD7270 | ##### 3.2.2.1.1 |  | X |  |  | 4.2.2.1.1 |  |
| PD7271 | PMCS or scheduled services shall include a means of checking the armor for damage and corrosion. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1318 | ##### 3.2.2.1.2 Armor Kit Interoperability. | X | X |  | X | 4.2.2.1.2 |  |
| PD1319 | The JLTV B Armor kits shall be designed to achieve maximum interoperability and commonality within the JLTV family of vehicles. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6702 | ###### 3.2.2.1.2.1 |  | X |  | X | 4.2.2.1.2.1 |  |
| PD6703 | B armor kit components shall be interchangeable across the same sub-configuration (T) payload category (O) throughout the life of the JLTV. |  | X |  | X | Testing and analysis shall be used to verify compliance with the Section 3 requirement. | 3 |
| PD1320 | ##### 3.2.2.1.3 JLTV Glass. | X | X | X |  | 4.2.2.1.3 |  |
| PD1321 | The JLTV shall be designed to accept ballistic glass. (See Annex G). The JLTV shall also be able to accept standard automotive glass. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD6714 | ###### 3.2.2.1.3.1 Ballistic Glass. |  | X |  |  | 4.2.2.1.3.1 |  |
| PD6707 | All ballistic glass shall meet the requirements of ATPD 2352. |  | X |  |  | Each JLTV glass kit configuration shall be installed and subjected testing per as required per ATPD 2352 including performance and ballistic testing. This requirement shall be considered met when the glass has successfully completed and passed to each test as specified. | 2 |
| PD6719 | 3.2.2.1.3.1.1 |  | X |  |  | 4.2.2.1.3.1.1 |  |
| PD6711 | All glass when installed shall be interoperable with all automotive functions IE: defrosters, wipers, etc. |  | X |  |  | The glass shall be tested to ensure interoperability with all automotive functions. | 3 |
| PD6720 | 3.2.2.1.3.1.2 |  | X |  |  | 4.2.2.1.3.1.2 |  |
| PD6712 | All glass and frame assemblies installed onto any JLTV shall not detach from their moorings and deflect into any Crew Compartment when subjected to any ballistic and blast events required in Annex E. |  | X |  |  | The glass and frame assemblies shall be tested for the requirement | 3 |
| PD6718 | 3.2.2.1.3.1.3 Protection Levels. |  |  |  |  | 4.2.2.1.3.1.3 |  |
| PD6706 | A structure and B armor kit ballistic glass protection levels are contained in Annex E. |  |  |  |  | See classified Annex E. |  |
| PD1316 | 3.2.2.1.3.1.4 Transparent Armor Storage. |  |  | X |  | 4.2.2.1.3.1.4 |  |
| PD1317 | The JLTV B transparent armor kit shall be designed for storage for 5 years (T), 15 years (O) IAW ATPD 2352 and without any adverse effects as defined in AR 70-38 without additional preparation or preservation equipment, except for elastomeric gaskets, adhesives, etc. The contractor shall minimize, and identify, the use of any materials which have shelf life limitations. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6717 | ###### 3.2.2.1.3.2 Automotive Glass kit. | X | X |  |  | 4.2.2.1.3.2 |  |
| PD6705 | The design of the standard automotive glass and its frame attachment, for the side windows, shall have the ability to open for ventilation purposes without impact from environmental conditions. | X | X |  |  | Inspection shall be conducted to demonstrate compliance with the Section 3 requirement. Testing shall be conducted to verify compliance. | 3 |
| PD1322 | ##### 3.2.2.1.4 Armor Kit Installation. |  | X |  |  | 4.2.2.1.4 |  |
| PD1323 | The B Armor Kit (excluding GPK) shall be mountable within seven (7) hours with MHE (T), five (5) hours without MHE using a two (2) man crew (O). The A structure surface shall require no additional preparation for mounting any B armor Kit. |  | X |  |  | Each B armor kit installation shall be timed during initial installation inspection as specified with MHE and with only onboard tools. The requirement shall be considered met when the time constraint is demonstrated. | 3 |
| PD1324 | #### 3.2.2.2 JLTV Ballistic Protection. |  |  |  | X | 4.2.2.2 |  |
| PD1325 | The JLTV shall provide the ballistic protection requirements contained in the classified annex E. |  |  |  |  | See classified Annex E. |  |
| PD1326 | ##### 3.2.2.2.1 Spall Liners |  |  |  |  | 4.2.2.2.1 |  |
| PD1327 | See classified Annex E. |  |  |  |  | See classified Annex E. | 3 |
| PD1726 | ##### 3.2.2.2.2 Recover from kinetic damage. |  |  |  | X | 4.2.2.2.2 |  |
| PD1727 | At GVW, less 50% fuel, the JLTV shall be capable of traveling for 1Km(T) and 5Km(O) after a single Small Arms fire perforation occurs anywhere in the fuel tank, engine oil reservoir, or cooling system. (See Annex G) |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD1354 | #### 3.2.2.3 Fire Extinguishing. | X | X | X | X | 4.2.2.3 |  |
| PD1355 | The JLTV shall be equipped with automatic fire extinguishing systems (AFES) for the crew area, engine compartment, and cargo compartment (if not isolated from the crew area). The systems shall address slow growth and rapidly developing fuel fires generated by any of the explosive effects contained in Annex E. The systems shall be able to extinguish petroleum, oil, and lubricant (POL) fires before crew members are incapacitated or significant vehicle damage occurs. |  |  |  |  |  | 2 |
| PD1356 | ##### 3.2.2.3.1 General Requirements. | X | X | X | X | 4.2.2.3.1 |  |
| PD1357 | The following requirements apply to the crew area, engine and cargo compartment (as applicable) automatic fire extinguishing systems (AFES). |  |  |  |  |  | 3 |
| PD1358 | ###### 3.2.2.3.1.1 |  | X |  |  | 4.2.2.3.1.1 |  |
| PD1359 | The AFES shall have independent battery back up for at least 10 minutes following any power failure to the system to include ballistic events identified in Annex E. |  | X |  |  | The AFES shall be tested IAW TOP 2-2-508. | 3 |
| PD1362 | ###### 3.2.2.3.1.2 | X |  |  |  | 4.2.2.3.1.2 |  |
| PD1363 | The AFES shall be capable of both automatic sensing and extinguishing, and mechanical manual extinguisher activation. The manual release shall be located within easy reach of the driver and be clearly identified. A guarded toggle switch shall be used for electrical release, if provided. Manual pull force shall not exceed 40 lbf at the point of activation for mechanical release, if provided. | X |  |  |  | To verify compliance with the Section 3 requirement, the AFES shall be examined for both automatic sensing and extinguishing and manual (electrical and/or mechanical) extinguisher activation. | 3 |
| PD1364 | ###### 3.2.2.3.1.3 | X |  |  |  | 4.2.2.3.1.3 |  |
| PD1365 | System status indicators shall be provided that visually inform crewmembers that the AFES is powered and operational. | X |  |  |  | Visual inspection of indicator shall verify crews ability to determine AFES operational status. | 3 |
| PD1366 | ###### 3.2.2.3.1.4 | X |  |  |  | 4.2.2.3.1.4 |  |
| PD1367 | A means shall be provided to disconnect AFES power to permit maintenance. The AFES maintenance power switch shall not be readily accessible to the crewmembers. | X |  |  |  | Visual inspection of the location and the manual actuation of the maintenance power switch shall verify requirement. | 3 |
| PD1368 | ###### 3.2.2.3.1.5 |  |  |  | X | 4.2.2.3.1.5 |  |
| PD1369 | Automatic protection shall be provided to preclude false activation of any extinguisher(s) during removal or replacement of any AFES component. Removal of any individual fire sensor or extinguisher shall not render the remainder of the system inoperative. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1370 | ###### 3.2.2.3.1.6 |  |  |  | X | 4.2.2.3.1.6 |  |
| PD1371 | Components shall be common between the compartments to the maximum extent practicable. Use of components already in the DoD supply system is desired consistent with meeting other requirements in this section. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1376 | ###### 3.2.2.3.1.7 Fire detectors. |  |  |  |  | 4.2.2.3.1.7 |  |
| PD1377 | 3.2.2.3.1.7.1 |  |  |  | X | 4.2.2.3.1.7.1 |  |
| PD1378 | The fire detectors shall not respond (i.e., cause extinguisher(s) to discharge) to external fire sources or other radiation stimuli when all vehicle windows and hatches are closed. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1379 | 3.2.2.3.1.7.2 |  |  |  | X | 4.2.2.3.1.7.2 |  |
| PD1380 | The fire detectors shall not respond to any of the radiation stimuli identified in Table 7 at distances greater than or equal to those specified. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1424 | Table 8. False Alarm Susceptibility Requirements |  |  |  |  |  |  |
| PD7367 |  |  |  |  |  |  |  |
| PD1495 | * Immune at Any Distance (IAD) |  |  |  |  |  |  |
| PD1381 | ###### 3.2.2.3.1.8 Extinguishers. | X |  | X | X | 4.2.2.3.1.8 |  |
| PD1384 | 3.2.2.3.1.8.1 |  |  | X |  | 4.2.2.3.1.8.1 |  |
| PD1385 | The agent and its byproducts shall not cause degradation to any vehicle components. Any cleanup after agent discharge, if required, shall be limited to a water wash after no less than 48 hours after discharge. |  |  | X |  | The Contractor shall certify that the agent and its byproducts shall not cause degradation to any vehicle components and clean-up as required. | 3 |
| PD1386 | 3.2.2.3.1.8.2 |  |  |  | X | 4.2.2.3.1.8.2 |  |
| PD1387 | Agent discharge and any resulting residue shall not interfere with operation of the vehicle or its systems. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1388 | 3.2.2.3.1.8.3 | X |  | X |  | 4.2.2.3.1.8.3 |  |
| PD1389 | All extinguishers shall be equipped with means to prevent accidental discharge during shipping, installation and maintenance. All safety components that are removed following extinguisher installation shall be attached to the extinguisher with a ring and cable or similar device. Mounting provisions shall be provided on the extinguisher mounting bracket to secure these items upon removal. | X |  | X |  | The Contractor shall certify that the all extinguishers are equipped with means to prevent accidental discharge. The extinguishers, when required, shall be inspected to verify compliance. | 3 |
| PD1390 | 3.2.2.3.1.8.4 | X |  |  |  | 4.2.2.3.1.8.4 |  |
| PD1391 | A visual means to accurately determine the charge status of each extinguisher (e.g. pressure gage) shall be provided. This indicator shall be clearly visible to vehicle crewmen when the extinguishers are installed in the vehicle. | X |  |  |  | Visual inspection of indicator shall verify crews ability to determine AFES charge status. | 3 |
| PD1392 | 3.2.2.3.1.8.5 |  |  | X |  | 4.2.2.3.1.8.5 |  |
| PD1393 | The extinguishers shall not leak more than one ounce per year. |  |  | X |  | The Contractor shall certify that the extinguishers conform to required leakage rate. | 3 |
| PD1394 | 3.2.2.3.1.8.6 |  |  | X |  | 4.2.2.3.1.8.6 |  |
| PD1395 | The extinguishers shall not discharge materials or items that could cause physical injury to crewmembers. The extinguishers shall not eject solid fragments larger than 300 microns in diameter under any circumstances. |  |  | X |  | The Contractor shall certify that the extinguishers conform to required fragment size. | 3 |
| PD1396 | 3.2.2.3.1.8.7 | X |  | X |  | 4.2.2.3.1.8.7 |  |
| PD1397 | Cylinders shall meet all applicable Department of Transportation (DOT) requirements, the non-shatterability requirements of MIL-C-7905 Cylinders, Steel, Compressed Gas, Non-Shatterable, Seamless, 1800 PSI and 2100 PSI and be marked and color coded in accordance with MIL-STD-101 Color Code for Pipelines and for Compressed Gas Cylinders. | X |  | X |  | The Contractor shall certify and the extinguishers inspected to verify compliance with the Section 3 requirement. | 3 |
| PD1398 | 3.2.2.3.1.8.8 |  |  | X |  | 4.2.2.3.1.8.8 |  |
| PD1399 | Each extinguisher shall be equipped with a safety relief device to vent internal pressure before it reaches levels that could cause damage to the extinguisher or injury to personnel. The relief device shall not activate at temperatures below 180°F. |  |  | X |  | The Contractor shall certify that the extinguishers conform to safety relief requirements. | 3 |
| PD1400 | 3.2.2.3.1.8.9 |  |  | X |  | 4.2.2.3.1.8.9 |  |
| PD1401 | Extinguisher refill capabilities and procedures shall be compatible with existing Army recharge equipment (NSN 4210-01-474-6206, TB 9-4210-245-50). Any replacement/rebuild parts required to service the extinguisher shall be available in individual kit form. This requirement does not apply if disposable extinguishers are used. |  |  | X |  | The Contractor shall certify that the extinguishers conform to refilling requirements. | 3 |
| PD1402 | ##### 3.2.2.3.2 Crew Compartment. |  | X | X | X | 4.2.2.3.2 |  |
| PD1403 | The fire extinguishing system in the crew compartment shall be capable of detecting and extinguishing peacetime and combat initiated Petroleum, Oil, and Lubricant (POL) fires before crewmen receive second-degree burns. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1404 | ###### 3.2.2.3.2.1 |  |  |  | X | 4.2.2.3.2.1 |  |
| PD1405 | Compartment parameters shall not exceed the crew incapacitation levels during and following any fire event, as contained in the Medical Evaluation of Non-Fragment Injury Effects in Armored Vehicle Live Fire Test, Instrumentation Requirements and Injury Criteria reference document dated September 1989. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1408 | ###### 3.2.2.3.2.2 |  |  | X |  | 4.2.2.3.2.2 |  |
| PD1409 | HFC-227ea (heptafluoropropane) with 5%-10% sodium bicarbonate powder by weight shall be used as the extinguishing agent. Agent concentrations shall not exceed the exposure limits of Table 8 (ref: NFPA 2001, Standard on Clean Agent Fire Extinguishing Systems) under any vehicle operating condition (hatches and windows open/closed, engine on/off, heater or A/C on/off, etc.). |  |  | X |  | The Contractor shall certify that the agent used is HFC-227ea as required. | 3 |
| PD1557 | Table 9. Exposure Limits for HFC-227ea |  |  |  |  |  |  |
| PD7368 |  |  |  |  |  |  |  |
| PD7369 |  |  |  |  |  |  |  |
| PD7370 |  |  |  |  |  |  |  |
| PD7371 |  |  |  |  |  |  |  |
| PD7372 |  |  |  |  |  |  |  |
| PD1410 | ###### 3.2.2.3.2.3 |  | X | X | X | 4.2.2.3.2.3 |  |
| PD1411 | Extinguishers shall not discharge directly at any normally occupied crew or passenger position. Discharge force shall be less than 20 psi at 5 inches from any extinguisher outlet or nozzle. |  | X | X | X | Contractor shall perform analyses and certify that extinguishers do not discharge as required. When required by contract, the AFES shall be tested to verify compliance. | 3 |
| PD1412 | ###### 3.2.2.3.2.4 |  | X | X | X | 4.2.2.3.2.4 |  |
| PD1413 | Impulse noise levels during extinguisher discharge shall be less than 140 dB(A) at any normally occupied crew or passenger positions. |  | X | X | X | Contractor shall perform analyses and certify that extinguishers do not exceed noise levels as required. When required by contract, the AFES shall be tested to verify compliance. | 3 |
| PD1414 | ##### 3.2.2.3.3 Engine Compartment. |  | X |  | X | 4.2.2.3.3 |  |
| PD1415 | POL fires in the engine compartment shall be detected and extinguished within 10 seconds of ignition to minimize vehicle damage. The system shall function either with the engine running or not running. The system shall be automatic with manual backup capability. |  | X |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. When required by contract, the AFES shall be tested to verify compliance. | 3 |
| PD1416 | ##### 3.2.2.3.4 Fuel Tank Protection. | X |  |  | X | 4.2.2.3.4 |  |
| PD1417 | ###### 3.2.2.3.4.1 |  |  |  | X | 4.2.2.3.4.1 |  |
| PD1418 | All fixed fuel tanks shall be self-sealing. (See Annex G) See MIL-T-5578 (as applicable to a tactical vehicle using JP-8) for reference. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD7295 | ###### 3.2.2.3.4.2 Portable Fuel Carrying Capacity. | X |  |  |  | 4.2.2.3.4.2 |  |
| PD1266 | JLTV FoV shall be able to accommodate two (2) standard jerry cans on the vehicle exterior. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD1419 | ###### 3.2.2.3.4.3 | X |  |  | X | 4.2.2.3.4.3 |  |
| PD1420 | All fixed fuel tanks shall be mounted external to the crew compartment or compartmented away from the crew to minimize the occurrence of internal fires. The JLTV design shall incorporate layering protection for fuel tanks where fixed fuel tanks shall be shielded by the JLTV structure. | X |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 1 |
| PD6893 | ###### 3.2.2.3.4.4 |  |  |  | X | 4.2.2.3.4.4 |  |
| PD6894 | A fire protection method shall be provided to allow safe egress of the crew and prevent any sustained fuel fires when the fuel tanks (to include jerry cans) are penetrated by any ballistic and/or blast events listed in Annex E. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD1422 | ##### 3.2.2.3.5 Portable Extinguisher. |  |  | X |  | 4.2.2.3.5 |  |
| PD1423 | The JLTV shall be equipped with at least one portable extinguisher with a minimum 20B:C rating per UL 711 that is mounted within easy reach of the driver. The agent and its byproducts of extinguishment must not be harmful to the crew, vehicle or equipment. If not already done, the agent shall receive a toxicity clearance from the Army Surgeon General via the US Army Center for Health Promotion and Preventive Medicine. |  |  | X |  | The Contractor shall certify that at least one portable extinguisher(s) is mounted and available to the driver as required. | 3 |
| PD1564 | #### 3.2.2.4 Ballistic Shock. |  | X |  | X | 4.2.2.4 |  |
| PD1563 | The B armor kitted JLTV shall absorb the energy in the structure to eliminate any secondary missiles generated from any on-board equipment tearing loose from their moorings and to allow, at a minimum, continued AFES and radio operation capabilities when subjected to any of the ballistic and blast loads specified in Annex E. |  | X |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. When required by contract, the vehicle(s) shall be tested to verify compliance. | 2 |
| PD1569 | #### 3.2.2.5 Signature Management. |  |  |  |  | 4.2.2.5 |  |
| PD1570 | The JLTV shall employ signature reduction techniques and materials in order to reduce its detection. See annex F. |  |  |  |  | See annex F. | 3 |
| PD1573 | #### 3.2.2.6 Vehicle Obscuration Smoke System | X | X |  | X | 4.2.2.6 |  |
| PD1574 | ##### 3.2.2.6.1 Smoke Grenade Launcher. |  |  |  | X | 4.2.2.6.1 |  |
| PD1575 | The JLTV shall have the ability to install the Light Vehicle Obscuration Smoke System (LVOSS) kit consisting of Discharger, Grenade, Smoke, Countermeasure: Lightweight, M7 (NSN 1040-01-454-1625). |  |  |  | X | The JLTV shall be analyzed to demonstrate the capability for installation of an LVOSS kit as required. | 2 |
| PD1576 | ##### 3.2.2.6.2 Configuration. |  |  |  | X | 4.2.2.6.2 |  |
| PD1577 | The JLTV shall be configured with the capability to install up to four (4), M7 discharger(s) with associated grenades, arming/firing unit (AFU), mounting brackets and hardware. |  |  |  | X | The JLTV shall be analyzed to demonstrate compliance with the Section 3 requirement. | 2 |
| PD6889 | ###### 3.2.2.6.2.1 | X |  |  |  | 4.2.2.6.2.1 |  |
| PD6890 | The M7 dischargers installed on the JLTV shall not impair the vehicle operational capabilities. The M7 dischargers shall not interfere with the host vehicle’s weapon systems (stowage, mounting/dismounting tasks/procedures) and shall not obstruct the soldiers' field of view from normal operation. | X |  |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1578 | ##### 3.2.2.6.3 Electrical Interface. |  | X |  |  | 4.2.2.6.3 |  |
| PD1579 | The JLTV shall be equipped with an integral electrical interface to connect the M7 discharger(s) to the appropriate AFU when kit is installed. The electrical interface shall be compatible with the overall JLTV electronic/data architecture and shall not have connectors susceptible to damage when the M7 discharger(s) and AFU are not installed. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6891 | ###### 3.2.2.6.3.1 |  | X |  |  | 4.2.2.6.3.1 |  |
| PD6892 | The JLTV wiring to accommodate multiple M7 dischargers shall be configured in parallel circuit so the loss of any launcher will not effect the function of the remaining dischargers. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1588 | #### 3.2.2.7 Weapon Provisions. | X | X |  |  | 4.2.2.7 |  |
| PD1589 | ##### 3.2.2.7.1 Weapons Mount. |  | X |  |  | 4.2.2.7.1 |  |
| PD6957 | ###### 3.2.2.7.1.1 MK-93 Weapons Mount. |  | X |  |  | 4.2.2.7.1.1 |  |
| PD1590 | The JLTV FoV shall have the capability to accept the MK93 weapons mount to mount the following weapons: |  | X |  |  | The FOV shall be tested to ensure that the MK93 can be mounted and the M2 and MK19 weapons can be mounted. | 3 |
| PD1591 | - M2 |  |  |  |  |  | 3 |
| PD1592 | - MK19 |  |  |  |  |  | 3 |
| PD6958 | ###### 3.2.2.7.1.2 M197 Weapons Mount. |  | X |  |  | 4.2.2.7.1.2 |  |
| PD6959 | The JLTV FoV shall have the capability to accept the M197 weapons mount to mount the following weapons. |  | X |  |  | The FOV shall be tested to ensure that the M197 can be mounted and the M240 and M249 weapons can be mounted. | 3 |
| PD1594 | - M240 |  |  |  |  |  | 3 |
| PD1595 | - M249 |  |  |  |  |  | 3 |
| PD6960 | ###### 3.2.2.7.1.3 ALGL Weapons Mount. |  | X |  |  | 4.2.2.7.1.3 |  |
| PD1593 | The JLTV FoV shall have the capability to accept the Mk 107 weapons mount to mount the Advanced Lightweight Grenade Launcher (ALGL) MK-47. |  | X |  |  | The FOV shall be tested to ensure that the MK107 can be mounted and the Advanced Lightweight Grenade Launcher (ALGL) MK-47 can be mounted. | 3 |
| PD1596 | ##### 3.2.2.7.2 Weapon Mount Integration. | X | X |  |  | 4.2.2.7.2 |  |
| PD6956 | The JLTV shall be capable of meeting the weapons mount integration requirements, as stated below, with the MK-93, M197 and Mk 107 weapons mount. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6961 | ###### 3.2.2.7.2.1 |  | X |  |  | 4.2.2.7.2.1 |  |
| PD1597 | The JLTV with weapons mount and mounted machine-guns, shall permit operation of the weapon while traversing 360 degrees azimuth without interfering with crew operations. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1598 | ###### 3.2.2.7.2.2 |  | X |  |  | 4.2.2.7.2.2 |  |
| PD1599 | The weapon mount equipped with a weapon shall be able to be operated by a 5th to 95th percentile soldier/marine. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1604 | ###### 3.2.2.7.2.3 |  | X |  |  | 4.2.2.7.2.3 |  |
| PD1605 | The crew shall be able to perform all crew service functions on the weapon while it is mounted in operating position, e.g. reloading, immediate action, maintenance. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1606 | ###### 3.2.2.7.2.4 |  | X |  |  | 4.2.2.7.2.4 |  |
| PD1607 | The integration of the weapons mount and weapon on the JLTV shall preclude the firing of the weapon at the vehicle. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1608 | ###### 3.2.2.7.2.5 | X |  |  |  | 4.2.2.7.2.5 |  |
| PD1609 | The weapon when mounted on the JLTV shall be capable of being elevated to +60 degrees and depressed to 20 degrees. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD1610 | ###### 3.2.2.7.2.6 |  | X |  |  | 4.2.2.7.2.6 |  |
| PD1611 | Weapons, when mounted, shall function within the published performance standards of that weapon system. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1616 | ###### 3.2.2.7.2.7 |  | X |  |  | 4.2.2.7.2.7 |  |
| PD1617 | A means shall be provided to prevent spent brass and links from entering the crew compartment. |  | X |  |  | The FOV shall be tested to ensure that the brass and links do not enter the crew compartment | 3 |
| PD1618 | ##### 3.2.2.7.3 Gunner Protection Kit. | X | X | X |  | 4.2.2.7.3 |  |
| PD1619 | ###### 3.2.2.7.3.1 | X |  |  |  | 4.2.2.7.3.1 |  |
| PD1620 | A gunner’s protection kit (GPK) shall be integrated for use on the JLTV. | X |  |  |  | Vehicle shall inspected to verify GPK is provided with weapons station. | 2 |
| PD1621 | ###### 3.2.2.7.3.2 |  | X | X |  | 4.2.2.7.3.2 |  |
| PD1622 | The GPK shall provide the gunner protection from small arms fire and below plane fragmentation at the same level of protection as the vehicle with B armor kit attached. (See Annex E) |  | X | X |  | The contractor shall certify that the GPK materiel provides the same protection level as the B armor kit. The JLTV shall be tested to ensure that the GPK provides the gunner protection from small arms fire and below plane fragmentation at the same level of protection as the vehicle with B armor kit attached. | 3 |
| PD6838 | 3.2.2.7.3.2.1 |  | X |  |  | 4.2.2.7.3.2.1 |  |
| PD6839 | The GPK shall provide the gunner with 360 degree azimuth protection. |  | X |  |  | The FOV shall be tested to ensure that the GPK provides the gunner, 360 degree protection. | 3 |
| PD6964 | 3.2.2.7.3.2.2 |  | X |  |  | 4.2.2.7.3.2.2 |  |
| PD6965 | The GPK shall protect the gunner to a minimum height of name tag defilade of a 95% soldier/marine when operating all required weapons. |  | X |  |  | The FoV shall be tested to the Section 3 requirement | 3 |
| PD1625 | ###### 3.2.2.7.3.3 |  | X |  |  | 4.2.2.7.3.3 |  |
| PD1626 | The GPK shall not interfere with the crew’s ability to operate the weapon. This includes all crew service functions IAW published weapons manual standards. |  | X |  |  | The FoV shall be tested to ensure that the GPK does not interfere with the crew’s ability to operate the weapon. This includes all crew service functions IAW published weapons manual standards. | 3 |
| PD7039 | ###### 3.2.2.7.3.4 Lateral Traverse. | X |  |  |  | 4.2.2.7.3.4 |  |
| PD7040 | The weapon shall traverse +/- 20 degrees minimum with GPK stationary while utilizing the MK-93, M197 and Mk 107 weapons mounts. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6962 | ###### 3.2.2.7.3.5 Traversing Unit. |  | X |  |  | 4.2.2.7.3.5 |  |
| PD6970 | 3.2.2.7.3.5.1 Motorized Traversing Unit. |  | X |  |  | 4.2.2.7.3.5.1 |  |
| PD7033 | The GPK shall have a motorized traversing unit to allow for it to rotate 360 degrees continuous. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7032 | 3.2.2.7.3.5.1.1 |  | X |  |  | 4.2.2.7.3.5.1.1 |  |
| PD6963 | The GPK turret shall have a slip ring for continuous power to a 360-degree motorized traversing unit and/or electronic equipment mounted above the turret. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6971 | 3.2.2.7.3.5.1.2 |  | X |  |  | 4.2.2.7.3.5.1.2 |  |
| PD6972 | On JLTV variants with a shelter that exceeds the roofline, the motorized traversing unit shall traverse 180 degrees (T) 220 degrees (O). |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6966 | 3.2.2.7.3.5.2 Manual Traversing Gear. |  | X |  |  | 4.2.2.7.3.5.2 |  |
| PD6967 | The GPK shall have a manual traversing gear to traverse the GPK in the event of power loss. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1629 | ###### 3.2.2.7.3.6 |  | X |  |  | 4.2.2.7.3.6 |  |
| PD1630 | The GPK shall be installable (to include assembly) with MHE using 2 operators plus an MHE operator within four (4) hours (T), two (2) hours (O). |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6840 | ###### 3.2.2.7.3.7 |  | X |  |  | 4.2.2.7.3.7 |  |
| PD6841 | The GPK shall be capable of accepting an overhead protection kit with the same level of B armor kit protection as the vehicle roof. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6886 | ###### 3.2.2.7.3.8 Stowage. | X | X |  |  | 4.2.2.7.3.8 |  |
| PD7036 | 3.2.2.7.3.8.1 Weapon Stowage. |  | X |  |  | 4.2.2.7.3.8.1 |  |
| PD6887 | The GPK shall be capable of storing and securing the M16 and M4 personal weapons with the butt stock in the collapsed or extended positions. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7034 | 3.2.2.7.3.8.1.1 | X | X |  |  | 4.2.2.7.3.8.1.1 |  |
| PD7035 | The personal weapons shall be easily accessible from the left or right sides of the GPK for immediate use within 2 seconds. | X | X |  |  | The JLTV shall be tested and inspected to verify compliance with the Section 3 requirement | 3 |
| PD7037 | 3.2.2.7.3.8.2 Ammo Stowage. | X |  |  |  | 4.2.2.7.3.8.2 |  |
| PD7038 | The GPK shall be capable of storing (See Annex G). | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD1639 | ###### 3.2.2.7.3.9 Gunner’s Restraint System. | X | X |  |  | 4.2.2.7.3.9 |  |
| PD6842 | 3.2.2.7.3.9.1 | X |  |  |  | 4.2.2.7.3.9.1 |  |
| PD6843 | The gunner's restraint system shall prevent the gunner from being inadvertently ejected during off-road operations or in an accident. The restraint shall not inhibit the gunner from ducking back inside the vehicle during a vehicle rollover. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6844 | 3.2.2.7.3.9.2 | X | X |  |  | 4.2.2.7.3.9.2 |  |
| PD6845 | The gunner's restraint system shall allow weapons operation of the gunner in full combat equipment for a duration of 10 hours. | X | X |  |  | The JLTV shall be inspected and tested to verify compliance with the Section 3 requirement. | 3 |
| PD7272 | 3.2.2.7.3.9.3 |  | X |  |  | 4.2.2.7.3.9.3 |  |
| PD7273 | The gunner's restraint system shall allow for multiple (adjustable) seating heights varying from complete defilade to name tag defilade, and accommodating 5th percentile female to the 95th percentile male gunners such that the gunner can effectively employ the assigned weapon. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7311 | 3.2.2.7.3.9.3.1 |  | X |  |  | 4.2.2.7.3.9.3.1 |  |
| PD7312 | The adjustable gunner restraint system shall enable the gunner to maintain 360 degree visibility with only his/her head/kevlar exposed. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1640 | ##### 3.2.2.7.4 Storage. | X | X | X | X | 4.2.2.7.4 |  |
| PD7221 | ###### 3.2.2.7.4.1 Ammunition Storage. |  |  | X |  | 4.2.2.7.4.1 |  |
| PD7220 | 3.2.2.7.4.1.1 |  |  | X |  | 4.2.2.7.4.1.1 |  |
| PD1641 | Provisions shall be made for weapon ammunition storage that meet U.S. Army Defense Ammunition Center and School (USADACS) security certification requirements to transport ammunition over the vehicle mission profile. |  |  | X |  | Certification shall be provided to verify compliance with this requirement. | 3 |
| PD1642 | 3.2.2.7.4.1.2 |  | X |  |  | 4.2.2.7.4.1.2 |  |
| PD1643 | Storage provisions shall have a readily accessible quick release. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1644 | 3.2.2.7.4.1.3 | X |  |  |  | 4.2.2.7.4.1.3 |  |
| PD1645 | The JLTV FoV shall have designated stowage locations, protected inside the vehicle, for the ammunition quantities listed in Annex G. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD1664 | ###### 3.2.2.7.4.2 Stored Ammunition Safety. |  |  |  |  | 4.2.2.7.4.2 |  |
| PD1665 | 3.2.2.7.4.2.1 |  |  |  | X | 4.2.2.7.4.2.1 |  |
| PD1666 | Each JLTV shall protect the crew from on-board weapon stored ammunition detonation to minimize crew casualties. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD1669 | 3.2.2.7.4.2.2 | X |  |  |  | 4.2.2.7.4.2.2 |  |
| PD1670 | Egress shall not be impeded by having to move or relocate parts of the ammunition storage compartment(s) | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD7222 | ###### 3.2.2.7.4.3 Weapons Storage | X |  |  |  | 4.2.2.7.4.3 |  |
| PD7223 | The JLTV shall have designated stowage locations for ground mount weapons components (e.g. tripods for guns). | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD1671 | ##### 3.2.2.7.5 Common Remotely Operated Weapons Station (CROWS). | X |  |  |  | 4.2.2.7.5 |  |
| PD1672 | ###### 3.2.2.7.5.1 | X |  |  |  | 4.2.2.7.5.1 |  |
| PD1673 | The JLTV shall be designed ready to accept a CROWS IAW the CROWS ICD. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD7224 | ###### 3.2.2.7.5.2 | X |  |  |  | 4.2.2.7.5.2 |  |
| PD7225 | The CROWS view shall be accessible to the vehicle commander via the Display and Control Subsystem. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD7226 | ###### 3.2.2.7.5.3 | X |  |  |  | 4.2.2.7.5.3 |  |
| PD7227 | The CROWS Integration shall receive all power from the vehicle power management/distribution system. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6846 | ###### 3.2.2.7.5.4 | X |  |  |  | 4.2.2.7.5.4 |  |
| PD6847 | The JLTV shall have a man-sized opening to facilitate manual operation of the CROWS. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD7266 | ###### 3.2.2.7.5.5 | X |  |  |  | 4.2.2.7.5.5 |  |
| PD7267 | The CROWS shall not interfere with crew egress. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD1700 | #### 3.2.2.8 Chemical, Biological, Radiological/Nuclear, and Explosive Incidents (CBRNE). | X | X | X | X | 4.2.2.8 |  |
| PD1704 | ##### 3.2.2.8.1 Chemical Protection. | X | X |  |  | 4.2.2.8.1 |  |
| PD1706 | The JLTV crew shall have the capability to operate in a CBRNE environment for 6 hours (T) 12 hours (O) wearing MOPP IV gear. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD1711 | ##### 3.2.2.8.2 Detection. | X |  |  |  | 4.2.2.8.2 |  |
| PD1702 | ###### 3.2.2.8.2.1 Chemical Detection. | X |  |  |  | 4.2.2.8.2.1 |  |
| PD1703 | The JLTV shall have the capability to accept a sensor (GFE), to detect and classify chemical and TIC/TIMs agents, compliant with the Common CBRN Sensor Interface (CCSI) document. | X |  |  |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD1712 | ###### 3.2.2.8.2.2 Nuclear Detection. | X |  |  |  | 4.2.2.8.2.2 |  |
| PD1713 | The JLTV shall have the capability to accept a sensor (GFE), to detect and quantify nuclear contamination gamma and neutron radiation (T), alpha and beta particles (O), compliant with the Common CBRN Sensor Interface (CCSI) document. | X |  |  |  | Certification shall be provided to verify compliance with the Section 3 requirements. | 2 |
| PD1718 | ##### 3.2.2.8.3 Decontamination. |  |  | X |  | 4.2.2.8.3 |  |
| PD1719 | The vehicle shall be capable of being decontaminated with Super Tropical Bleach per MIL-DTL-12468. The equipment and components within the JLTV shall be decontaminated using non-corrosive decontaminates, e.g. soap and water or individual decontamination kits. All equipment shall be capable of withstanding the material damaging effects of CBRNE decontamination unless the equipment will be replaced instead of being decontaminated. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2683 | #### 3.2.2.9 EMI |  | X | X |  | 4.2.2.9 |  |
| PD1714 | ##### 3.2.2.9.1 High-Altitude Electromagnetic Pulse (HEMP). |  |  | X |  | 4.2.2.9.1 |  |
| PD1715 | The JLTV shall survive the initial nuclear weapons effects (INWE) of blast, thermal radiation, and initial nuclear radiation (INR) to the extent that a vehicle with crew maintains its combat effectiveness. The JLTV shall survive a high altitude electromagnetic pulse (HEMP) in accordance with criteria specified in MIL STD 2169. The JLTV shall be capable of normal restart immediately after a HEMP induced shutdown or disruption (T), operate through HEMP event (O). |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD1716 | ###### 3.2.2.9.1.1 Electromagnetic Emissions and HEMP. |  |  | X |  | 4.2.2.9.1.1 |  |
| PD1717 | Electromagnetic Interference (EMI). The JLTV shall comply with applicable military EMI and electromagnetic emission susceptibility requirements per MIL-STD-461, MIL-STD-464 and MIL-STD-2169, and commercial electromagnetic compatibility standards/recommendations as needed to support electronic engine, transmission, braking, CTIS controls, and other vehicle electronic components/controls. The system shall protect personnel, fuels and ordnance from the hazardous effects of electromagnetic radiation. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirements. | 2 |
| PD2684 | ##### 3.2.2.9.2 Electromagnetic Environmental Effects (E3) |  | X | X |  | 4.2.2.9.2 |  |
| PD2685 | ###### 3.2.2.9.2.1 |  |  | X |  | 4.2.2.9.2.1 |  |
| PD2686 | The JLTV shall fully meet the requirements of MIL-STD-464. Components and Sub-systems shall fully meet the EMI requirements of MIL-STD-461 when integrated. |  |  | X |  | Certification shall be provided to verify compliance with this requirement. | 2 |
| PD2689 | ###### 3.2.2.9.2.2 |  |  | X |  | 4.2.2.9.2.2 |  |
| PD2690 | There shall be neither unacceptable response nor malfunction of any JTLV system or subsystem due to EMI produced by any or all of the JTLV systems and sub-systems. The JTLV systems and Sub-systems shall be Electromagnetically Compatible (EMC) with each other. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2691 | ###### 3.2.2.9.2.3 |  | X | X |  | 4.2.2.9.2.3 |  |
| PD2692 | The E3 MIL-STD-464 methods to be applied to the JTLV are listed as follows: |  |  |  |  |  |  |
| PD2697 | 3.2.2.9.2.3.1 |  |  | X |  | 4.2.2.9.2.3.1 |  |
| PD2698 | The JTLV shall meet the requirements of Electrostatic Discharge (ESD) for 25KV using both 500pF and 500ohms and 500pF and 5000 ohms. For 15KV use 150 picofarads + 20% capacitance, and 330 Ohms + 20 % series resistance IAW RTCA DO-160D, Section 25. RF Hazards to personnel, ordinance, and fuel (HERP/HERO/HERF) |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6983 | 3.2.2.9.2.3.2 |  | X |  |  | 4.2.2.9.2.3.2 |  |
| PD6984 | The JTLV shall not exhibit RF level which effect Hazards to personnel, ordinance, and fuel (HERP/HERO/HERF) in accordance with MIL-STD-464. |  | X |  |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2699 | 3.2.2.9.2.3.3 |  | X |  |  | 4.2.2.9.2.3.3 |  |
| PD2700 | The JTLV shall meet Life cycle hardness requirements for maintainability and component replacement. |  | X |  |  | Testing shall be verified by using anechoic chambers and calibrated test equipment. Demonstration shall be considered successful for when the JLTV can be maintained after reliability testing and each JLTV system and sub-system operate normally according the required specifications. | 3 |
| PD2727 | #### 3.2.2.10 Grounding and Bonding | X | X |  | X | 4.2.2.10 |  |
| PD2728 | ##### 3.2.2.10.1 |  | X |  |  | 4.2.2.10.1 |  |
| PD2729 | The JTLV shall provide a grounding system that is compliant with the Electromagnetic Environment and Electrical Safety requirements in accordance with CECOM TR 98-6, MIL-HDBK-1857, and UL 1950. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2730 | ##### 3.2.2.10.2 |  | X |  |  | 4.2.2.10.2 |  |
| PD2731 | The grounding system shall provide an adequate ground in a variety of environmental soil conditions, for use while the vehicle is halted in accordance with CECOM TR 98-6. Additionally, the JTLV systems and sub-systems shall be compliant with the following: |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2734 | ###### 3.2.2.10.2.1 |  | X |  |  | 4.2.2.10.2.1 |  |
| PD2735 | The JTLV system and associated subsystems shall provide external grounding provisions to control electrical current flow and static charging for protection of personnel from shock, prevention of inadvertent ignition of ordnance, fuel and flammable vapors, and protection of hardware from damage in accordance with CECOM TR 98-6 and MIL-HDBK-1857. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2738 | ###### 3.2.2.10.2.2 |  | X |  |  | 4.2.2.10.2.2 |  |
| PD2739 | For JTLV systems using structure for power return currents, bonding provisions shall be provided for current return paths for the electrical power sources such that the total voltage drops between the point of regulation for the power system and the electrical loads are within the tolerances of the applicable power quality standard in accordance with CECOM TR 98-6 and MIL-HDBK-1857. |  | X |  |  | The test will use data collected from the data bus and visual inspection of C4/EW system displays to verify successful Data transfer between all C4/EW systems.Verification shall be considered successful when the test shows that all message types can be sent and successfully received between all C4/EW systems IAW their ICD's. | 3 |
| PD2752 | ###### 3.2.2.10.2.3 |  | X |  |  | 4.2.2.10.2.3 |  |
| PD2753 | For instances where specific controls have not been established for a system and approved by the procuring activity, the following Direct Current (DC) bonding levels shall apply throughout the life of the JTLV system in accordance with MIL-STD-464 and MIL-HDBK-1857: |  | X |  |  | Test shall be performed using a calibrated milliohm meter. Testing shall be considered successful for meeting milliohm requirements for faying surfaces and connectors of all JLTV systems and sub-systems. | 3 |
| PD2754 | a. 10 milliohms (mohms) or less from the JLTV equipment enclosure to the system structure, including the cumulative effect of all faying surface interfaces. |  | X |  |  | Test shall be performed using a calibrated milliohm meter. Testing shall be considered successful for meeting milliohm requirements for faying surfaces and connectors of all JLTV systems and sub-systems. | 3 |
| PD2755 | b. 15 mohms or less from cable shields to the equipment enclosure, including the cumulative effect of all connector and accessory interfaces. |  | X |  |  | Test shall be performed using a calibrated milliohm meter. Testing shall be considered successful for meeting milliohm requirements for faying surfaces and connectors of all JLTV systems and sub-systems. | 3 |
| PD2756 | c. 2.5 mohms or less across individual faying interfaces within the equipment, such as between subassemblies or sections. |  | X |  |  | Test shall be performed using a calibrated milliohm meter. Testing shall be considered successful for meeting milliohm requirements for faying surfaces and connectors of all JLTV systems and sub-systems. | 3 |
| PD2757 | ##### 3.2.2.10.3 | X |  |  | X | 4.2.2.10.3 |  |
| PD2758 | Ground rods shall be supported, for use while the JTLV is halted, to create a ground point for the vehicle IAW the requirements of CECOM TR 98-6. | X |  |  | X | Inspection and Analysis shall be performed to verify compliance with the requirement. | 3 |
| PD2761 | ##### 3.2.2.10.4 | X |  |  |  | 4.2.2.10.4 |  |
| PD2762 | The JTLV systems and sub-systems shall meet the requirements of the National Electrical Code (NEC). | X |  |  |  | Inspection shall be performed visually and compared to NEC and configuration documentation. Inspection is successful if JLTV meets NEC and configuration documentation.. | 2 |
| PD2763 | ##### 3.2.2.10.5 | X |  |  |  | 4.2.2.10.5 |  |
| PD2764 | Equipment grounding shall comply with the requirements of CECOM TR 98-6. Hinges and slides shall not be relied upon as the sole means of grounding. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD2769 | #### 3.2.2.11 Intra System Electromagnetic Compatibility (EMC) |  | X |  |  | 4.2.2.11 |  |
| PD2770 | MIL-STD-464 E3 for Intra-system Electromagnetic Compatibility (EMC) where the JTLV systems and sub-systems including C4/EW system shall be compatible within itself such that system operational performance requirements are met. |  | X |  |  | Test shall require an anechoic chamber and calibrated test equipment, where analysis shall use M&S or paper calculation. Test shall be successful for the JLTV system and sub-systems to meet operational performance and analysis is successful if calculation or M&S where system and sub-system meet operational performance. | 3 |
| PD2771 | ##### 3.2.2.11.1 Radiated Susceptibility. |  | X |  |  | 4.2.2.11.1 |  |
| PD2772 | The JTLV systems and sub-systems shall meet the RS103 radiated susceptibility requirements of MIL-STD-461, Table IV, as specified for Army ground vehicles with exception, including the optional requirements. Frequencies and field strengths shall be as follows: |  | X |  |  | Test shall require Anechoic chambers, calibrated RF test equipment, calibrated antennas, and High Powered Amplifiers. Test shall be successful for when each system and sub-system of the JLTV is not susceptible to the specified RS 103 levels. Not being susceptible means each system and sub-system shall meet operational performance. | 2 |
| PD2773 | a. 10 kHz to 2 MHz at 20 V/m |  | X |  |  | Test shall require Anechoic chambers, calibrated RF test equipment, calibrated antennas, and High Powered Amplifiers. Test shall be successful for when each system and sub-system of the JLTV is not susceptible to 20 V/m levels. Not being susceptible means each system and sub-system shall meet operational performance. |  |
| PD2774 | b. 2 MHz to 40 GHz at 50 V/m |  | X |  |  | Test shall require Anechoic chambers, calibrated RF test equipment, calibrated antennas, and High Powered Amplifiers. Test shall be successful when each system and sub-system of the JLTV is not susceptible to 50 V/m levels. Not being susceptible means each system and sub-system shall meet operational performance. |  |
| PD2775 | ##### 3.2.2.11.2 Radiated Emissions |  | X |  |  | 4.2.2.11.2 |  |
| PD2776 | The JTLV plus system and sub-systems shall meet the Army Ground RE102 requirements for MIL-STD-461, for frequencies from 2 MHz to 20 GHz or 10 times the highest intentionally generated frequency of the JTLV. |  | X |  |  | Tests shall require anechoic chambers, RF calibrated test equipment, and calibrated antennas. Test shall be successful when the JLTV plus systems and sub-systems have emissions equal to or less than the RE102 Army Ground requirement. | 2 |
| PD2777 | ##### 3.2.2.11.3 Conducted Susceptibility |  | X |  |  | 4.2.2.11.3 |  |
| PD2778 | The JTLV shall meet the following Conducted Susceptibility requirements of MIL-STD-461 via Signal or Power entry panels of the JTLV: |  | X |  |  | Tests shall require a calibrated fixture for the injection and monitor probes. Test shall be successful when the JLTV exhibits no susceptibilities to the applied CS levels. | 2 |
| PD2779 | a. CS101, Figure CS101-1 |  | X |  |  | Tests shall require a calibrated fixture for the injection and monitor probes. Test shall be successful when the JLTV exhibits no susceptibilities to the applied CS 101 levels. |  |
| PD2780 | b. CS114, Figure CS114-1, Curve #3 from 10 kHz to 2 MHz, and curve #4 from 2 MHz to 400 MHz |  | X |  |  | Tests shall require a calibrated fixture for the injection and monitor probes. Test shall be successful when the JLTV exhibits no susceptibilities to the applied CS114 levels. |  |
| PD2781 | c. CS115, Figure CS115-1, at a 30 Hz rate for one minute |  | X |  |  | Tests shall require a calibrated fixture for the injection and monitor probes. Test shall be successful when the JLTV exhibits no susceptibilities to the applied CS 115 levels. |  |
| PD2782 | ##### 3.2.2.11.4 Co-site Interference |  | X |  |  | 4.2.2.11.4 |  |
| PD2783 | ###### 3.2.2.11.4.1 |  | X |  |  | 4.2.2.11.4.1 |  |
| PD2784 | The JLTV platforms shall mitigate cosite interference from multiple co-located antennas with simultaneous adjacent channel transmitting frequencies and harmonics. |  | X |  |  | Test shall be performed on the JTLV requiring certified anechoic chambers for antennas and analysis shall use modeling and simulation (M&S) for recommended antenna placement on the JLTV platform, band pass/notch filters, active cosite devices, and/or spectrum/ frequency authorization. Test and Analysis shall be considered successful when JLTV co-site interference interactions are minimized by test results and M&S recommendations. | 2 |
| PD2785 | ###### 3.2.2.11.4.2 |  | X |  |  | 4.2.2.11.4.2 |  |
| PD2786 | Commercial Off The Shelf (COTS), Non Developmental Items (NDI), and untested radios components shall meet the requirements for RE102 and RS103 of MIL-STD-461 at a minimum to ensure RF compatibility between intra-system components. |  | X |  |  | RE102 and RS103 tests shall be performed on all JTLV COTS and NDI items before being accepted by the JTLV contractor. Testing will require the use of Anechoic chambers, calibrated RF test equipment, calibrated antennas, and High Powered Amplifiers. Test shall be considered successful upon JTLV COTS and NDI meeting RE102 and RS103 requirements. Once JTLV COTS and NDI are RE102 and RS103 approved, each COT and NDI delivery shall have a certification associated with it. Periodic RE102 and RS103 testing of the COTS or NDI deliveries shall be required. | 1 |
| PD1728 | ### 3.2.3 TRANSPORTABILITY. | X | X | X | X | 4.2.3 |  |
| PD1732 | #### 3.2.3.1 Cargo Tiedowns. | X |  | X |  | 4.2.3.1 |  |
| PD1733 | The JLTV FoV shall be equipped with cargo tiedowns (recessed tie-downs where applicable) to secure the payload and equipment. | X |  | X |  | Inspection and Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3988 | ##### 3.2.3.1.1 |  |  | X |  | 4.2.3.1.1 |  |
| PD3989 | The cargo tiedowns shall be in accordance with MIL-STD-209. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD1734 | #### 3.2.3.2 Lifting & Tiedown Provisions. | X |  | X | X | 4.2.3.2 |  |
| PD1735 | JLTV FoV and companion trailers shall be equipped with lifting and tiedown provisions in accordance with MIL-STD-209, MIL-STD-1366 and design guidelines contained in MIL-HDBK-1791. | X |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3991 | ##### 3.2.3.2.1 | X |  |  |  | 4.2.3.2.1 |  |
| PD3992 | The lifting and tiedown provisions shall be permanently marked. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3990 | ##### 3.2.3.2.2 | X |  |  | X | 4.2.3.2.2 |  |
| PD3993 | The lifting and tiedown provisions shall not interfere with the payload. | X |  |  | X | The JLTV shall be inspected, and an Analysis provided, to verify compliance to the Section 3 requirement. | 3 |
| PD1736 | #### 3.2.3.3 Air Transport. | X | X |  | X | 4.2.3.3 |  |
| PD1738 | ##### 3.2.3.3.1 Fixed Wing Transport. | X | X |  | X | 4.2.3.3.1 |  |
| PD1739 | The JLTV FoV and companion trailers at GVW shall be air-transportable by C-130 and larger military aircraft. | X | X |  | X | Inspection and Analysis shall be provided to demonstrate compliance to the Section 3 requirement. Testing shall be conducted to verify compliance. | 2 |
| PD1740 | ###### 3.2.3.3.1.1 | X | X |  | X | 4.2.3.3.1.1 |  |
| PD1741 | The JLTV FoV and companion trailers shall be in accordance with air transport requirements of MIL-STD-1366 and design guidelines contained in MIL-HDBK-1791. | X | X |  | X | Inspection and Analysis shall be provided to demonstrate compliance to the Section 3 requirement. Testing shall be conducted to verify compliance. | 2 |
| PD1742 | ###### 3.2.3.3.1.2 Mission Scenario. |  |  |  |  | 4.2.3.3.1.2 |  |
| PD1743 | The C-130 mission conditions shall be based on an Armored C-130E/H with payload limited for a range of 1,000 nautical miles. Range requirement shall be met under ideal conditions, at sea level, with a normal landing on a 5,000 foot runway and the aircraft being refueled at the landing site. |  |  |  |  |  | 2 |
| PD1746 | ###### 3.2.3.3.1.3 Vehicle Quantities. |  |  |  |  | 4.2.3.3.1.3 |  |
| PD1747 | Quantities of vehicles per mission and vehicle preparation for fixed wing transport are identified in the vehicle specific annexes. |  |  |  |  | See vehicle specific annex |  |
| PD3938 | ###### 3.2.3.3.1.4 Low Velocity Aerial Delivery (LVAD). |  |  |  | X | 4.2.3.3.1.4 |  |
| PD4019 | The JLTV FoV and companion trailers shall be capable of LVAD in accordance with MIL-HDBK-669, MIL-STD-814, MIL-HDBK-1791 and MIL-STD-1366. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD4021 | 3.2.3.3.1.4.1 |  |  |  | X | 4.2.3.3.1.4.1 |  |
| PD4020 | The JLTV FoV shall be capable of LVAD without shelters installed (T), with shelters (O). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD3939 | 3.2.3.3.1.4.2 |  |  |  | X | 4.2.3.3.1.4.2 |  |
| PD1762 | The JLTV FoV at GVW (excluding GPK) and companion trailers at GVW shall be individually capable of LVAD from C-130, C-17 and C-5 aircraft (T). The JLTV FoV at GVW (excluding GPK) and companion trailers at GVW shall be capable of LVAD simultaneously (on the same platform) from C-130, C-17 and C-5 aircraft (O). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD7228 | 3.2.3.3.1.4.2.1 |  |  |  | X | 4.2.3.3.1.4.2.1 |  |
| PD7229 | The JLTV-C-AMB four-litter ambulance shall be air-droppable by C-5 and C-17 only. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD4022 | 3.2.3.3.1.4.3 |  |  |  | X | 4.2.3.3.1.4.3 |  |
| PD1764 | The vehicle shall be ready for operation after LVAD within 15 minutes (T) and 10 minutes (O). Time to remount the shelter shall not be included in this time period. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1748 | ##### 3.2.3.3.2 Rotary Wing Aircraft. |  | X | X | X | 4.2.3.3.2 |  |
| PD1749 | The JLTV FoV and companion trailers shall be air-transportable by military rotary wing aircraft in accordance with air transport requirements of MIL-STD-1366, with the exception that the maximum transport weights of the aircraft shall be as called out in the transportability table below, under the stated mission conditions. |  | X |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. Testing shall be conducted to verify compliance. | 1 |
| PD7076 | Table 10. Rotary Wing Transport Weights and Mission Scenarios (assuming 4 wheel configuration) |  |  |  |  |  |  |
| PD7077 | Temperature, ºF (ºC) Elevation, ft Combat Radius, nm CH-53K 91.5 (33) 3000 110 27,000 2500 lbs per wheel (10,000 lbs max) CH-53E 59 (15) sea-level 110 16,800 2500 lbs per wheel (10,000 lbs max) MV-22 Block C 91.5 (33) 3000 110 10,000 Not Applicable CH-47F 95 (35) 4000 60 17,940 2500 lbs per wheel (10,000 lbs max) MH-47G 95 (35) 4000 30 12,000 2500 lbs per wheel (10,000 lbs max) UH-60M 95 (35) 4000 30 9,000 Not Applicable Transport Mission Scenario Lift Capacity, lbs (external) Lift Capacity, lbs (internal) |  |  |  |  |  |  |
| PD4025 | ###### 3.2.3.3.2.1 Slinging |  | X | X |  | 4.2.3.3.2.1 |  |
| PD4026 | The sling provisions shall comply with the design and performance criteria specified in MIL-STD-209. |  | X | X |  | Testing shall be conducted, and a certification provided, to verify compliance with the Section 3 requirement. | 2 |
| PD1752 | ###### 3.2.3.3.2.2 Vehicle Quantity. |  |  |  |  | 4.2.3.3.2.2 |  |
| PD1753 | Quantities of vehicles per mission and vehicle preparation for rotary wing transport are identified in the vehicle specific annexes. |  |  |  |  | See vehicle specific annex |  |
| PD1754 | #### 3.2.3.4 Highway Transport. |  |  |  | X | 4.2.3.4 |  |
| PD1755 | The JLTV FoV shall not exceed USA and NATO highway legal permit limits when operated as a self-propelled vehicle. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD4029 | ##### 3.2.3.4.1 |  |  |  | X | 4.2.3.4.1 |  |
| PD4031 | The JLTV vehicle shall not exceed highway permit limits when operated in combination with the companion trailer. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD4028 | ##### 3.2.3.4.2 |  |  |  | X | 4.2.3.4.2 |  |
| PD4033 | The JLTV FoV and companion trailers (coupled and uncoupled) shall not exceed highway weight and dimensional permit limits when carried as cargo by highway transportation assets of the Army, USMC, NATO, or allied countries. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD4027 | ##### 3.2.3.4.3 |  |  |  | X | 4.2.3.4.3 |  |
| PD4034 | The highway transport requirements shall be applicable at all vehicle load conditions from CW to GCVW. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1756 | #### 3.2.3.5 Rail Transport. |  | X |  | X | 4.2.3.5 |  |
| PD1757 | The JLTV FoV and companion trailers shall be rail transportable on CONUS and NATO country railways in accordance with MIL-STD-1366. |  | X |  | X | Analysis and Testing will be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD4036 | ##### 3.2.3.5.1 Dimensional Requirements. |  | X |  | X | 4.2.3.5.1 |  |
| PD4038 | The JLTV FoV and companion trailers shall meet the dimensional requirements of the Association of American Railroads (AAR) “Outline Diagram for Single Loads, Without End Overhang, on Open Top Cars” and the Gabarit International de Chargement (GIC) gauge when loaded on a 51-inch deck-height railcar. |  | X |  | X | Analysis and Testing will be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD4035 | ##### 3.2.3.5.2 Rail Impact. |  | X |  | X | 4.2.3.5.2 |  |
| PD4039 | The JLTV FoV and companion trailers at GCVW shall withstand the rail impact test specified in MIL-STD-810. |  | X |  | X | Analysis and Testing will be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD4037 | ###### 3.2.3.5.2.1 |  | X |  | X | 4.2.3.5.2.1 |  |
| PD4040 | The JLTV FoV shall be capable of rail transport requirements coupled or un-coupled to their companion trailer. |  | X |  | X | Analysis and Testing will be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD4041 | ##### 3.2.3.5.3 |  |  |  | X | 4.2.3.5.3 |  |
| PD4042 | Disassembly of the JLTV FoV or companion trailers shall not be required. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD1758 | #### 3.2.3.6 Sealift Transport. | X | X |  | X | 4.2.3.6 |  |
| PD6911 | ##### 3.2.3.6.1 | X | X |  | X | 4.2.3.6.1 |  |
| PD4043 | The JLTV FoV and companion trailers shall be transportable on all classes of ocean-going transport ships in accordance with MIL-STD-1366. | X | X |  | X | Inspection, Analysis and Tests will be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD4046 | ##### 3.2.3.6.2 | X | X |  | X | 4.2.3.6.2 |  |
| PD4048 | The JLTV FoV shall meet sealift transport requirements coupled or un-coupled to the companion trailer. | X | X |  | X | Inspection, Analysis and Tests will be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3933 | ##### 3.2.3.6.3 | X | X |  | X | 4.2.3.6.3 |  |
| PD3934 | The JLTV FoV and companion trailers shall be transportable by Amphibious Ships, Maritime Prepositioning Force (MPF), Afloat Pre-positioning Ship (APS), Roll-On Roll-Off (RORO) ships, Joint High Speed Vessel (JHSV), Logistics Support Vessel (LSV), and all vessels of Army, Navy, and Marine Corps watercraft fleet as defined in MIL-STD-1366. | X | X |  | X | Inspection, Analysis and Tests will be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD4045 | ##### 3.2.3.6.4 | X | X |  | X | 4.2.3.6.4 |  |
| PD1759 | The JLTV FoV and companion trailers shall be transportable by Landing Craft Air-Cushioned (LCAC), Joint High Speed Vessel (JHSV), Landing Craft Utility (LCU), Logistic Support Vehicle (LSV), Landing Craft Medium (LCM), Improved Navy Lighterage System (INLS) in accordance with MIL-STD-1366. | X | X |  | X | Inspection, Analysis and Tests will be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD4047 | ##### 3.2.3.6.5 | X | X |  | X | 4.2.3.6.5 |  |
| PD4049 | Removal of the weapon and GPK shall be permitted to meet transportability requirements. No further disassembly of the JLTV FoV or companion trailers shall be required. | X | X |  | X | Inspection, Analysis and Tests will be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD3388 | ##### 3.2.3.6.6 | X | X |  | X | 4.2.3.6.6 |  |
| PD3389 | The JLTV system shall be capable of withstanding external saltwater spray while on-board an LCAC during typical LCAC operations for periods up 60 minutes(T) and 120 minutes(O) at a time, without essential function failure. | X | X |  | X | Inspection, Analysis and Tests will be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD7298 | #### 3.2.3.7 Adjustable Height Suspension. |  | X |  |  | 4.2.3.7 |  |
| PD7299 | ##### 3.2.3.7.1 |  | X |  |  | 4.2.3.7.1 |  |
| PD7300 | If an adjustable height suspension is implemented on the JLTV prime mover and/or trailer (JLTV-T) to achieve transportability requirements, then the following requirements shall be met: |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. |  |
| PD7301 | ###### 3.2.3.7.1.1 |  | X |  |  | 4.2.3.7.1.1 |  |
| PD7302 | The adjustable height suspension shall be raised or lowered within 1 minute. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7303 | ###### 3.2.3.7.1.2 |  | X |  |  | 4.2.3.7.1.2 |  |
| PD7304 | The activation mechanism for suspension height adjustment shall incorporate at least two distinct safety features that the driver or co-driver must overcome before adjustments can be made. In case of failure of one or more of these safety override features, suspension height adjustment shall be automatically disabled. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7305 | ###### 3.2.3.7.1.3 |  | X |  |  | 4.2.3.7.1.3 |  |
| PD7306 | Suspension height adjustment shall be disabled while the JLTV and/or JLTV-T are in motion. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7307 | ###### 3.2.3.7.1.4 |  | X |  |  | 4.2.3.7.1.4 |  |
| PD7308 | Indicator(s) shall be provided that visually informs the driver/co-driver of the current suspension height position, and that sends a notification if the suspension travel fails to reach the targeted height. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1766 | #### 3.2.3.8 Hazardous Material Transport. |  |  | X |  | 4.2.3.8 |  |
| PD1767 | JLTV FoV and companion trailers shall meet all CONUS, OCONUS and NATO highway requirements for the transport of class VIII (medical), class III (POL), class V (ammunition and missiles) where applicable. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD1768 | #### 3.2.3.9 Preparation Time. |  | X |  |  | 4.2.3.9 |  |
| PD1769 | The JLTV FoV and the companion trailers shall not require more than 30 minutes (T), 15 minutes (O), beginning when the first preparation step has begun (e.g. folding mirrors) using on-board tools and equipment (T), no tools (O), to prepare for embarkation or operation after debarkation on any form of transport. This requirement applies to vehicles already at transport weight. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD4050 | ##### 3.2.3.9.1 |  |  |  |  | 4.2.3.9.1 |  |
| PD4051 | The following shall not be included in the preparation time: a. Shelter mounting and dismounting. b. Vehicle or trailer rigging or rigging removal. c. GPK removal and installation. |  |  |  |  |  | 3 |
| PD1770 | ### 3.2.4 VEHICLE COMMAND, CONTROL, COMMUNICATIONS, COMPUTERS, INTELLIGENCE, SURVEILLANCE & RECONNAISSANCE / ELECTRONIC WARFARE (C4ISR/EW) SYSTEM |  |  |  |  | 4.2.4 |  |
| PD1771 | #### 3.2.4.1 Network Centric (NetCentric) Vehicle Requirements |  |  |  |  | 4.2.4.1 |  |
| PD1772 | This section specifies integration of Command, Control, Communications, Computers, Intelligence, Surveillance & Reconnaissance (C4I) and Electronic Warfare (EW) components into the vehicle to minimize the number of independent “Bolt On” components and to promote sharing of components and capabilities across multiple applications. The integrated design shall implement an open architecture. Specifications that define the vehicle C4ISR/EW architecture shall be made public. This includes primarily commercial standards as well as design specifications specific to the JLTV architecture. The intent of the open architecture is to allow for future requirements and products to be easily designed and integrated into the vehicle. One of the goals of this program is to realize reduced installation and maintenance costs. |  |  |  |  |  |  |
| PD1773 | Integrated components include the following areas: |  |  |  |  |  |  |
| PD1774 | a. Command, Control and Situational Awareness |  |  |  |  |  |  |
| PD1775 | b. Communications |  |  |  |  |  |  |
| PD1776 | c. Electronic Survivability |  |  |  |  |  |  |
| PD1777 | d. Logistics and Maintenance Automation |  |  |  |  |  |  |
| PD1779 | Implementation of this concept is via an A Kit/B Kit electronics design: A-Kit electronics refers to the components which are “built-in” into the vehicle during vehicle production, B-Kit electronics refers to the components which get installed onto the vehicle at a later date and may be optional based on vehicle mission. |  |  |  |  |  |  |
| PD1782 | #### 3.2.4.2 C4ISR/EW Data Bus Architecture Requirements |  | X |  |  | 4.2.4.2 |  |
| PD1783 | ##### 3.2.4.2.1 General |  | X |  |  | 4.2.4.2.1 |  |
| PD1784 | The C4ISR/EW Architecture shall enable data exchange between Command, Control, Communications, Computers, Intelligence, Surveillance and Reconnaissance / Electronic Warfare (C4ISR/EW) systems and other bussed systems on the Family of Joint Light Tactical Vehicles (JLTV) in accordance with ICDs provided as GFI and the requirements identified in the Information Assurance section of this document. |  | X |  |  | The test will use data collected from the data bus and C4ISR/EW system displays to verify successful Data transfer between all C4ISR/EW systems. Verification will be considered successful when the test shows that all message types can be sent and successfully received between all C4ISR/EW systems IAW the ICD's provided. | 1 |
| PD1785 | ##### 3.2.4.2.2 |  | X |  |  | 4.2.4.2.2 |  |
| PD1786 | The C4ISR/EW Architecture shall support the transfer of data from onboard C4ISR/EW systems to networks external to the JLTV. |  | X |  |  | The test will perform data export from JLTV to external operational network. | 1 |
| PD6859 | ##### 3.2.4.2.3 |  | X |  |  | 4.2.4.2.3 |  |
| PD6860 | The C4ISR/EW architecture shall perform a graceful shutdown of critical systems in the event of total loss of power. |  | X |  |  | The test will entail the following sequence of events: system functionality check (baseline), power shutdown, re-power and system functionality check. Verification will be considered successful when system functionality is verified as intact per baseline functionality upon re-power. | 2 |
| PD1820 | ##### 3.2.4.2.4 |  | X |  |  | 4.2.4.2.4 |  |
| PD1821 | The vehicle routing/switching functionality shall be configured and monitored from the Operational Display Unit (ODU). |  | X |  |  | The test will entail the following sequence of events:1) verification of system C4/EW functionality (baseline), clearing all router and switch configurations from ODU, verification that all configurations have been cleared from routers and switches (from ODU), reinitialization of all routers and switches (from ODU), and verification of system operability. Verification will be successful after each event above has been successfully executed and system functionality has been restored after the final event. | 3 |
| PD1797 | #### 3.2.4.3 JLTV Data Bus Architecture Requirements | X |  | X |  | 4.2.4.3 |  |
| PD1798 | ##### 3.2.4.3.1 | X |  | X |  | 4.2.4.3.1 |  |
| PD7241 | The JLTV Data Bus architecture shall include a C4ISR/EW Data Bus and a Vehicle Sensor Data Bus. | X |  |  |  | JLTV shall be inspected to verify compliance with the Section 3 requirement. | 2 |
| PD7242 | ###### 3.2.4.3.1.1 |  |  | X |  | 4.2.4.3.1.1 |  |
| PD7243 | The C4ISR/EW Data Bus shall implement a network backbone that provides throughput and access time performance comparable to IEEE 802.3z and IEEE 802.3ab standards. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD7244 | ###### 3.2.4.3.1.2 |  |  | X |  | 4.2.4.3.1.2 |  |
| PD7246 | The C4ISR/EW Data Bus and connected components shall support IPV4 and IPV6 transport. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD7245 | ###### 3.2.4.3.1.3 |  |  | X |  | 4.2.4.3.1.3 |  |
| PD7247 | The Vehicle Sensor Data Bus shall be compliant with appropriate industry standards (i.e. SAE J1708, J1939 and IEEE 1451) in order to enable and facilitate the exchange and update of vehicle diagnostics data. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD1832 | ##### 3.2.4.3.2 Growth. | X | X |  |  | 4.2.4.3.2 |  |
| PD1843 | ###### 3.2.4.3.2.1 | X | X |  |  | 4.2.4.3.2.1 |  |
| PD1844 | The routing/switching functionality shall provide 6 spare ports more than the maximum vehicle B-Kit electronics requirement to provide connections to future vehicle C4ISR/EW systems. | X | X |  |  | This test will entail visual inspection and test of the routing/switching hardware to verify that there are 6 functional spare ports. Verification will be successful after all ports have been identified and a successful "ping"has been issued that reaches, and is returned, by each C4/EW system on the LAN. | 3 |
| PD1845 | ###### 3.2.4.3.2.2 | X |  |  |  | 4.2.4.3.2.2 |  |
| PD1846 | All Data Bus connectors that are left unused shall be secured and protected to prevent damage and ensure future availability. The protection device shall be easily removable. | X |  |  |  | All data bus connectors will be visually inspected to confirm that the unused connectors are secured and protected. | 3 |
| PD1847 | ##### 3.2.4.3.3 Data Bus Diagnostics. | X |  | X |  | 4.2.4.3.3 |  |
| PD1848 | The JLTV Data Bus architecture fault and functional health shall be monitored by the Electronic Logistics Diagnostics and Reporting Capabilities. | X |  | X |  | Contractor will provide certification of Electronic Logistics Diagnostics and Reporting Capabilities via physical documentation and onsite demonstration. | 3 |
| PD1849 | ##### 3.2.4.3.4 Harness and Routing: | X |  |  |  | 4.2.4.3.4 |  |
| PD1854 | ###### 3.2.4.3.4.1 | X |  |  |  | 4.2.4.3.4.1 |  |
| PD1855 | All harnesses shall be protected from physical damage such as impact and abrasion and routed in a fashion that avoids contact with rough surfaces or sharp edges. | X |  |  |  | All harnesses will be visually inspected to confirm that they are routed and secured to avoid damage. | 3 |
| PD1860 | ###### 3.2.4.3.4.2 | X |  |  |  | 4.2.4.3.4.2 |  |
| PD1861 | All Data Bus harnesses shall be marked with circuit identification on each wire. Identification shall include brief but intuitive description of wiring function and intended connection devices. All Data Bus port connectors shall be labeled with intended A-Kit/B-Kit electronics component identifiers. Labels shall also identify the destination switch/router and port number/identifier. | X |  |  |  | All harnesses will be visually inspected to confirm that they are marked as stated in the requirement. | 3 |
| PD1874 | ##### 3.2.4.3.5 Timing Latency and Jitter: |  | X |  |  | 4.2.4.3.5 |  |
| PD1875 | ###### 3.2.4.3.5.1 |  | X |  |  | 4.2.4.3.5.1 |  |
| PD1876 | The C4ISR/EW Data Bus timing distribution shall support data exchange with latency no greater than that of the proper implementation and distribution of GPS timing and data IAW IS-GPS-154C and ICD-GPS-153C. |  | X |  |  | The test will entail instrumentation of the C4ISR/EW Data Bus to determine signal jitter measurements. Verification will be considered successful when the test shows that jitter measurements indicate that it is within in the specifications of IS-GPS-154C and ICD-GPS-153C. | 3 |
| PD1877 | ###### 3.2.4.3.5.2 |  | X |  |  | 4.2.4.3.5.2 |  |
| PD1878 | The C4ISR/EW Data Bus timing distribution shall support data exchange with jitter no greater than that of the proper implementation and distribution of GPS timing and data IAW IS-GPS-154C and ICD-GPS-153C. |  | X |  |  | The test will entail instrumentation of the C4ISR/EW Data Bus to determine signal fidelity, jitter and delay measurements. Verification will be considered successful when the test shows that fidelity, jitter and delay measurements indicate that they are with in specifications of IS-GPS-154C and ICD-GPS-153C. | 3 |
| PD1879 | ##### 3.2.4.3.6 Environment and Reliability: | X | X |  |  | 4.2.4.3.6 |  |
| PD6575 | ###### 3.2.4.3.6.1 | X |  |  |  | 4.2.4.3.6.1 |  |
| PD6576 | All harnesses and A-Kit/B-Kit electronics components shall be routed/installed to prevent water damage from fording, condensation from HVAC system and environmental effects. | X |  |  |  | All harnesses will+G25 be visually inspected to confirm that they are routed to avoid water damage as stated in the requirement. | 3 |
| PD1882 | ###### 3.2.4.3.6.2 Redundancy. |  | X |  |  | 4.2.4.3.6.2 |  |
| PD1883 | The C4ISR/EW Data Bus shall provide for failover modes and critical function redundancy if any segment of the C4ISR/EW Data Bus primary bus fails. |  | X |  |  | The test will entail the following sequence of events: Verification of system C4/EW functionality (baseline), arbitrary disconnection of primary data bus cable(s) in such a way as to prevent data from traversing this physical path and testing system functionality. Verification will be successful after system functionality has been verified after the interruption of the primary data bus, without operator intervention. | 2 |
| PD1888 | #### 3.2.4.4 Display and Control Subsystem | X | X |  |  | 4.2.4.4 |  |
| PD4002 | The Display and Control Subsystem shall consist of a primary Operator Display Unit (ODU) and any number of Auxillary Display Units (ADUs) that may be required to enable the crew to perform its mission. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 1 |
| PD1889 | ##### 3.2.4.4.1 Operator Display Unit (ODU) |  | X |  |  | 4.2.4.4.1 |  |
| PD1890 | The Operator Display Unit (ODU) shall be used to view and control at a minimum, but not limited to communications, sensor, and application information from the following components: |  | X |  |  | The system will be tested for full functionality via the ODU. | 2 |
| PD1891 | a. Battle Command & Control |  |  |  |  | The system will be tested for full functionality via the ODU. | 3 |
| PD1892 | b. Diagnostics |  |  |  |  | The system will be tested for full functionality via the ODU. | 3 |
| PD1893 | c. Sensor Control |  |  |  |  | The system will be tested for full functionality via the ODU. | 3 |
| PD1894 | d. Electronic Warfare (EW) Control |  |  |  |  | The system will be tested for full functionality via the ODU. | 3 |
| PD1895 | e. Radio Control |  |  |  |  | The system will be tested for full functionality via the ODU. | 3 |
| PD1896 | f. Router Control |  |  |  |  | The system will be tested for full functionality via the ODU. | 3 |
| PD1897 | g. Router Configuration |  |  |  |  | The system will be tested for full functionality via the ODU. | 3 |
| PD1904 | ##### 3.2.4.4.2 Auxiliary Display Unit (ADU) | X | X |  |  | 4.2.4.4.2 |  |
| PD1905 | Any ADUs shall be configurable to permit viewing and control of any subset of ODU functionality. |  | X |  |  | The test will employ a random set of procedures to confirm ADU and ODU cross-functionality. | 2 |
| PD4003 | ###### 3.2.4.4.2.1 | X |  |  |  | 4.2.4.4.2.1 |  |
| PD4004 | The Display and Control Subsystem shall include at a minimum a driver ADU, located in front of the driver to permit viewing and control of vehicle controls and Electronic Logistics Diagnostics component information. | X |  |  |  | The JLTV will be visually inspected to confirm functionality and compliance. | 3 |
| PD1912 | ##### 3.2.4.4.3 Common ODU and ADU requirements | X | X | X |  | 4.2.4.4.3 |  |
| PD4007 | ###### 3.2.4.4.3.1 |  |  | X |  | 4.2.4.4.3.1 |  |
| PD4008 | The ODU and ADU shall be identical hardware and be interchangeable. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD4006 | ###### 3.2.4.4.3.2 | X |  |  |  | 4.2.4.4.3.2 |  |
| PD4009 | The ODU and ADU shall include a minimum 13-inch diagonal screen. | X |  |  |  | The displays will be inspected for conformance to the section 3 requirement. | 3 |
| PD4005 | ###### 3.2.4.4.3.3 | X |  | X |  | 4.2.4.4.3.3 |  |
| PD4010 | The ODU and ADU shall include at least 1280x1024 native resolution. | X |  | X |  | Certification and Visual Inspection shall be used to verify compliance with this requirement. | 3 |
| PD1915 | ###### 3.2.4.4.3.4 | X |  | X |  | 4.2.4.4.3.4 |  |
| PD1916 | The ODU and ADU shall be flat panel technology and more specifically not cathode ray tube (CRT) technology. | X |  | X |  | Certification and Visual Inspection shall be used to verify compliance with this requirement. | 3 |
| PD1919 | ###### 3.2.4.4.3.5 | X |  |  |  | 4.2.4.4.3.5 |  |
| PD1920 | The ODU and ADU shall include the ability to receive power directly from vehicle power management/distribution system. | X |  |  |  | The displays will be inspected for conformance to the section 3 requirement. | 3 |
| PD1921 | ###### 3.2.4.4.3.6 | X |  |  |  | 4.2.4.4.3.6 |  |
| PD1922 | The ODU and ADU shall include applicable connectors to accommodate viewing of components, provided as GFI, IAW ICDs. | X |  |  |  | The displays will be inspected for conformance to the section 3 requirement. | 3 |
| PD1923 | ###### 3.2.4.4.3.7 | X |  |  |  | 4.2.4.4.3.7 |  |
| PD1924 | The ODU and ADU shall include anti-glare or other filtering components as required for viewing discernible images in direct sunlight, and permit automatic and manual brightness reduction for nighttime viewing, black out mode and viewing through Night Vision Goggles (NVGs). | X |  |  |  | The displays will be inspected for conformance to the section 3 requirement. | 3 |
| PD1925 | ###### 3.2.4.4.3.8 | X |  | X |  | 4.2.4.4.3.8 |  |
| PD1926 | The ODU and ADU shall include a contrast ratio of at least 100 to 1 in a daylight readability ambient environment with all filters in place when viewed from any position + 85 / - 85 degrees horizontal and + 85 / - 85 degrees vertical about a vector normal to the display surface | X |  | X |  | Certification and Visual Inspection shall be used to verify compliance with this requirement. | 3 |
| PD1927 | ###### 3.2.4.4.3.9 | X |  |  |  | 4.2.4.4.3.9 |  |
| PD1928 | The ODU and ADU shall include adjustable contrast and brightness controls. | X |  |  |  | The displays will be inspected for conformance to the section 3 requirement. | 3 |
| PD1929 | ###### 3.2.4.4.3.10 | X |  |  |  | 4.2.4.4.3.10 |  |
| PD1930 | The ODU and ADU shall include vertical and horizontal image adjustment controls. | X |  |  |  | The displays will be inspected for conformance to the section 3 requirement. | 3 |
| PD1933 | ###### 3.2.4.4.3.11 | X |  |  |  | 4.2.4.4.3.11 |  |
| PD1934 | The ODU and ADU shall include the power indicator, and a front panel indicator to indicate “sync operation” for both analog and digital signals | X |  |  |  | The displays will be inspected for conformance to the section 3 requirement. | 3 |
| PD1937 | ###### 3.2.4.4.3.12 |  | X |  |  | 4.2.4.4.3.12 |  |
| PD1938 | The ODU and ADU shall include the capability of reducing the display brightness to a level that is sufficient to not be detected by unaided human eyes at ranges greater than 50 meters or by Night Vision Goggles (AN/PVS-7 & AN/PVS-14) at ranges greater than 500 meters from the host vehicle under clear atmospheric conditions and starlight. |  | X |  |  | The displays will be tested for conformance to the section 3 requirement. | 3 |
| PD7230 | 3.2.4.4.3.12.1 | X |  |  |  | 4.2.4.4.3.12.1 |  |
| PD7231 | All vehicle displays shall dim to the above levels when black out mode is enabled. | X |  |  |  | The displays will be inspected for conformance to the section 3 requirement. | 3 |
| PD1939 | ###### 3.2.4.4.3.13 |  | X |  |  | 4.2.4.4.3.13 |  |
| PD1940 | The ODU and ADU display luminance shall be dimmable to 0.05 foot-lamberts or less. Light security filter is permissible. |  | X |  |  | The displays will be tested for conformance to the section 3 requirement. | 3 |
| PD4012 | ###### 3.2.4.4.3.14 |  | X |  |  | 4.2.4.4.3.14 |  |
| PD4013 | The ODU and ADU shall prevent burn-in of latent images. |  | X |  |  | The displays will be tested for conformance to the section 3 requirement. | 3 |
| PD6861 | ###### 3.2.4.4.3.15 |  | X |  |  | 4.2.4.4.3.15 |  |
| PD6862 | The ODU and ADU shall permit viewing under blue and red LED lighting. |  | X |  |  | The displays will be tested for conformance to the section 3 requirement. | 3 |
| PD1941 | ##### 3.2.4.4.4 The ODU and ADU Interface | X | X | X |  | 4.2.4.4.4 |  |
| PD1942 | ###### 3.2.4.4.4.1 |  |  | X |  | 4.2.4.4.4.1 |  |
| PD1943 | The ODU and ADU Interface electronics shall have the ability to display analog and digital data and video signals fed from C4ISR/EW DATA BUS and auto-sync within a pixel frequency range of 10 to 204 MHz (the analog interface shall accept a separate sync signal and composite sync) |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD1944 | ###### 3.2.4.4.4.2 |  | X |  |  | 4.2.4.4.4.2 |  |
| PD1945 | The ODU and ADU Interface electronics shall have the ability to display static and live National Television Systems Committee (NTSC) interlaced motion video information with no perceivable flicker with refresh rate of 30 Hz. |  | X |  |  | The displays will be tested for conformance to the section 3 requirement. | 3 |
| PD1946 | ###### 3.2.4.4.4.3 |  | X |  |  | 4.2.4.4.4.3 |  |
| PD1947 | The ODU and ADU Interface electronics shall have the ability to display dynamic analog and digital data signals with no perceivable flicker with refresh rate between 60 to 76 Hz |  | X |  |  | The displays will be tested for conformance to the section 3 requirement. | 3 |
| PD1948 | ###### 3.2.4.4.4.4 | X |  |  |  | 4.2.4.4.4.4 |  |
| PD1949 | The ODU and ADU Interface electronics shall have the ability to display motion video information. | X |  |  |  | The displays will be inspected for conformance to the section 3 requirement. | 3 |
| PD1950 | ###### 3.2.4.4.4.5 |  | X |  |  | 4.2.4.4.4.5 |  |
| PD1951 | The ODU and ADU Interface electronics shall have the ability to display interlace or progressive scanning inputs through a scaling processor with frame memory integrated in display |  | X |  |  | The displays will be tested for conformance to the section 3 requirement. | 3 |
| PD1952 | ###### 3.2.4.4.4.6 |  |  | X |  | 4.2.4.4.4.6 |  |
| PD1953 | The ODU and ADU Interface electronics shall have the ability to display multi-format video input formats, possessing disparate resolution formats, IAW ICDs provided as GFI, at a minimum, but not limited to: VGA, SVGA, XGA, SXGA, SXGA+, UXGA, WUXGA |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD1954 | ###### 3.2.4.4.4.7 |  |  | X |  | 4.2.4.4.4.7 |  |
| PD1955 | The ODU and ADU Interface electronics shall have the ability to display formats greater or less than the screen's native grid size, scaled to match the screen's parameters |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD1956 | ###### 3.2.4.4.4.8 |  |  | X |  | 4.2.4.4.4.8 |  |
| PD1957 | The ODU and ADU Interface electronics shall have the ability to display, at a minimum, 32-bit color depth (24-bit color and 8 bits to accommodate conventional computer data) at the native resolution of the screen |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD1958 | ###### 3.2.4.4.4.9 |  |  | X |  | 4.2.4.4.4.9 |  |
| PD1959 | The ODU and ADU Interface electronics shall have the ability to display varying aspect ratios such as, but not limited to: 4:3, 5:4, 16:9, and automatically switch between modes |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD1960 | ###### 3.2.4.4.4.10 |  |  | X |  | 4.2.4.4.4.10 |  |
| PD1961 | The ODU and ADU Interface electronics shall have the ability to display high information content through the VCS 3D graphics acceleration or additional graphics processing. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6743 | ##### 3.2.4.4.5 Controls. |  | X |  |  | 4.2.4.4.5 |  |
| PD1913 | ###### 3.2.4.4.5.1 |  | X |  |  | 4.2.4.4.5.1 |  |
| PD1914 | The ODU and ADU shall include a touch screen and buttons around the perimeter of the display as Human Machine Interfaces (HMIs) for manipulation of displayed object. The HMIs shall be operable in MOPP IV & Cold Weather gear. |  | X |  |  | The displays will be tested for conformance to the section 3 requirement. | 3 |
| PD1981 | #### 3.2.4.5 Information Assurance |  | X | X | X | 4.2.4.5 |  |
| PD1998 | ##### 3.2.4.5.1 Classification |  |  | X |  | 4.2.4.5.1 |  |
| PD1999 | The C4I/EW architecture shall process from Unclassified to Secret with caveats to include Secret NATO. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD1982 | ##### 3.2.4.5.2 Information Assurance Capabilities |  |  | X |  | 4.2.4.5.2 |  |
| PD1983 | ###### 3.2.4.5.2.1 |  |  | X |  | 4.2.4.5.2.1 |  |
| PD1984 | The JLTV shall be certified and accredited in accordance with DoDD 8500.1 and DODI 8500.bb (DIACAP) implementing the corresponding IA controls as described in DoD information assurance (IA) certification and accreditation (C&A). |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD1987 | ###### 3.2.4.5.2.2 |  |  | X |  | 4.2.4.5.2.2 |  |
| PD1988 | There shall be no connection of any device to both the SECRET and the UNCLASSIFIED bus that has not been approved as a Cross Domain Peripheral Sharing device or Cross Domain Solution that has completed a favorable Certification Test and Evaluation conducted by the National Security Agency or trusted agent designated by the National Security Agency to perform such testing on their behalf, to include CT&E SR 9 Penetration Testing. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD1993 | ###### 3.2.4.5.2.3 |  |  | X |  | 4.2.4.5.2.3 |  |
| PD1994 | If a Firewall, IDS or other IA product is developed on this effort, then it shall meet NIST requirements and the IA product will be assessed under the National Information Assurance Partnership (NIAP) Common Criteria Evaluation and Validation Scheme (CCEVS) for approval. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD1997 | ##### 3.2.4.5.3 Information Description |  | X |  |  | 4.2.4.5.3 |  |
| PD2000 | ###### 3.2.4.5.3.1 Least Privileges |  | X |  |  | 4.2.4.5.3.1 |  |
| PD4014 | 3.2.4.5.3.1.1 |  | X |  |  | 4.2.4.5.3.1.1 |  |
| PD2002 | The C4I/EW architecture shall be reconfigurable in order to ensure that Users are limited to minimal capabilities required to perform their duties. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2001 | 3.2.4.5.3.1.2 |  | X |  |  | 4.2.4.5.3.1.2 |  |
| PD2003 | The C4I/EW architecture shall ensure that Applications are limited to capabilities required to perform its functions. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2004 | 3.2.4.5.3.1.3 Data at Rest |  | X |  |  | 4.2.4.5.3.1.3 |  |
| PD2005 | All data shall be encrypted while at rest within the boundary of the C4I/EW System IAW DoD approved methods. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2006 | 3.2.4.5.3.1.4 Tamper Protection |  | X |  |  | 4.2.4.5.3.1.4 |  |
| PD2007 | The C4I/EW architecture shall include tamper protection measures as required by the applicable Federal Information Processing Standards (FIPS) 140-2 or NSA approved methods as defined in the UIC. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2010 | ##### 3.2.4.5.4 Management |  | X |  |  | 4.2.4.5.4 |  |
| PD2011 | ###### 3.2.4.5.4.1 System Manager |  | X |  |  | 4.2.4.5.4.1 |  |
| PD2014 | 3.2.4.5.4.1.1 Operating System |  | X |  |  | 4.2.4.5.4.1.1 |  |
| PD2015 | The secure configuration of the Operating Systems shall be consistent with ICD's for hosted applications and any integrating hardware and software devices. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2016 | 3.2.4.5.4.1.2 |  | X |  |  | 4.2.4.5.4.1.2 |  |
| PD2017 | The C4I/EW architecture shall allow for Operating System Patches and Updates to be applied. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2018 | ##### 3.2.4.5.5 System IA Products |  | X | X | X | 4.2.4.5.5 |  |
| PD2019 | ###### 3.2.4.5.5.1 Firewalls |  | X |  |  | 4.2.4.5.5.1 |  |
| PD2020 | The A-Kit electronics shall include firewalls to limit the ports and protocols used to communicate IAW the principles of least privilege operation as required. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 1 |
| PD2021 | ###### 3.2.4.5.5.2 Anti Virus & Malware |  | X |  |  | 4.2.4.5.5.2 |  |
| PD2022 | The computing system shall include anti-virus & Malware products for all applicable OS. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2023 | 3.2.4.5.5.2.1 |  | X |  |  | 4.2.4.5.5.2.1 |  |
| PD2024 | The computing system shall allow for the anti-virus & malware definitions to be updated. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2025 | ###### 3.2.4.5.5.3 Intrusion Detection |  | X |  |  | 4.2.4.5.5.3 |  |
| PD2026 | The A-Kit electronics shall include an Intrusion Detection System (IDS) capability to detect networked based attacks. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 1 |
| PD2027 | 3.2.4.5.5.3.1 |  | X |  |  | 4.2.4.5.5.3.1 |  |
| PD2028 | The C4ISR/EW architecture shall allow for the Intrusion Detection System to be updated. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2029 | ###### 3.2.4.5.5.4 Intrusion Prevention and Reaction |  | X |  |  | 4.2.4.5.5.4 |  |
| PD2030 | The C4ISR/EW architecture shall include capabilities to respond to and countermeasures to defeat network attacks. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2031 | 3.2.4.5.5.4.1 |  | X |  |  | 4.2.4.5.5.4.1 |  |
| PD2032 | The C4ISR/EW architecture shall allow for the Intrusion Prevention and Reaction Components to be updated. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2037 | ###### 3.2.4.5.5.5 Access Control |  | X |  |  | 4.2.4.5.5.5 |  |
| PD2038 | The computing architectures shall require Administrator accounts to authenticated and authorized. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2042 | ###### 3.2.4.5.5.6 Management of System Security policies |  | X | X |  | 4.2.4.5.5.6 |  |
| PD2043 | The C4I/EW architecture shall include a single interface to manage the system security policies implemented for the aforementioned IA products. In addition, the security management shall have these services: |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2044 | a. Authentication (user/source) |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2045 | b. Integrity |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2046 | c. Confidentiality |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2047 | d. Protocols such as SNMP V3/SSL/Secure Shell |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6577 | e. Availability |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD6578 | f. Non-repudiation |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2048 | g. Others as required |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2052 | ###### 3.2.4.5.5.7 Local & Remote Controller |  | X |  |  | 4.2.4.5.5.7 |  |
| PD2053 | 3.2.4.5.5.7.1 |  | X |  |  | 4.2.4.5.5.7.1 |  |
| PD2054 | The C4ISR/EW architecture shall support the local and remote control of system functions in a secure manner IAW IA requirements. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2060 | ###### 3.2.4.5.5.8 End Crypto Unit Management (ECU) |  |  | X | X | 4.2.4.5.5.8 |  |
| PD2061 | 3.2.4.5.5.8.1 Cryptographic Functions |  |  | X | X | 4.2.4.5.5.8.1 |  |
| PD2062 | 3.2.4.5.5.8.1.1 NSA approved cryptography |  |  | X | X | 4.2.4.5.5.8.1.1 |  |
| PD2063 | The C4ISR/EW architecture shall meet all security requirements necessary to obtain an NSA certification, to meet the operational needs of the program. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 1 |
| PD2064 | 3.2.4.5.5.8.1.1.1 |  |  |  | X | 4.2.4.5.5.8.1.1.1 |  |
| PD2065 | The C4ISR/EW architecture shall meet all the requirements necessary to be handled as COMSEC Controlled Item (CCI) when un-keyed. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD2066 | 3.2.4.5.5.8.1.1.2 |  |  | X |  | 4.2.4.5.5.8.1.1.2 |  |
| PD2067 | If IP traffic will be present on the C4ISR/EW architecture, then the architecture shall adhere to the HAIPE Specification for all Network layer Cryptography. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2068 | 3.2.4.5.5.8.1.2 NIST based cryptography |  |  | X |  | 4.2.4.5.5.8.1.2 |  |
| PD2069 | The C4ISR/EW architecture shall meet all security requirements as defined in FIPS 140-2/3 to meet the operational needs of the program. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2071 | 3.2.4.5.5.8.1.3 Zeroization |  |  |  |  | 4.2.4.5.5.8.1.3 |  |
| PD2072 | For the purposes of this section, the term “keys” includes system keys, certificates, and FIREFLY vectors, or any other cryptographic vectors, as well as user traffic keys. |  |  |  |  |  |  |
| PD2073 | 3.2.4.5.5.8.1.3.1 |  | X |  |  | 4.2.4.5.5.8.1.3.1 |  |
| PD2074 | The act of Zeroization shall zeroize all Red keys and erase all classified data (at a minimum, includes hopsets, frequencies, mission parameters, IP Addresses, User traffic) stored in the System. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2075 | 3.2.4.5.5.8.1.3.2 |  | X |  |  | 4.2.4.5.5.8.1.3.2 |  |
| PD2076 | A panic zeroize capability shall be provided on the System and it shall require dual actions to avoid inadvertent operation. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2079 | 3.2.4.5.5.8.1.3.3 |  | X |  |  | 4.2.4.5.5.8.1.3.3 |  |
| PD2080 | The operator shall have the capability, through the HMI, to selectively zeroize any Red key. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2081 | 3.2.4.5.5.8.1.3.4 |  | X |  |  | 4.2.4.5.5.8.1.3.4 |  |
| PD2082 | The operator shall have the capability to selectively erase any Black keys. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2083 | 3.2.4.5.5.8.1.4 Key Management |  |  |  |  | 4.2.4.5.5.8.1.4 |  |
| PD2084 | 3.2.4.5.5.8.1.4.1 |  |  | X |  | 4.2.4.5.5.8.1.4.1 |  |
| PD2085 | The C4ISR/EW system architecture shall be compatible with the NSA Key Management System/Electronic Key Management System (EKMS) key distribution and net management systems IAW the Defense Information Infrastructure (DII) Key Management Infrastructure. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2086 | 3.2.4.5.5.8.1.4.2 |  | X |  |  | 4.2.4.5.5.8.1.4.2 |  |
| PD2087 | Red and Black keys shall be loaded via a single common fill connector. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2090 | 3.2.4.5.5.8.1.4.3 |  | X |  |  | 4.2.4.5.5.8.1.4.3 |  |
| PD2091 | The C4ISR/EW system architecture shall interface with the Simple Key Loader (SKL) or AN/CYZ-10A Data Transfer Device (DTD) and the Common Tier 3 Software Application for receiving key and terminal parameters. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2092 | 3.2.4.5.5.8.1.4.4 |  |  | X |  | 4.2.4.5.5.8.1.4.4 |  |
| PD2093 | The C4ISR/EW system architecture shall support migration to the NSA led Key Management Infrastructure (KMI). |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2094 | 3.2.4.5.5.8.1.4.5 |  |  | X |  | 4.2.4.5.5.8.1.4.5 |  |
| PD2095 | The C4ISR/EW system architecture shall comply with DOD PKI certificates for all User and Device authentication requirements. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2100 | 3.2.4.5.5.8.1.4.6 PKI |  |  | X |  | 4.2.4.5.5.8.1.4.6 |  |
| PD2101 | The C4ISR/EW system architecture shall be capable of using PKI Certificates (on a token) for User authentication. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2102 | 3.2.4.5.5.8.1.4.7 |  | X |  |  | 4.2.4.5.5.8.1.4.7 |  |
| PD2103 | The C4ISR/EW system architecture shall use PKI Certificates for Device authentication |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2104 | 3.2.4.5.5.8.1.4.8 |  | X |  |  | 4.2.4.5.5.8.1.4.8 |  |
| PD2105 | The C4ISR/EW system architecture shall support Certification Revocation |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2106 | 3.2.4.5.5.8.1.4.9 Key Loading |  |  | X |  | 4.2.4.5.5.8.1.4.9 |  |
| PD2107 | The C4ISR/EW system architecture shall support key loading specifications as defined in EKMS which include: |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2108 | a. DS-101 fill protocol |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2109 | b. DS-102 fill protocol |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2110 | c. DS-100 key tagging |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2111 | d. Interface definitions |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2112 | e. Mission Data. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2113 | 3.2.4.5.5.8.1.4.10 Key control functions |  | X |  |  | 4.2.4.5.5.8.1.4.10 |  |
| PD2114 | The C4ISR/EW system architecture shall support key maintenance functions as defined in EKMS which include: |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2115 | a. Key Update |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2116 | b. Key Rollover |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2117 | c. Over-The Air Functions (OTAR, OTAT, OTAD, OTAZ) |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2118 | d. Compromise |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2119 | e. Recovery |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2120 | f. Zeroize |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2121 | ##### 3.2.4.5.6 Information Sharing |  |  | X |  | 4.2.4.5.6 |  |
| PD2126 | ###### 3.2.4.5.6.1 |  |  | X |  | 4.2.4.5.6.1 |  |
| PD2127 | The system shall customize the Rule Sets to meet the System operational needs and DSAWG/NSA requirements. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2128 | #### 3.2.4.6 C4ISR/EW Survivability |  |  |  |  | 4.2.4.6 |  |
| PD2130 | ##### 3.2.4.6.1 General |  |  |  |  | 4.2.4.6.1 |  |
| PD2131 | To accommodate the expected wide range of missions and operational scenarios, the JLTV Family of Vehicle (FoV) designs must incorporate provisions that allow survivability requirements to be met at different levels. The network centric survivability requirements describe the electronic and network issues for vehicle self protection technologies. The JLTV mission requirements will require a tailorable suite of defensive measures to provide defensive measures, situational awareness, and crew and passenger safety. |  |  |  |  |  |  |
| PD2133 | ##### 3.2.4.6.2 Survivability Architecture |  |  | X |  | 4.2.4.6.2 |  |
| PD2134 | ###### 3.2.4.6.2.1 |  |  | X |  | 4.2.4.6.2.1 |  |
| PD2135 | The JLTV survivability architecture shall utilize the C4ISR/EW DATA BUS to support system level needs. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2136 | ###### 3.2.4.6.2.2 |  |  | X |  | 4.2.4.6.2.2 |  |
| PD2137 | Survivability components shall be capable of information sharing with the C4ISR/EW systems to identify potential threats and to define the location of friendly forces in the local area. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2140 | ###### 3.2.4.6.2.3 |  |  | X |  | 4.2.4.6.2.3 |  |
| PD2141 | The crew shall be alerted to potential threats identified by the survivability suite along with a continuous picture of the survivability suite status. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2160 | ##### 3.2.4.6.3 Situational Awareness |  |  | X |  | 4.2.4.6.3 |  |
| PD2161 | Situational Awareness information shall be shared between the C4/EW systems and defensive measures sensor information to support reporting local situational awareness to the vehicle Display and Control subsystem. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2166 | #### 3.2.4.7 Expansion/Growth |  |  |  |  | 4.2.4.7 |  |
| PD2167 | The NetCentric Vehicle C4/EW evolutionary capability can be expressed in terms of: |  |  |  |  |  |  |
| PD2168 | 1) Expansion: denotes accommodation of future capability physical space and mounting location requirements. |  |  |  |  |  |  |
| PD2169 | 2) Growth: denotes accommodation of future C4/EW functional capability requirements |  |  |  |  |  |  |
| PD2170 | ##### 3.2.4.7.1 Expansion |  |  |  |  | 4.2.4.7.1 |  |
| PD2171 | ###### 3.2.4.7.1.1 | X |  |  |  | 4.2.4.7.1.1 |  |
| PD2172 | The JLTV design shall anticipate the need for additional space and C4ISR/EW Data Bus routing to accommodate, to the greatest extent, the introduction of new, C4ISR/EW A-Kit electronics and B-Kit electronics systems. | X |  |  |  | This event will entail visual inspection of both architectural diagrams and physical inspection of architecture applications on prototype that account for expansion to accommodate additional C4ISR/EW systems to meet future needs and evolving threats. | 3 |
| PD2175 | ###### 3.2.4.7.1.2 | X |  |  |  | 4.2.4.7.1.2 |  |
| PD2176 | The JLTV design shall anticipate the need for additional/upgraded antennas. | X |  |  |  | This event will entail visual inspection of both architectural diagrams and physical inspection of architecture applications on prototype that account for expansion to accommodate additional antennas that support upgraded C4ISR/EW systems to meet future needs and evolving capabilities. | 3 |
| PD2179 | ##### 3.2.4.7.2 Growth | X |  |  |  | 4.2.4.7.2 |  |
| PD2180 | ###### 3.2.4.7.2.1 | X |  |  |  | 4.2.4.7.2.1 |  |
| PD2181 | The JLTV design shall anticipate the need for additional functionality and C4ISR/EW Data Bus routing to accommodate, to the greatest extent, the introduction of new, C4/EW A-Kit electronics and B-Kit electronics systems. | X |  |  |  | This event will entail visual inspection of both architectural diagrams and physical inspection of architecture applications on prototype that account for expansion to accommodate additional C4ISR/EW systems to meet future needs and evolving threats. | 3 |
| PD2189 | #### 3.2.4.8 Communication Capability | X | X |  |  | 4.2.4.8 |  |
| PD2190 | ##### 3.2.4.8.1 Military Radio Capability | X | X |  |  | 4.2.4.8.1 |  |
| PD2191 | ###### 3.2.4.8.1.1 SINCGARS Integration | X | X |  |  | 4.2.4.8.1.1 |  |
| PD2193 | 3.2.4.8.1.1.1 | X | X |  |  | 4.2.4.8.1.1.1 |  |
| PD2194 | The SINCGARS Integration shall receive all power from the vehicle power management/distribution system. | X |  |  |  | The SINCGARS Integration will be inspected for conformance to the section 3 requirement. | 2 |
| PD2195 | 3.2.4.8.1.1.2 |  | X |  |  | 4.2.4.8.1.1.2 |  |
| PD2196 | The SINCGARS Integration shall be remote controllable with regards to viewing all radio parameters, via the Display and Control Subsystem, similar to front panel operations of the equipment. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2197 | 3.2.4.8.1.1.3 |  | X |  |  | 4.2.4.8.1.1.3 |  |
| PD2198 | The SINCGARS Integration shall be remote controllable with regards to changing all radio parameters, via the Display and Control Subsystem, similar to front panel operations of the equipment. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2199 | 3.2.4.8.1.1.4 |  | X |  |  | 4.2.4.8.1.1.4 |  |
| PD2200 | The SINCGARS Integration shall utilize the Vehicle Intercom system headsets for voice communications. Secondary handsets for the SINCGARS Integration shall be accessible for voice communications. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2201 | 3.2.4.8.1.1.5 |  | X |  |  | 4.2.4.8.1.1.5 |  |
| PD2202 | All error messages for the SINCGARS Integration shall be visible through Display and Control Subsystem. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2207 | ###### 3.2.4.8.1.2 EPLRS Integration | X | X |  |  | 4.2.4.8.1.2 |  |
| PD2210 | 3.2.4.8.1.2.1 | X | X |  |  | 4.2.4.8.1.2.1 |  |
| PD2211 | The EPLRS Integration shall receive all power off the vehicle power management/distribution system. | X |  |  |  | The EPLRS Integration will be inspected for conformance to the section 3 requirement. | 2 |
| PD2212 | 3.2.4.8.1.2.2 |  | X |  |  | 4.2.4.8.1.2.2 |  |
| PD2213 | The EPLRS Integration shall transmit all data through the C4I/EW Data Bus. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2214 | 3.2.4.8.1.2.3 |  | X |  |  | 4.2.4.8.1.2.3 |  |
| PD2215 | The EPLRS Integration shall be remote controllable with regards to displaying all viewable radio parameters, via the Display and Control Subsystem, similar to front panel operations of the equipment. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2216 | 3.2.4.8.1.2.4 |  | X |  |  | 4.2.4.8.1.2.4 |  |
| PD2217 | The EPLRS Integration shall be remote controllable with regards to changing all settable radio parameters, via the Display and Control Subsystem, similar to front panel operations of the equipment. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2218 | 3.2.4.8.1.2.5 |  | X |  |  | 4.2.4.8.1.2.5 |  |
| PD2219 | All alarm indicators and error messages for the EPLRS Integration shall be visible through Display and Control subsystem. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2226 | ###### 3.2.4.8.1.3 L-Band SATCOM |  |  |  |  | 4.2.4.8.1.3 |  |
| PD2227 | See Section entitled JCR Networking Capabilities. |  |  |  |  | See Section entitled JCR Networking Capabilities. |  |
| PD2228 | ##### 3.2.4.8.2 Vehicle Intercom System. | X | X | X |  | 4.2.4.8.2 |  |
| PD2229 | ###### 3.2.4.8.2.1 | X |  |  |  | 4.2.4.8.2.1 |  |
| PD2230 | The vehicle intercom system shall provide a modular and expandable access to analog and/or digital voice communications for vehicle operators and passengers. Wired (T) Wired and Wireless (O). | X |  |  |  | The vehicle intercom system will be inspected for conformance to the section 3 requirement. | 2 |
| PD2235 | ###### 3.2.4.8.2.2 |  | X |  |  | 4.2.4.8.2.2 |  |
| PD2236 | Screens and menus shall display the available and active functions of the intercom system. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2239 | ###### 3.2.4.8.2.3 | X |  |  |  | 4.2.4.8.2.3 |  |
| PD2240 | The vehicle intercom system shall receive all power off the vehicle power management/distribution system. | X |  |  |  | The vehicle intercom system will be inspected for conformance to the section 3 requirement. | 3 |
| PD2243 | ###### 3.2.4.8.2.4 |  | X |  |  | 4.2.4.8.2.4 |  |
| PD2244 | The vehicle intercom system shall provide an interface for use with standard tactical headsets/handsets including H-250 type handsets, or hand microphones and shall be accessible from each crew station. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2245 | ###### 3.2.4.8.2.5 |  | X |  |  | 4.2.4.8.2.5 |  |
| PD2246 | Noise canceling headsets shall also be provided. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2247 | ###### 3.2.4.8.2.6 |  | X |  |  | 4.2.4.8.2.6 |  |
| PD2248 | The electrical interface shall provide for radio and intercom push-to-talk. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2249 | ###### 3.2.4.8.2.7 |  | X |  |  | 4.2.4.8.2.7 |  |
| PD2250 | Side tone suppression and echo cancellation shall be included. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2251 | ###### 3.2.4.8.2.8 |  | X |  |  | 4.2.4.8.2.8 |  |
| PD2252 | A connection for at least one adjustable volume level, external speaker with on/off capability shall be provided. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2253 | ###### 3.2.4.8.2.9 |  | X |  |  | 4.2.4.8.2.9 |  |
| PD2254 | The vehicle intercom system shall be configurable by the user to connect operators together as an intercom in any combination and to connect any or all operator(s) to any voice capable radio or transmission system that is connected to the C4ISR/EW Data Bus. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2255 | ###### 3.2.4.8.2.10 |  |  | X |  | 4.2.4.8.2.10 |  |
| PD2256 | The vehicle intercom system shall be capable of generating Voice over Internet Protocol (VoIP) calls using G.711, G.729A and g.723.1 CODECs at a minimum. VoIP shall not be the primary means for voice communication. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD7293 | ###### 3.2.4.8.2.11 |  | X |  |  | 4.2.4.8.2.11 |  |
| PD7294 | The vehicle intercom system shall function independently of the VCS. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2288 | #### 3.2.4.9 Command and Control Capability | X |  | X | X | 4.2.4.9 |  |
| PD2289 | ##### 3.2.4.9.1 Vehicle Computing System | X |  | X | X | 4.2.4.9.1 |  |
| PD2290 | The JLTV vehicle architecture requires computing and networking interfaces to support the applications that provide the following capabilities and services: |  |  |  |  |  | 2 |
| PD2291 | a. Battle Command & Control |  |  |  |  |  | 2 |
| PD2292 | b. Diagnostics |  |  |  |  |  | 2 |
| PD2293 | c. Sensor Control |  |  |  |  |  | 2 |
| PD6579 | d. Electronic Warfare (EW) Control |  |  |  |  |  | 2 |
| PD6580 | e. Radio Control |  |  |  |  |  | 2 |
| PD6581 | f. Router Control |  |  |  |  |  | 2 |
| PD6582 | g. Router Configuration |  |  |  |  |  | 2 |
| PD2294 | ###### 3.2.4.9.1.1 | X |  |  | X | 4.2.4.9.1.1 |  |
| PD2295 | The Vehicle Computing System shall be an open system. | X |  |  | X | Inspection and analysis of architecture diagrams for conformance to common standards (ANSI, IEEE, ISO, MIL STD, SAE) will be employed to determine that there are no proprietary software or physical interfaces. | 3 |
| PD2296 | ###### 3.2.4.9.1.2 | X |  |  | X | 4.2.4.9.1.2 |  |
| PD2297 | The VCS shall be designed to be scalable, integrated, and modular. The VCS shall be designed to take advantage of constantly evolving computing and networking technologies and shall be able to upgrade or replace hardware resources without impacting software applications, including Operating Systems. | X |  |  | X | The JLTV shall be inspected and analyzed for conformance to the section 3 requirement. | 2 |
| PD2302 | ###### 3.2.4.9.1.3 | X |  | X |  | 4.2.4.9.1.3 |  |
| PD2303 | The VCS shall be modular at the component (board / subsystem) level. | X |  | X |  | Certification and inspection shall be used to verify conformance to the section 3 requirement. | 3 |
| PD2304 | ###### 3.2.4.9.1.4 |  |  |  | X | 4.2.4.9.1.4 |  |
| PD2305 | The architecture must be highly scaleable to address significant variations in individual vehicular platform processing requirements. It is anticipated that the VCS will have to process voice, video and data within the vehicle. The architecture should be resilient to failure and prioritize resources to support critical system capabilities as required. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD2312 | ###### 3.2.4.9.1.5 VCS Computing Requirements |  | X | X |  | 4.2.4.9.1.5 |  |
| PD2313 | 3.2.4.9.1.5.1 |  |  | X |  | 4.2.4.9.1.5.1 |  |
| PD2314 | The VCS shall include the following computing requirements at a minimum to support C4ISR/EW JLTV software applications: |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2315 | - CPU with at least 3 GHz clock speed; dual core |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2316 | - 2 GB RAM |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2317 | - 80 GB Hard Drive |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2318 | - 3D graphics |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2319 | 3.2.4.9.1.5.2 |  |  | X |  | 4.2.4.9.1.5.2 |  |
| PD2320 | The design may require multiple computing resources to support all JLTV SW apps. |  |  | X |  | Certification shall be used to determine this. Verification shall be successful when it is determined that the VCS meets or exceeds this parameter. | 3 |
| PD2321 | 3.2.4.9.1.5.3 |  |  | X |  | 4.2.4.9.1.5.3 |  |
| PD2322 | The VCS shall include the following computing I/O and interface requirements: |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2323 | - Appropriate industry standards for Diagnostics (i.e. SAE J1939, J1708/1587, and J1850) |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2324 | - 10/100/1000 Mbit Ethernet |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2325 | - USB 1.0 and 2.0 |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2326 | - 2 RS-232 ports |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2327 | - 3 RS-422 ports |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2328 | - Microphone/speaker audio I/O |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6583 | 3.2.4.9.1.5.4 |  | X |  |  | 4.2.4.9.1.5.4 |  |
| PD6584 | The VCS shall have a maximum startup time of 30 seconds. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2329 | 3.2.4.9.1.5.5 |  |  | X |  | 4.2.4.9.1.5.5 |  |
| PD2330 | The system shall provide short-range wireless connectivity. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD4307 | 3.2.4.9.1.5.5.1 |  |  | X |  | 4.2.4.9.1.5.5.1 |  |
| PD4308 | The system shall enable electronic logistic and maintenance assets to interface to vehicle short range wireless transceivers for transmittal of logistic data to battlefield network nodes and transponders that operate with the MIMOSA implementation of ISO 13374-1. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 1 |
| PD2337 | ###### 3.2.4.9.1.6 VCS Operating System Requirements |  |  | X |  | 4.2.4.9.1.6 |  |
| PD2338 | The VCS shall implement the appropriate versions of the following operating systems: |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2339 | a. Red Hat Linux with security features (JCR/MTS or specified by the application ICD) |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2340 | b. Microsoft Windows (Logistic/Maintenance Applications or specified by the application ICD) |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2341 | ##### 3.2.4.9.2 JCR Capability |  | X |  |  | 4.2.4.9.2 |  |
| PD2342 | ###### 3.2.4.9.2.1 JCR FBCB2 Capability |  | X |  | X | 4.2.4.9.2.1 |  |
| PD2343 | 3.2.4.9.2.1.1 |  | X |  |  | 4.2.4.9.2.1.1 |  |
| PD2344 | The system shall provide a totally integrated system environment for the Government Furnished Equipment (GFE) FBCB2 JCR software which includes the MTS-ES capability. This integration includes but is not limited to: FBCB2 JCR software, computer system environment, radios, L-band satellite, tactical internet networking, MTS-ES linkages, RFID readers, fuel sensors, vehicle health sensors, graphical user interfaces, voice, LAN, keyboards, pointing devices, function keys, and all ancillary devices (i.e. hardware and software) to make FBCB2 JCR operational. |  | X |  |  | The JCR FBCB2 system shall be tested during a larger JLTV Integration and Test to validate total system integration. The test shall be considered successful after JCR connects to the terrestrial via radios and the celestial networks tactical networks via L-band satellite transceivers, C2 and situational awareness messages are passed between JLTV's, MTS-ES is functioning with RFID readers, LAN connectivity passing operational data, and the JCR system made fully operational. | 1 |
| PD2345 | 3.2.4.9.2.1.2 |  | X |  |  | 4.2.4.9.2.1.2 |  |
| PD2346 | The system shall utilize the C4ISR/EW Data Bus to interface with the provided JCR software interface. |  | X |  |  | The JCR system shall be tested during developmental and formal Integration testing to validate the software is utilizing the C4ISR/EW data bus properly IAW information technology policy for passing classified and unclassified data over the data buss. The test shall be considered successful when data traffic flows IAW the IT policy. | 2 |
| PD2347 | 3.2.4.9.2.1.3 |  | X |  |  | 4.2.4.9.2.1.3 |  |
| PD2348 | The ODU, and any appropriate ADUs, connected to the C4ISR/EW Data Bus shall control JCR start-up and shut down. |  | X |  |  | The JCR system shall be tested during developmental and formal Integration testing to validate the software is utilizing the C4ISR/ES data bus properly . The test shall be considered successful when the JLTV hardware/software successfully startup into its fully operational state, pass information such as fuel over the data bus, and successfully shuts down while interfacing to the appraise ADUs. | 2 |
| PD2349 | 3.2.4.9.2.1.4 |  | X |  |  | 4.2.4.9.2.1.4 |  |
| PD2350 | The system shall provide the interface to obtain the necessary joint, digital, Battle Command information that provides integrated, on-the-move, timely, relevant C2/SA information to the tactical combat, combat support and combat service support commanders, leaders and key C2 nodes, by sending/receiving messages and graphical data. |  | X |  |  | The JCR FBCB2 system shall be tested during a larger JLTV Integration and Test to validate total sub-system integration. The test shall be considered successful after JCR connects to the terrestrial via radios and the celestial networks tactical networks via L-band satellite transceivers, C2 and situational awareness messages are passed between JLTV's, MTS-ES is functioning with RFID readers, LAN connectivity passing operational data, and the JCR system made fully operational. | 2 |
| PD2351 | 3.2.4.9.2.1.5 |  |  |  | X | 4.2.4.9.2.1.5 |  |
| PD2352 | The system shall operate and retain the command and control at the vehicle level across the Services enabling Joint situational understanding and battle space awareness. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 1 |
| PD2357 | ###### 3.2.4.9.2.2 JCR Movement Tracking Capability |  | X |  |  | 4.2.4.9.2.2 |  |
| PD2358 | 3.2.4.9.2.2.1 |  | X |  |  | 4.2.4.9.2.2.1 |  |
| PD2359 | The system shall provide a totally integrated environment for the Movement Tracking System - Enhanced Software (MTS-ES) which incorporates the MTS-ES logistics and MTS control Station functionality as an application within the Joint Capability Release (JCR) Version. |  | X |  |  | The MTS-ES system shall be tested during a all JCR-FBCB2 JLTV Integration and Tests. The test shall be considered successful after MTS-ES connects via L-band satellite transceivers and utilize the RFID readers. | 1 |
| PD2360 | 3.2.4.9.2.2.2 |  | X |  |  | 4.2.4.9.2.2.2 |  |
| PD2361 | The system shall integrate the ODU, and any appropriate ADUs, with the MTS-ES GUI application functionalities of logistics and JCR Control Station which are configurable to be enabled/disabled at login role; for example, a logistic role will enable logistics functionalities in additional to JCR capabilities; a control station role will enable additional application functionalities specific to that role. |  | X |  |  | The MTS-ES system shall be tested during a all JCR-FBCB2 JLTV Integration and Tests. The test shall be considered successful after MTS-ES connects via L-band satellite transceivers, utilize the RFID readers, and outputs the GUI to the ODU and appropriate ADUs. | 2 |
| PD2364 | 3.2.4.9.2.2.3 |  | X |  |  | 4.2.4.9.2.2.3 |  |
| PD2365 | The system shall integrate the RFID subsystem with the ODU to: transmit tagged data to a remote Control Station having been interrogated by other MTS equipped units on an on-demand basis, query and poll RFID tagged header data using tag ID, TCN, consignee DODAAC, and/or commodity class, obtain position information of desired cargo on-demand, and allow the operator to remotely turn off the active RFID Tag interrogation function and turn it back on as needed by the operators. |  | X |  |  | The MTS-ES system shall be tested during a all JCR-FBCB2 JLTV Integration and Tests. The test shall be considered successful after MTS-ES connects via L-band satellite transceivers, utilize the RFID readers, and outputs the GUI to the ODU and appropriate ADUs. | 3 |
| PD2378 | ###### 3.2.4.9.2.3 JCR System Software Environment | X |  |  |  | 4.2.4.9.2.3 |  |
| PD2379 | The VCS shall host and integrate the JCR System Software. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 2 |
| PD2380 | ###### 3.2.4.9.2.4 JCR Networking Capabilities |  | X |  |  | 4.2.4.9.2.4 |  |
| PD2381 | 3.2.4.9.2.4.1 EPLRS. |  | X |  |  | 4.2.4.9.2.4.1 |  |
| PD2382 | The system shall interface and integrate the EPLRS radio to the JCR SW / HW in the JLTV (Reference: JCR v1.0 Tactical Internet System Design Document (TISDD). The EPLRS radio provides data communication services to directly disseminate SA and C2 data among tactical hosts. |  | X |  |  | The EPLRS radio shall be tested by starting and making operational the JCR FBCB2 software thence joining the terrestrial portion of the Tactical Internet. The EPLRS radio shall transmit/receive C2 and SA data IAW the network address book. The test shall be considered successful when the EPLRS radios are operational, join the Tactical Internet IAW network topology and pass JCR C2, and SA. | 2 |
| PD2383 | 3.2.4.9.2.4.2 L-band Satellite. |  | X |  |  | 4.2.4.9.2.4.2 |  |
| PD2384 | The system shall integrate JCR to the L-band Satellite Network (Reference: JCR v1.0 Tactical Internet System Design Document (TISDD). The L-Band Satellite Network utilizes a “hub and spoke” network architecture where all message processing and routing among L-band platforms are performed at the JCR NOC. |  | X |  |  | The L-Band transceiver shall be tested by starting and making operational the JCR FBCB2 software thence joining the terrestrial portion of the Tactical Internet. The L-Band transceiver shall transmit/receive C2 and SA data IAW the network address book. The test shall be considered successful when the L-Band transceiver is operational, joins the celestial Tactical Internet IAW network topology and pass JCR C2, and SA. | 3 |
| PD2385 | 3.2.4.9.2.4.3 |  | X |  |  | 4.2.4.9.2.4.3 |  |
| PD2386 | The system shall integrate the L-band Satellite Network equipment comprised of two primary components: L-band Satellite Transceivers and Programmable Inline Encryption Device (PIED) |  | X |  |  | The L-Band transceiver and in-line PIED shall be tested by starting and making operational the JCR FBCB2 software thence joining the terrestrial portion of the Tactical Internet. The L-Band transceiver with PIED shall transmit/receive C2 and SA data IAW the network address book. The test shall be considered successful when the L-Band transceiver with PIED is operational, joins the celestial Tactical Internet IAW network topology and pass JCR C2, and SA in a SECRET mode. | 3 |
| PD2387 | ##### 3.2.4.9.3 Electronic Logistics Diagnostics and Reporting Capabilities | X | X | X |  | 4.2.4.9.3 |  |
| PD2388 | ###### 3.2.4.9.3.1 General | X |  |  |  | 4.2.4.9.3.1 |  |
| PD2389 | 3.2.4.9.3.1.1 | X |  |  |  | 4.2.4.9.3.1.1 |  |
| PD2390 | The vehicle architecture shall provide the following capabilities and services as detailed in subsequent paragraphs: | X |  |  |  | The JLTV will be inspected for conformance to the section 3 requirement. | 3 |
| PD2391 | a. Centrally managed electronic maintenance | X |  |  |  | The JLTV will be inspected for conformance to the section 3 requirement. | 3 |
| PD2392 | b. Logistic data information management | X |  |  |  | The JLTV will be inspected for conformance to the section 3 requirement. | 3 |
| PD2393 | 3.2.4.9.3.1.2 |  |  |  |  | 4.2.4.9.3.1.2 |  |
| PD2394 | The Electronic Logistics Diagnostics and Reporting Capability computing, I/O and interface requirements are identified in the section entitled Vehicle Computing System. The Electronic Logistics Diagnostics and Reporting Capability display requirements are identified in section entitled "Display and Control Subsystem". |  |  |  |  |  | 3 |
| PD2395 | 3.2.4.9.3.1.3 | X |  | X |  | 4.2.4.9.3.1.3 |  |
| PD2396 | Global Positioning System (GPS) data shall be accessible to system requiring GPS data via the JLTV Data Bus architecture. | X |  | X |  | This event will entail visual inspection of both architectural diagrams and physical inspection of architecture applications on prototype that demonstrate GPS data integration via the J1939 Data Bus. | 2 |
| PD2399 | ###### 3.2.4.9.3.2 Centrally Managed Electronic Maintenance |  | X |  |  | 4.2.4.9.3.2 |  |
| PD2400 | 3.2.4.9.3.2.1 |  | X |  |  | 4.2.4.9.3.2.1 |  |
| PD2401 | The vehicle architecture shall have the ability to query digital and analog subsystems, modules, and components in support of embedded diagnostics. At a minimum, electronic monitoring will cover the following systems: fuel, air intake, engine, cooling, transmission, energy storage, power generation and vehicle speed included as part of the platform or as a kit. |  | X |  |  | This test will verify collection of fault codes, sensor readings, parameters, states, modes by maintenance computer for use by diagnostic algorithms and CBM data logger. | 3 |
| PD2402 | 3.2.4.9.3.2.2 |  | X |  |  | 4.2.4.9.3.2.2 |  |
| PD2403 | The JLTV shall provide total vehicle embedded diagnostic software that shall detect 90% (T) 97% (O) of failures through Built-in Test / Built-in Test Equipment (BIT/BITE) and automatically isolate to a single component/LRU/LRM with 97.5% (T) 99% (O) accuracy. The contractor shall interface to and utilize the information from any existing embedded diagnostic capabilities that are already included in the subsystems, components, and modules resident on the vehicle. |  | X |  |  | This test will verify that the diagnostic software can fault isolate to the accuracies indicated in conjunction with any subordinate diagnostic algorithms in use on subsystems, modules, components. | 3 |
| PD2406 | 3.2.4.9.3.2.3 |  | X |  |  | 4.2.4.9.3.2.3 |  |
| PD2407 | The vehicle architecture shall enable Condition Based Maintenance Plus (CBM+) functionality, based on the total vehicle embedded diagnostic software, such as routine health and status checks, diagnostic monitoring, and data logging. The architecture shall include adequate computer memory storage for storing this CBM+ data |  | X |  |  | This test will verify CBM algorithms are created and that data is shared from the embedded diagnostic and IETM functions to enable CBM analyses. | 3 |
| PD2408 | 3.2.4.9.3.2.4 |  | X |  |  | 4.2.4.9.3.2.4 |  |
| PD2409 | The vehicle architecture shall have the hardware and software interfaces from the electronic maintenance assets to interface with command, control, and communications systems to transmit this information. |  | X |  |  | This test will verify that diagnostic and maintenance data is passed to the C3 systems for reporting to command centers. | 2 |
| PD2414 | ###### 3.2.4.9.3.3 Logistic Data Information Management |  | X |  |  | 4.2.4.9.3.3 |  |
| PD2415 | 3.2.4.9.3.3.1 |  | X |  |  | 4.2.4.9.3.3.1 |  |
| PD2416 | The vehicle architecture shall host and interact with Army logistic software applications and interfaces, such as Digital Preventative Maintenance Checks and Services (DPMCS), Digital Logbook, and Standard Army Maintenance Systems - Enhanced (SAMS-E). |  | X |  |  | This test will verify adequate computing and architectural resources to implement automated logistic applications. | 2 |
| PD7248 | ###### 3.2.4.9.3.4 At-Platform Diagnostics. | X | X |  |  | 4.2.4.9.3.4 |  |
| PD7249 | 3.2.4.9.3.4.1 |  | X |  |  | 4.2.4.9.3.4.1 |  |
| PD7250 | The JLTV shall be compatible with the U. S. Army Maintenance Support Device (MSD), with the auxiliary Internal Combustion Engine (ICE) test hardware. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD7251 | 3.2.4.9.3.4.2 |  | X |  |  | 4.2.4.9.3.4.2 |  |
| PD7252 | The JLTV shall be compatible with the USMC Vehicle Automated Diagnostic System (VADS). |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD7253 | 3.2.4.9.3.4.3 | X |  |  |  | 4.2.4.9.3.4.3 |  |
| PD7254 | The JLTV At-Platform diagnostic connectors shall be easily accessible, hard mounted and environmentally protected. | X |  |  |  | The JLTV will be inspected for conformance to the section 3 requirement. | 3 |
| PD7255 | 3.2.4.9.3.4.4 |  | X |  |  | 4.2.4.9.3.4.4 |  |
| PD7256 | Diagnostic outputs shall be transmitted to the vehicle mounted J1939 female 9 pin Deutsch Connector, which shall conform to SAE J1939-13 ‘Off-board Diagnostic Connector’ dated July 1999, using a format conforming to SAE J1587. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7257 | 3.2.4.9.3.4.5 | X |  |  |  | 4.2.4.9.3.4.5 |  |
| PD7258 | The JLTV shall have a vehicle Diagnostic Connector Assembly (DCA) harness that will allow testing a vehicle without disassembly IAW the Design Guide For Vehicle Diagnostic Connector Assemblies (DCA) Report No. CR-82-588-003 Rev 1, Feb 1998. | X |  |  |  | The JLTV will be inspected for conformance to the section 3 requirement. | 3 |
| PD2421 | #### 3.2.4.10 Antennas | X | X |  |  | 4.2.4.10 |  |
| PD2423 | The JLTV shall integrate a combination of antennas or multi-band antenna(s) that will provide service for each of the communications radios including, but not limited to: SINCGARS, EPLRS, GPS, L-Band SATCOM, and Counter - Radio Controlled Improvised Explosive Device - Electronic Warfare (CREW) systems. | X |  |  |  | Inspection shall be required for government furnished antennas to installed on/integrated with the JTLV. Inspection and Certification shall be considered successful when antennas show no damage and the nameplate matches expected the antenna information.. | 2 |
| PD7232 | ##### 3.2.4.10.1 |  | X |  |  | 4.2.4.10.1 |  |
| PD7233 | If multi-band antenna(s) are provided the performance shall be comparable to the following: |  | X |  |  | Multi-band antennas shall be tested to verify performance equal to or greater than the GFE Antennas. |  |
| PD2430 | ###### 3.2.4.10.1.1 | X |  |  |  | 4.2.4.10.1.1 |  |
| PD2492 | The JLTV shall provide a GPS antenna that complies with the requirements described in MIL-PRF-GB-GRAM-300. | X |  |  |  | Inspection shall be required for government furnished antennas to installed on/integrated with the JTLV. Inspection and Certification shall be considered successful when antennas show no damage and the nameplate matches expected the antenna information.. | 2 |
| PD2489 | ###### 3.2.4.10.1.2 | X |  |  |  | 4.2.4.10.1.2 |  |
| PD2496 | For the CREW system, the antenna(s) shall meet or exceed the performance of the currently fielded CREW antenna suite. | X |  |  |  | Inspection shall be required for government furnished antennas to installed on/integrated with the JTLV. Inspection and Certification shall be considered successful when antennas show no damage and the nameplate matches expected the antenna information.. | 2 |
| PD4017 | ###### 3.2.4.10.1.3 | X |  |  |  | 4.2.4.10.1.3 |  |
| PD2494 | The JLTV shall integrate a L-Band SATCOM antenna of the MT-2011 or MT-2012 series. These antennas are integrated with the transceiver, and no additional antenna is required. | X |  |  |  | Inspection shall be required for government furnished antennas to installed on/integrated with the JTLV. Inspection and Certification shall be considered successful when antennas show no damage and the nameplate matches expected the antenna information. Expected antenna information shall be the I-FIX MT-2011 and MT-2012 series antennas. | 3 |
| PD2284 | ##### 3.2.4.10.2 |  | X |  |  | 4.2.4.10.2 |  |
| PD2285 | All antenna integrations shall include a method of quick stowage or removal for transportation. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2509 | #### 3.2.4.11 Electronic Self-Protection Capability | X | X |  |  | 4.2.4.11 |  |
| PD2510 | ##### 3.2.4.11.1 Counter Radio-Controlled Improvised Explosive Device (RCIED) Electronic Warfare (CREW) | X | X |  |  | 4.2.4.11.1 |  |
| PD4015 | ###### 3.2.4.11.1.1 CREW Integration. |  | X |  |  | 4.2.4.11.1.1 |  |
| PD7059 | The JLTV shall integrate the CREW system based upon the following size, weight and power. CREW 2.1 System 1 has (H W D) dimensions of 13.00” x 13.4” x 19.12”, 960 watts, and weights 63 lbs. CREW 2.1 System 2 has (H W D) dimensions of 4.6” x 18” x 19”, 770 watts and weighs 37 lbs. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD7060 | ###### 3.2.4.11.1.2 | X |  |  |  | 4.2.4.11.1.2 |  |
| PD7061 | The CREW Integration shall receive all power from the vehicle power management/distribution system. | X |  |  |  | The CREW Integration shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD7062 | ###### 3.2.4.11.1.3 |  | X |  |  | 4.2.4.11.1.3 |  |
| PD7063 | If the electronic interface is available, the CREW Integration shall be remote controllable, via the Display and Control Subsystem, with regards to viewing and controlling all parameters similar to front panel operations of the equipment. |  | X |  |  | The CREW Integration shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7066 | ###### 3.2.4.11.1.4 |  | X |  |  | 4.2.4.11.1.4 |  |
| PD7067 | If the electronic interface is available, all error messages for the CREW Integration shall be visible through Display and Control Subsystem. |  | X |  |  | The CREW Integration shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD4309 | ###### 3.2.4.11.1.5 | X |  |  |  | 4.2.4.11.1.5 |  |
| PD4312 | The CREW Remote Control Unit shall be integrated in the crew compartment and be accessible to the crew for operation. | X |  |  |  | The CREW Integration shall be inspected to verify compliance with the Section 3 requirement. | 2 |
| PD4310 | ###### 3.2.4.11.1.6 | X |  |  |  | 4.2.4.11.1.6 |  |
| PD4313 | The CREW Integration shall integrate the CREW GPS antenna and RF antenna cable. | X |  |  |  | The CREW Integration shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD4311 | ###### 3.2.4.11.1.7 | X |  |  |  | 4.2.4.11.1.7 |  |
| PD4314 | The CREW antennas will be provided as GFE, two for CREW 2.1 system 1, and four for CREW 2.1 system 2 shall be integrated into the JLTV. | X |  |  |  | The CREW Integration shall be inspected to verify compliance with the Section 3 requirement. | 2 |
| PD2528 | #### 3.2.4.12 Position and Timing Capability (includes GPS) | X | X |  |  | 4.2.4.12 |  |
| PD2529 | ##### 3.2.4.12.1 |  | X |  |  | 4.2.4.12.1 |  |
| PD2530 | The JLTV shall integrate the Small Serial Interface (SSI) Ground-Based GPS Receiver Applications Module (GB-GRAM) IAW MIL-PRF-GB-GRAM-300 and ICD-GPS-153C. |  | X |  |  | The CREW Integration shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2547 | ##### 3.2.4.12.2 |  | X |  |  | 4.2.4.12.2 |  |
| PD2548 | The C4ISR/EW Architecture shall ensure the proper implementation and distribution of GPS timing and data to include, but not limited to, AN/PSN-11 TOD and One Pulse Per Second (1 PPS) IAW IS-GPS-154C and ICD-GPS-153C. |  | X |  |  | The CREW Integration shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2549 | ##### 3.2.4.12.3 | X |  |  |  | 4.2.4.12.3 |  |
| PD2550 | The GPS unit shall receive all power off the vehicle power management/distribution system. | X |  |  |  | The CREW Integration shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2553 | #### 3.2.4.13 Driver’s Visual Aid Capability (includes DVE) | X | X |  |  | 4.2.4.13 |  |
| PD2568 | ##### 3.2.4.13.1 |  | X |  |  | 4.2.4.13.1 |  |
| PD2569 | The JLTV shall integrate the DVE IAW the DVE Specification MIL-PRF-49256A (CR) Rev. B, the DVE Interface Control Document 3236577, the DVE Electronic Pan and Tilt Module (EPTM) Specification, and the DVE EPTM ICD. |  | X |  |  | The DVE Specification MIL-PRF-49256A (CR) Rev. B, the DVE Interface Control Document 3236577, the DVE Electronic Pan and Tilt Module (EPTM) Specification, and the DVE EPTM ICD shall be made available by the government for information. | 2 |
| PD2554 | ##### 3.2.4.13.2 |  | X |  |  | 4.2.4.13.2 |  |
| PD2555 | The DVE integration shall enable use in both the front and rear directions. |  | X |  |  | The DVE Integration shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2556 | ##### 3.2.4.13.3 | X | X |  |  | 4.2.4.13.3 |  |
| PD2557 | The hardware to secure the DVE sensor and Pan & Tilt Module to the vehicle shall not inhibit the driver's vision. | X | X |  |  | The DVE Integration shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2560 | ##### 3.2.4.13.4 |  | X |  |  | 4.2.4.13.4 |  |
| PD2561 | The vehicle electronics architecture shall accommodate the electronics for the DVE Pan and Tilt Module. |  | X |  |  | The DVE Integration shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2566 | ##### 3.2.4.13.5 |  | X |  |  | 4.2.4.13.5 |  |
| PD2567 | The driver shall be able to position the display using one hand. The display shall lock into place in front of the driver and also in its return position away from the driver. The display positioning shall be able to be adjusted for different driver height profiles. |  | X |  |  | The DVE Integration shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2570 | #### 3.2.4.14 Power Management and Distribution Capability. |  | X |  | X | 4.2.4.14 |  |
| PD2571 | ##### 3.2.4.14.1 General |  | X |  | X | 4.2.4.14.1 |  |
| PD2572 | ###### 3.2.4.14.1.1 |  | X |  |  | 4.2.4.14.1.1 |  |
| PD2573 | The Power Management and Distribution System shall have the capability to detect faults such as electrical shorts, opens, arcs, under voltage, under current, over voltage, over current and report back through the Vehicle Diagnostics System. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2578 | ###### 3.2.4.14.1.2 |  |  |  | X | 4.2.4.14.1.2 |  |
| PD2579 | The power distribution subsystem shall prevent electric shock to the operators and maintainers. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD2580 | ###### 3.2.4.14.1.3 | X |  |  |  | 4.2.4.14.1.3 |  |
| PD2581 | The Power Management and Distribution System shall have an isolated electrical bus IAW MIL-STD-1275D separate from an isolated low voltage electronics power bus IAW MIL-STD-704F. The isolated low voltage bus shall be able to interconnect with the low voltage electronics bus in emergency situations for starting. | X |  |  |  | The Power Management and Distribution System shall be inspected to verify compliance with the Section 3 requirement. | 2 |
| PD2582 | ###### 3.2.4.14.1.4 |  | X |  |  | 4.2.4.14.1.4 |  |
| PD2583 | The Power Management and Distribution System shall be capable of transitioning between on board power sources without any loss of functionality. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD4315 | ###### 3.2.4.14.1.5 | X |  |  |  | 4.2.4.14.1.5 |  |
| PD4316 | The Power Management and Distribution System shall provide for electrical isolation between any AC voltage buses and grounded DC voltage buses. | X |  |  |  | The Power Management and Distribution System shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD4317 | ###### 3.2.4.14.1.6 |  | X |  |  | 4.2.4.14.1.6 |  |
| PD4318 | The Power Management and Distribution System shall provide protection from voltage reversals, short circuits, and arcing. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2623 | ##### 3.2.4.14.2 Low Voltage Distribution |  | X |  |  | 4.2.4.14.2 |  |
| PD2626 | ###### 3.2.4.14.2.1 |  | X |  |  | 4.2.4.14.2.1 |  |
| PD2627 | The low voltage power distribution subsystem shall separate clean and dirty loads. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD4319 | 3.2.4.14.2.1.1 |  | X |  |  | 4.2.4.14.2.1.1 |  |
| PD2629 | Clean loads will be MIL-STD-704F compatible loads and shall not cause transients on the bus outside of the MIL-STD-704F standard. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD4320 | 3.2.4.14.2.1.2 |  | X |  |  | 4.2.4.14.2.1.2 |  |
| PD2631 | Dirty loads will be MIL-STD-1275D compatible loads and that may cause transients outside of the MIL-STD-704F tolerance. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1222 | ##### 3.2.4.14.3 Power Generation. | X | X | X | X | 4.2.4.14.3 |  |
| PD1223 | ###### 3.2.4.14.3.1 DC Power Source/ On-board electrical power requirement. |  | X |  |  | 4.2.4.14.3.1 |  |
| PD1231 | 3.2.4.14.3.1.1 |  |  |  |  | 4.2.4.14.3.1.1 |  |
| PD1232 | The on-board power generation requirements apply to all JLTV sub-configuration except the Command and Control on the Move (C2OTM) sub-configuration. See the vehicle specific annex for the C2OTM requirements. |  |  |  |  | See the vehicle specific annex for the C2OTM requirements. |  |
| PD6687 | 3.2.4.14.3.1.2 |  | X |  |  | 4.2.4.14.3.1.2 |  |
| PD1224 | The JLTV shall be capable of providing 15kW(T), and 20kW (O) sustained electrical power to on-board vehicle subsystems, in addition to the power required for the vehicle hotel loads. Hotel loads include loads required to run the vehicle and not provided as GFE (i.e. vehicle lighting, environmental control units, heater fans and blowers, cooling fans, engine ECU, wipers). |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD1225 | 3.2.4.14.3.1.3 |  | X |  |  | 4.2.4.14.3.1.3 |  |
| PD1226 | The On-board power generation shall be simultaneously provided as the vehicle is charging the batteries and/or capacitors consistent with the specifications of the battery manufacturer. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1227 | 3.2.4.14.3.1.4 |  | X |  |  | 4.2.4.14.3.1.4 |  |
| PD1228 | The On-board power generation shall be provided with engine at tactical idle (T); normal idle (O) and while the vehicle is moving (T). |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1233 | ###### 3.2.4.14.3.2 Exportable electrical power. |  | X | X |  | 4.2.4.14.3.2 |  |
| PD1234 | The JLTV shall be capable of providing 10 kW(T) and 30 kW(O) of sustained electrical power, with engine running at tactical idle speed and/or while the vehicle is moving, to an external power “customer”. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD1235 | 3.2.4.14.3.2.1 |  | X |  |  | 4.2.4.14.3.2.1 |  |
| PD1236 | This power generation capability is in addition to the on-board power generation capability and shall be provided simultaneously with the on board power generation requirements. This capability can be provided as a vehicle add on kit. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD1237 | 3.2.4.14.3.2.2 |  | X | X |  | 4.2.4.14.3.2.2 |  |
| PD1238 | Exportable power shall be IAW MIL-STD-705, MIL-STD-1332D and MEP-STD-OO1. |  | X | X |  | Certification shall be used, and a test conducted, to verify compliance with this requirement. | 3 |
| PD1239 | 3.2.4.14.3.2.3 |  | X |  |  | 4.2.4.14.3.2.3 |  |
| PD1240 | Exportable power shall be provided at nominally 110/120V and 220/240V A/C @ 60 Hz. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1252 | ###### 3.2.4.14.3.3 Depleted Battery Engine Start. |  | X |  |  | 4.2.4.14.3.3 |  |
| PD1253 | In the event of the use of supplemental electronic devices (i.e. radios), the vehicle shall be equipped with a device, which prevents the batteries or other storage devices from being depleted past the appropriate charge level sufficient to start the vehicle. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1254 | 3.2.4.14.3.3.1 |  | X |  |  | 4.2.4.14.3.3.1 |  |
| PD1255 | This device shall have a shutoff switch to allow power to the radios in case of emergency. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1258 | ###### 3.2.4.14.3.4 Energy Storage. | X |  | X | X | 4.2.4.14.3.4 |  |
| PD1259 | Energy storage devices shall be maintenance free, except for removal, replacement and recharging, and shall be of sufficient power to meet the demand of the vehicle subsystems in all climatic conditions. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6871 | 3.2.4.14.3.4.1 | X |  |  |  | 4.2.4.14.3.4.1 |  |
| PD6872 | Energy storage devices shall be readily accessible for service and shall be protected from the environment. | X |  |  |  | The Energy storage devices shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD6869 | 3.2.4.14.3.4.2 | X |  |  |  | 4.2.4.14.3.4.2 |  |
| PD6870 | The energy storage shall be insulated to prevent short circuiting during maintenance and operation. | X |  |  |  | The Energy storage devices shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD6867 | 3.2.4.14.3.4.3 | X |  |  |  | 4.2.4.14.3.4.3 |  |
| PD6868 | The energy storage enclosures shall not be vented into the cab. | X |  |  |  | The Energy storage devices shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD6865 | 3.2.4.14.3.4.4 |  |  |  | X | 4.2.4.14.3.4.4 |  |
| PD6866 | Energy storage enclosures shall be designed to preclude major systems damage or serious personnel injury in the event of a violent gas venting or rupture of energy storage cells causing high pressure within the enclosure. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD6863 | 3.2.4.14.3.4.5 |  |  | X |  | 4.2.4.14.3.4.5 |  |
| PD6864 | Energy storage terminal connectors shall be of such material as to prevent terminal corrosion while maintaining good conductivity. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6981 | 3.2.4.14.3.4.6 |  |  | X |  | 4.2.4.14.3.4.6 |  |
| PD6982 | If lead-acid Starting, Lighting, and Ignition (SLI) batteries are utilized they shall meet the requirements of MIL-PRF-32143 and NATO STANAG 4015. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2584 | ##### 3.2.4.14.4 Power Management System. |  | X | X |  | 4.2.4.14.4 |  |
| PD2585 | ###### 3.2.4.14.4.1 |  | X |  |  | 4.2.4.14.4.1 |  |
| PD2586 | The Vehicle Computer System will use power management via the C4ISR/EW and Automotive Sensor Data Busses to control and collect the status of the power generation, energy storage, and power control/distribution components. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2587 | ###### 3.2.4.14.4.2 |  | X |  |  | 4.2.4.14.4.2 |  |
| PD2588 | The vehicle shall be capable of dynamic load prioritization and load shedding and shall be reconfigurable and allow the crew to prioritize and shed unneeded loads during operations. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD909 | ###### 3.2.4.14.4.3 Capacitor starting |  | X |  |  | 4.2.4.14.4.3 |  |
| PD6875 | If the contractor chooses Capacitor Starting, the following requirements apply: |  | X |  |  |  |  |
| PD910 | The capacitor shall have a minimum service life of 500,000 cycles, maintain at least 75% of cranking power at -25 °F; retain at least 60% of its rated energy capacity after 6 months storage at 75°F, have a minimum operating temperature range of -58 °F to 140 °F, shall be readily air shippable and shall have an internal resistance of less than 22 mOhm at -58°F. |  | X |  |  | The requirement is for the vehicle to start in cold weather conditions. This requirement shall be verified by testing IAW TOP 2-2-650 "Engine cold start". Verification shall be considered successful when the test show that the specified requirement has been met. | 3 |
| PD2589 | ###### 3.2.4.14.4.4 |  | X |  |  | 4.2.4.14.4.4 |  |
| PD2590 | The Power Management System shall also monitor and manage the vehicle isolated low voltage bus batteries to ensure that the vehicle electronics does not drain the batteries below its critical engine starting capability and warn the user when that condition is reached. |  | X |  |  | The vehicle Power Management System shall be tested for proper voltage and functions (all conditions) for conformance to the section 3 requirement. | 3 |
| PD2591 | ###### 3.2.4.14.4.5 |  |  | X |  | 4.2.4.14.4.5 |  |
| PD2592 | The Power Management and Distribution System shall comply with the Power Management Application Program Interface (FCS Document 786-0000102150). (O) |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2599 | ###### 3.2.4.14.4.6 |  | X |  |  | 4.2.4.14.4.6 |  |
| PD2600 | The power distribution subsystem controllers shall be able to control load circuit engagement to all required vehicle loads. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2601 | 3.2.4.14.4.6.1 |  | X |  |  | 4.2.4.14.4.6.1 |  |
| PD2602 | The power controller shall prevent sympathetic tripping. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2603 | 3.2.4.14.4.6.2 |  | X |  |  | 4.2.4.14.4.6.2 |  |
| PD2604 | The power controller shall be able to open the circuit without any damage to itself or other circuits. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2605 | 3.2.4.14.4.6.3 |  | X |  |  | 4.2.4.14.4.6.3 |  |
| PD2606 | The power controller shall have the ability to control power output rise and fall time. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2607 | ##### 3.2.4.14.5 Power Connector/Interfaces. | X |  | X |  | 4.2.4.14.5 |  |
| PD2608 | ###### 3.2.4.14.5.1 | X |  | X |  | 4.2.4.14.5.1 |  |
| PD2609 | The power distribution subsystem shall use standard military grade connectors for interfaces. | X |  | X |  | The power distribution subsystem shall be inspected and Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2612 | ###### 3.2.4.14.5.2 | X |  |  |  | 4.2.4.14.5.2 |  |
| PD2613 | The connector/interfaces shall be accessible by operators, be marked clearly, and provide a secured cap for safety and environmental protection. | X |  |  |  | The power distribution subsystem shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2614 | ##### 3.2.4.14.6 Power Interface for COTS. | X | X |  |  | 4.2.4.14.6 |  |
| PD2615 | ###### 3.2.4.14.6.1 | X | X |  |  | 4.2.4.14.6.1 |  |
| PD2616 | The Power Management and Distribution System will provide at least one 15A 120 VAC outlet accessible by driver and one outlet accessible by the crew, Sine Wave 60 Hz with less than 6% total harmonic distortion. The outlets shall be protected with GFI capability. The outlets shall be covered by a protective cap and controlled by a power switch that provides a visible indicator. | X | X |  |  | The JLTV shall be inspected and tested to verify compliance with the Section 3 requirement. | 2 |
| PD2617 | ###### 3.2.4.14.6.2 | X | X |  |  | 4.2.4.14.6.2 |  |
| PD2618 | The Power Management and Distribution System will provide at least one 10A 12 VDC outlet accessible by driver and one outlet accessible by the crew. The outlets shall be covered by a protective cap and controlled by a power switch that provides a visible indicator. | X | X |  |  | The JLTV shall be inspected and tested to verify compliance with the Section 3 requirement. | 2 |
| PD2621 | ###### 3.2.4.14.6.3 | X | X |  |  | 4.2.4.14.6.3 |  |
| PD2622 | The Power Management and Distribution System will provide at least one 5A 24 VDC outlet accessible by driver and one outlet accessible by the crew. The outlets shall be covered by a protective cap and controlled by a power switch that provides a visible indicator. | X | X |  |  | The JLTV shall be inspected and tested to verify compliance with the Section 3 requirement. | 2 |
| PD2636 | ##### 3.2.4.14.7 Battery Management |  | X |  | X | 4.2.4.14.7 |  |
| PD2641 | ###### 3.2.4.14.7.1 |  | X |  |  | 4.2.4.14.7.1 |  |
| PD2642 | Battery management shall provide SOC for each battery. SOC will be for each series battery pair if batteries are in a series configuration. SOC indication shall be no less than 95% accurate. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2645 | ###### 3.2.4.14.7.2 |  | X |  |  | 4.2.4.14.7.2 |  |
| PD2646 | Battery management shall learn SOC and state of health characteristics of battery newly introduced to the system within two discharge cycles. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2647 | ###### 3.2.4.14.7.3 |  |  |  | X | 4.2.4.14.7.3 |  |
| PD2648 | Battery charging shall be optimized for maximum battery life for the type of energy storage present on the vehicle. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD2649 | ###### 3.2.4.14.7.4 |  | X |  |  | 4.2.4.14.7.4 |  |
| PD2650 | Battery charging shall provide battery equalization to balance the SOC between each battery in a series configuration. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2651 | ##### 3.2.4.14.8 NATO Slave Interface | X | X | X |  | 4.2.4.14.8 |  |
| PD2652 | ###### 3.2.4.14.8.1 | X |  | X |  | 4.2.4.14.8.1 |  |
| PD2653 | The vehicle shall be equipped with standard NATO slave interfaces as defined by STANAG No. 2601. | X |  | X |  | The JLTV shall be inspected and Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2654 | ###### 3.2.4.14.8.2 |  | X |  |  | 4.2.4.14.8.2 |  |
| PD2655 | The NATO slave interfaces shall support recharging of all energy storage on the vehicle, provide MIL-STD-1275D electrical power input into the host vehicle, provide “Slave” starting the host or another vehicle, and provide 28VDC output electrical power to another vehicle. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2656 | ###### 3.2.4.14.8.3 |  | X |  |  | 4.2.4.14.8.3 |  |
| PD2657 | The power distribution system will provide 28 VDC through NATO slave interfaces located externally at the front and rear of the vehicle. These interfaces shall be protected from environmental and fording events. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1244 | ###### 3.2.4.14.8.4 Slave Receptacles and Cable. | X | X | X |  | 4.2.4.14.8.4 |  |
| PD1245 | The vehicle shall be provided with a 24-volt starting/accessory system. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1246 | 3.2.4.14.8.4.1 | X |  | X |  | 4.2.4.14.8.4.1 |  |
| PD1247 | The vehicle shall be equipped with a NATO Intervehicle Power Cable IAW STANAG 4074 with Type 1 plugs that is at least 12 ft. in length. | X |  | X |  | The JLTV shall be inspected and certified for conformance to the section 3 requirement. | 3 |
| PD1248 | 3.2.4.14.8.4.2 |  | X |  |  | 4.2.4.14.8.4.2 |  |
| PD1249 | The vehicle shall be capable of being jump started with or without the batteries connected using a standard NATO power cable and plug assembly. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD1250 | 3.2.4.14.8.4.3 | X |  |  |  | 4.2.4.14.8.4.3 |  |
| PD1251 | The receptacles shall be IAW STANAG 4074 (Type 1 receptacles) and located so as to preclude damage, corrosion or contamination, and tripping hazards upon entering or exiting the cab. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD1260 | ##### 3.2.4.14.9 Silent Watch Capability. |  | X |  |  | 4.2.4.14.9 |  |
| PD1261 | The JLTV FoV, with the engine off and without the use of the auxiliary power unit, shall have the capability of supplying continuous, rechargeable electrical power for silent watch for two (2) hours (T) six (6) hours (O). Silent watch systems are defined in Annex K. This capability may be kitted. |  | X |  |  | The Power Management and Distribution System shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD2662 | ##### 3.2.4.14.10 High Voltage Distribution | X |  | X |  | 4.2.4.14.10 |  |
| PD2663 | ###### 3.2.4.14.10.1 |  |  | X |  | 4.2.4.14.10.1 |  |
| PD2664 | High power low voltage electric loads (greater than 840 watts) shall be integrated onto the high voltage bus. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2667 | ###### 3.2.4.14.10.2 | X |  | X |  | 4.2.4.14.10.2 |  |
| PD2668 | The high voltage power distribution subsystem shall incorporate safety features (e.g. interlocks) as required for high voltage power distribution. | X |  | X |  | The high voltage power distribution subsystem shall be inspected and certified for conformance to the section 3 requirement. | 2 |
| PD2669 | ###### 3.2.4.14.10.3 | X |  |  |  | 4.2.4.14.10.3 |  |
| PD2670 | High voltage power distribution shall be labeled as such and connectors will be physically different from the low voltage connectors. | X |  |  |  | The Power Management and Distribution System shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2671 | ###### 3.2.4.14.10.4 |  |  | X |  | 4.2.4.14.10.4 |  |
| PD2672 | The vehicle shall be designed and manufactured to comply with High Voltage Corona (HVC) per requirements defined in MIL-HDBK-454, Guideline 45 for altitudes up to 15,000 feet. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 2 |
| PD2818 | #### 3.2.4.15 Vehicle Lighting | X | X | X |  | 4.2.4.15 |  |
| PD2819 | ##### 3.2.4.15.1 General | X | X | X |  | 4.2.4.15.1 |  |
| PD2820 | ###### 3.2.4.15.1.1 | X |  |  |  | 4.2.4.15.1.1 |  |
| PD2821 | All clearance lights, marker lights and military composite lights shall be LED. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2822 | ###### 3.2.4.15.1.2 | X |  |  |  | 4.2.4.15.1.2 |  |
| PD2823 | All interior lighting shall be LED (O). | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2824 | ###### 3.2.4.15.1.3 | X |  |  |  | 4.2.4.15.1.3 |  |
| PD2825 | The vehicle exterior and interior lights shall be protected to preclude any damage when interfacing with other vehicles or ancillary equipment and shall be protected from terrain and natural obstacles while traveling cross-country. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2826 | ###### 3.2.4.15.1.4 | X |  | X |  | 4.2.4.15.1.4 |  |
| PD2827 | The vehicle shall be equipped with lamps, reflective devices, and associated equipment as specified per FMVSS 571.108. | X |  | X |  | The JLTV shall be inspected and certified for conformance to the section 3 requirement. | 3 |
| PD2828 | ###### 3.2.4.15.1.5 |  | X |  |  | 4.2.4.15.1.5 |  |
| PD2829 | Actuation of the brakes shall override the vehicle hazard flasher lights. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2830 | ###### 3.2.4.15.1.6 |  | X |  |  | 4.2.4.15.1.6 |  |
| PD2831 | All indicators and gauges shall be illuminated in service mode. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2832 | ##### 3.2.4.15.2 Headlights. | X |  | X |  | 4.2.4.15.2 |  |
| PD2833 | ###### 3.2.4.15.2.1 | X |  |  |  | 4.2.4.15.2.1 |  |
| PD2834 | The JLTV shall incorporate LED headlights. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2835 | ###### 3.2.4.15.2.2 |  |  | X |  | 4.2.4.15.2.2 |  |
| PD2836 | The headlights shall meet the illumination requirements of FMVSS 108. The headlight height restrictions of FMVSS 108 do not apply. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2837 | ###### 3.2.4.15.2.3 |  |  | X |  | 4.2.4.15.2.3 |  |
| PD2838 | Headlights shall meet DOT illumination requirements; LED headlight lamp spec MIL-PRF-32243 (Draft) |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2857 | ##### 3.2.4.15.3 Secure Lighting. | X | X |  |  | 4.2.4.15.3 |  |
| PD2858 | The JLTV shall have a blackout lighting system. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2861 | ###### 3.2.4.15.3.1 | X |  |  |  | 4.2.4.15.3.1 |  |
| PD2862 | Exterior blackout lighting shall consist of, either separately mounted or in a composite light assembly, one blackout drive lamp (reference 12360910), and two rear mounted blackout stop lamp assemblies (reference 12360870). | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2863 | ###### 3.2.4.15.3.2 |  | X |  |  | 4.2.4.15.3.2 |  |
| PD2864 | Interior blackout lighting shall be as required for safe operation of the vehicle and compatible with night vision devices (i.e. night goggles) in use at time of fielding. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2865 | ###### 3.2.4.15.3.3 |  | X |  |  | 4.2.4.15.3.3 |  |
| PD2866 | The emission of any vehicle interior or exterior light source, which may be illuminated (including warning lights) in the blackout mode, shall be limited to the visible spectrum (380 to 700 nanometers). |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2867 | ###### 3.2.4.15.3.4 |  | X |  |  | 4.2.4.15.3.4 |  |
| PD2868 | No energy shall be emitted in the 700 to 1200-nanometer portion of the electromagnetic (EM) spectrum. (Emission peaks shall not exceed 1% relative to the peak emission in the visible spectrum.) |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2869 | ###### 3.2.4.15.3.5 | X |  |  |  | 4.2.4.15.3.5 |  |
| PD2870 | Colored warning lights shall be maintained as necessary while meeting the above requirements. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2871 | ##### 3.2.4.15.4 Crew Indicators. | X | X | X |  | 4.2.4.15.4 |  |
| PD2872 | ###### 3.2.4.15.4.1 | X |  |  |  | 4.2.4.15.4.1 |  |
| PD2873 | The JLTV shall be equipped with gauges/indicators that shall be readily visible to the driver and illuminated for night operation. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2874 | ###### 3.2.4.15.4.2 |  |  |  |  | 4.2.4.15.4.2 |  |
| PD2875 | Gauges/indicators may be replaced by the driver ADU. Speed, tach, water temperature, oil pressure and fuel indicators shall be dedicated gauges that continue to operate if the driver ADU fails. |  |  |  |  |  | 3 |
| PD2876 | ###### 3.2.4.15.4.3 | X |  |  |  | 4.2.4.15.4.3 |  |
| PD2877 | Gauges/indicator shall include as a minimum, fuel level, engine oil level, engine oil quality, engine coolant temperature, transmission fluid temperature, engine oil pressure, engine tachometer, speedometer/odometer, power management suite, air pressure (air assist vehicle/trailer brakes), brake warning, park brake on and air filter gauge. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2878 | 3.2.4.15.4.3.1 | X |  |  |  | 4.2.4.15.4.3.1 |  |
| PD2879 | The JLTV speedometer shall be calibrated in both MPH and KPH. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2880 | 3.2.4.15.4.3.2 | X |  |  |  | 4.2.4.15.4.3.2 |  |
| PD2881 | The JLTV shall be equipped with an odometer to indicate mileage and kilometers. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2884 | ###### 3.2.4.15.4.4 |  | X |  |  | 4.2.4.15.4.4 |  |
| PD2885 | The JLTV shall be equipped with a visual indicator and an audible warning to indicate low air pressure, low oil pressure, and high coolant temperature and shall report thru the vehicle diagnostic system. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2886 | 3.2.4.15.4.4.1 |  | X |  |  | 4.2.4.15.4.4.1 |  |
| PD2887 | The audible warning indicators shall be inactive while in the blackout mode. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2888 | ###### 3.2.4.15.4.5 |  | X |  |  | 4.2.4.15.4.5 |  |
| PD2889 | The JLTV shall be equipped with self canceling turn indicators |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7072 | ###### 3.2.4.15.4.6 | X |  |  |  | 4.2.4.15.4.6 |  |
| PD7073 | The JLTV shall be equipped with an emergency flasher system. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD7074 | ###### 3.2.4.15.4.7 |  | X |  |  | 4.2.4.15.4.7 |  |
| PD7075 | The JLTV shall be equipped with controls to operate and adjust service lights, blackout lights, driving lights and instrument panel lights. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD2890 | ###### 3.2.4.15.4.8 | X |  |  |  | 4.2.4.15.4.8 |  |
| PD2891 | Gauges and switches shall be color coded on the face scale to indicate information such as: desirable operating range in green; cautious, undesirable, or ineffective usage in yellow; dangerous or harmful operating level in red. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2892 | ###### 3.2.4.15.4.9 |  |  | X |  | 4.2.4.15.4.9 |  |
| PD2893 | Lenses shall not discolor throughout the life of the vehicle. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD2894 | ###### 3.2.4.15.4.10 | X |  |  |  | 4.2.4.15.4.10 |  |
| PD2895 | The JLTV shall have one map light located at each occupant seat with individual switches. These lights shall be over-ridden during blackout mode. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD2903 | ### 3.2.5 SUPPORTABILITY. |  |  |  | X | 4.2.5 |  |
| PD6562 | The JLTV FOV and companion trailers shall be designed to maximize the use of common components. Examples of commonality utilization include: instrument panels and gauges; seats; suspensions; wheels, tires, and run-flat inserts; ballistic glass; engines; transmissions; transfer cases; differentials; brushguards; etc. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. |  |
| PD7234 | #### 3.2.5.1 Measurement Standard. |  |  | X |  | 4.2.5.1 |  |
| PD7332 | ##### 3.2.5.1.1 |  |  | X |  | 4.2.5.1.1 |  |
| PD7235 | The JLTV FoV shall utilize only one measurement standard, either Metric or US English. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD7333 | ##### 3.2.5.1.2 |  |  | X |  | 4.2.5.1.2 |  |
| PD7334 | The JLTV FoV shall utilize only the Metric measurement standard. (O) |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 objective requirement. | 3 |
| PD2904 | #### 3.2.5.2 RELIABILITY, AVAILABILITY, and MAINTAINABILITY (RAM) |  |  |  |  | 4.2.5.2 |  |
| PD2905 | ##### 3.2.5.2.1 Reliability. |  |  |  |  | 4.2.5.2.1 |  |
| PD2907 | The extremely high system-level reliability of the JLTV platform coupled with a trained operator/crew is essential to make platform availability goals. It is through inherent, high reliability and maintainability that the JLTV is able to meet its operational goals with a smaller force and logistics footprint. The RAM requirements for the JLTV Family of Vehicles do not include government furnished equipment or mission specific equipment such as radios, weapons, C4, medical, etc. |  |  |  |  |  | 1 |
| PD2908 | ###### 3.2.5.2.1.1 Mean Miles Between Hardware Mission Failure. |  |  |  |  | 4.2.5.2.1.1 |  |
| PD2909 | See vehicle specific annex |  |  |  |  | See vehicle specific annex |  |
| PD2917 | ##### 3.2.5.2.2 Operational Availability. |  |  |  | X | 4.2.5.2.2 |  |
| PD2918 | The JLTV shall demonstrate the operational availability (Ao) of 95% (T), 98% (O). |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 1 |
| PD2919 | Operational Availability (Ao) is the degree (expressed as a decimal between 0 and 1, or the percentage equivalent) to which one can expect a piece of equipment or weapon system to work properly when it is required. Ao is calculated by dividing uptime by the sum of uptime and downtime. It is the quantitative link between readiness objectives and supportability. |  |  |  |  |  |  |
| PD2920 | Ao = Uptime . Uptime + Downtime |  |  |  |  |  |  |
| PD2921 | Uptime is that time when the system is considered to be ready for use and is either operating, in standby, or off |  |  |  |  |  |  |
| PD2922 | Downtime is the time the system is down for repair of operational mission hardware failures and/or for restoration from operational mission software faults, including off-board logistic delays. It also includes planned maintenance time with a periodicity less than or equal to the test duration time that prevents the system from performing its assigned mission. |  |  |  |  |  |  |
| PD3125 | ###### 3.2.5.2.2.1 Service Life. |  |  |  | X | 4.2.5.2.2.1 |  |
| PD3126 | The JLTV shall have a design life that maintains the RAM requirements and is supportable for at least 25 years under the same environment and failure criteria. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD2924 | ##### 3.2.5.2.3 Maintainability. |  |  |  |  | 4.2.5.2.3 |  |
| PD4304 | ###### 3.2.5.2.3.1 |  |  |  |  | 4.2.5.2.3.1 |  |
| PD4305 | The JLTV maintainability requirements do not include the maintenance time required to configure a vehicle into mission mode through conversion or installation of kits. However, the installation of mission kits (to include B armor kit) shall not hinder the maintainability of the system. The time required to repair missions kits as a result of damage caused by design failure or corrosion will be included in the maintenance ratio, mean time to repair, and maximum time to repair calculations. |  |  |  |  |  |  |
| PD3946 | ###### 3.2.5.2.3.2 Maintenance Ratio (Field Level) |  |  |  |  | 4.2.5.2.3.2 |  |
| PD3947 | See vehicle specific annex |  |  |  |  | See vehicle specific annex |  |
| PD6732 | 3.2.5.2.3.2.1 Definition of Field Level Maintenance. |  |  |  |  | 4.2.5.2.3.2.1 |  |
| PD2933 | Field Level Maintenance is repair and return to the user. Maintenance operations assigned to Field Maintenance include: (1) Performance of PMCS (Operator/Crew); (2) Inspections by sight and touch of accessible components per the Technical Manual and condition-based maintenance indicators or instrumentation; (3) Lubrication, cleaning (including corrective actions to repair corrosive damage), preserving (including spot painting), tightening, replacement, and adjustments; (4) Diagnosis and fault isolation; (5) Replacement of unserviceable parts, modules, Line Replaceable Unit (LRU) / Line Replaceable Module (LRM), and assemblies; (6) Verification of faults and level of repair of unserviceable materiel prior to evacuation; (7) Recovery or coordination for transportation of equipment for Field Level Maintenance; (8) Diagnosis and isolation of materiel or module malfunctions, adjustment, and alignment of modules that can be readily completed with assigned tools and Test, Measurements, and Diagnostic Equipment (TMDE); (9) Performance of body repair, including straightening, welding, sanding, and spot painting of skirts, fenders, body, and hull sections when required to stop corrosion or restore structural integrity; and (10) Turn-in of unserviceable end items and components. |  |  |  |  |  |  |
| PD3945 | ###### 3.2.5.2.3.3 Maintenance Ratio (Sustainment Level) |  |  |  |  | 4.2.5.2.3.3 |  |
| PD2925 | See vehicle specific annex |  |  |  |  | See vehicle specific annex |  |
| PD6733 | 3.2.5.2.3.3.1 Definition of Sustainment Level Maintenance. |  |  |  |  | 4.2.5.2.3.3.1 |  |
| PD2934 | Sustainment maintenance is repair and return to the supply system. Operations assigned to sustainment maintenance units/activities include: (1) Inspection, diagnosis, isolation, and repair of faults within assemblies, modules and components for repair and return to stock; (2) Sustainment maintenance returns part assembles to a National Maintenance Program standard [This process involves inspection and diagnosis, according to the depot maintenance work requirements or similar technical directions, that identifies all components exhibiting wear and directs the replacement or adjustment of those items to original equipment specification]; (3) Performance of heavy body, hull, turret, and frame repair; (4) Collection and classification of Class VII materiel (less medical materiel) for proper disposition; (5) Turn-in of unserviceable end items and components; (6) Special inspections and modifications of equipment requiring extensive disassembly or elaborate test equipment as part of cyclic overhaul or special depot maintenance programs; and (7) Nondestructive testing to determine the serviceability of removed used parts. |  |  |  |  |  |  |
| PD2970 | ###### 3.2.5.2.3.4 Time to Repair. |  |  |  |  | 4.2.5.2.3.4 |  |
| PD2971 | 3.2.5.2.3.4.1 Mean time to repair |  |  |  | X | 4.2.5.2.3.4.1 |  |
| PD2972 | Each JLTV shall have a Mean Time To Repair (MTTR) of 0.5 clock-hours or less for field level maintenance. MTTR is measured as "hood up to hood down repair time and includes isolation of failure and repair, remove and replace" |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 2 |
| PD2975 | MTTR is the sum of corrective maintenance times divided by the total number of corrective maintenance actions during a given period of time under stated conditions. MTTR applies to the system-level configuration; it will be used as an "on-system" maintainability index and not for the repair of components. |  |  |  |  |  |  |
| PD2973 | 3.2.5.2.3.4.2 Maximum time to repair (Field level) |  |  |  |  | 4.2.5.2.3.4.2 |  |
| PD3953 | That time below which a specified percentage of all corrective maintenance tasks must be completed. MaxTTR is used as an "on-system" maintainability parameter; it is not used for the off-system repair of replaced components. MaxTTR is measured as "hood up to hood down repair time and includes isolation of failure and repair / remove and replace." |  |  |  |  |  | 3 |
| PD3955 | 3.2.5.2.3.4.2.1 |  |  |  | X | 4.2.5.2.3.4.2.1 |  |
| PD2977 | The JLTV family of vehicles shall have a maximum time to repair (MaxTTR) for Crew/Operator maintenance tasks of 0.5 clock-hours. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3954 | 3.2.5.2.3.4.2.2 |  |  |  | X | 4.2.5.2.3.4.2.2 |  |
| PD3956 | The JLTV family of vehicles shall have a maximum time to repair (MaxTTR) for field level maintenance tasks performed by a MOS Mechanic(s) of 2.5 clock-hours (T) and 2 clock-hours (O). |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3000 | 3.2.5.2.3.4.2.2.1 Removal and Replacement |  |  |  | X | 4.2.5.2.3.4.2.2.1 |  |
| PD3001 | All vehicle variants of this family shall be designed so that each of the following (if applicable) can be removed from the vehicle and replaced in under 2.5 clock hours (T) and 1 hour (O) by 2 maintainers. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3002 | a. Transfer Case (if applicable) |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3003 | b. Engine (only) |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3004 | c. Transmission (only) |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3005 | d. Engine & transmission (powerpack) |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3964 | ###### 3.2.5.2.3.5 Tools | X |  |  | X | 4.2.5.2.3.5 |  |
| PD3970 | The JLTV family of vehicles shall be capable of being maintained by 20 total common tools for both Crew/Operator and mechanics. The tools include two sets (screw driver set and a socket driver handle set). | X |  |  | X | The JLTV shall be inspected and analyzed to verify compliance with the Section 3 requirement. | 3 |
| PD3968 | 3.2.5.2.3.5.1 | X |  |  | X | 4.2.5.2.3.5.1 |  |
| PD3971 | The operator tool set shall include no more than 10 common tools, including BII. | X |  |  | X | The JLTV shall be inspected and analyzed to verify compliance with the Section 3 requirement. | 3 |
| PD3091 | 3.2.5.2.3.5.2 | X |  |  | X | 4.2.5.2.3.5.2 |  |
| PD3092 | Mechanic tool set shall require only 10 additional common tools from the operator tool set to perform the remainder of the Field Level tasks. | X |  |  | X | The JLTV shall be inspected and analyzed to verify compliance with the Section 3 requirement. | 3 |
| PD3972 | 3.2.5.2.3.5.3 Special tools | X |  |  | X | 4.2.5.2.3.5.3 |  |
| PD3976 | The JLTV family of vehicles shall be able to be maintainable without special tools or test measuring diagnostic equipment (TMDE) beyond a multimeter (no calibration required) and a torque wrench (no calibration required) at field level. | X |  |  | X | The JLTV shall be inspected and analyzed to verify compliance with the Section 3 requirement. | 3 |
| PD3973 | 3.2.5.2.3.5.3.1 | X |  |  | X | 4.2.5.2.3.5.3.1 |  |
| PD3094 | No special tools or TMDE shall be required at the operator level. | X |  |  | X | The JLTV shall be inspected and analyzed to verify compliance with the Section 3 requirement. | 3 |
| PD2946 | ###### 3.2.5.2.3.6 Component Accessibility and Identification. | X |  |  | X | 4.2.5.2.3.6 |  |
| PD2947 | 3.2.5.2.3.6.1 | X |  |  |  | 4.2.5.2.3.6.1 |  |
| PD2948 | All reservoirs, filters, drains, vents and valves shall be easily accessible and identified for inspection and servicing. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD2949 | 3.2.5.2.3.6.1.1 | X |  |  |  | 4.2.5.2.3.6.1.1 |  |
| PD2950 | Drain plugs installed in engine, transmission, transfer case, axles, and hydraulic reservoir shall be of the permanent magnet type and readily accessible. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD2951 | 3.2.5.2.3.6.1.2 | X |  |  |  | 4.2.5.2.3.6.1.2 |  |
| PD2952 | The function of all drains, vents and valve openings shall not adversely affect the function of or damage to any other vehicle component (i.e. battery box). | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD2953 | 3.2.5.2.3.6.1.3 | X |  |  |  | 4.2.5.2.3.6.1.3 |  |
| PD2954 | Provisions shall be in place to prevent draining fluids from contacting other components of the vehicle. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD2963 | 3.2.5.2.3.6.2 | X |  |  |  | 4.2.5.2.3.6.2 |  |
| PD2964 | The JLTV shall have mounting and stowage provisions for all Basic Issue Items (BII) and onboard tools that is operationally accessible and securable. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD7274 | 3.2.5.2.3.6.2.1 | X |  |  | X | 4.2.5.2.3.6.2.1 |  |
| PD7275 | The JLTV BII shall not include any special tools for maintenance. | X |  |  | X | The JLTV shall be inspected and analyzed to verify compliance with the Section 3 requirement. | 3 |
| PD2978 | ###### 3.2.5.2.3.7 Preventive Maintenance Checks & Services (PMCS). | X |  |  | X | 4.2.5.2.3.7 |  |
| PD2980 | 3.2.5.2.3.7.1 Preventive Maintenance Checks and Services (PMCS). | X |  |  | X | 4.2.5.2.3.7.1 |  |
| PD2981 | Crew chief/operator PMCS requirements shall be automated to the maximum extent possible using onboard sensors and diagnostics to simplify checks and reduce operator workload. The section entitled Electronic Logistics Diagnostics and Reporting system will be used to manage and record maintenance actions. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD2982 | 3.2.5.2.3.7.1.1 | X |  |  |  | 4.2.5.2.3.7.1.1 |  |
| PD2983 | The PMCS results shall be displayed to the crew/operators. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD2988 | 3.2.5.2.3.7.1.2 | X |  |  | X | 4.2.5.2.3.7.1.2 |  |
| PD2989 | PMCS in total shall require no more than 10 minutes to complete. | X |  |  | X | The vehicle shall be analyzed and inspected to verify compliance with the Section 3 requirement. | 3 |
| PD3006 | 3.2.5.2.3.7.1.3 | X |  |  |  | 4.2.5.2.3.7.1.3 |  |
| PD3007 | JLTV equipment checked as part of routing/daily maintenance checks, i.e., engine oil, coolant level, battery liquid level, etc., shall be physically accessible without the use of tools. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3014 | ###### 3.2.5.2.3.8 Maintenance Access (doors and panels). | X |  |  | X | 4.2.5.2.3.8 |  |
| PD3015 | The JLTV shall provide accessibility for servicing, adjusting, and replacing elements without tear-down of any major part, component, or element to be compatible with the maintenance ratio requirement. | X |  |  | X | The JLTV shall be analyzed and inspected to verify compliance with the Section 3 requirement. | 2 |
| PD3032 | ###### 3.2.5.2.3.9 Electrical Connectors. | X |  |  |  | 4.2.5.2.3.9 |  |
| PD3034 | 3.2.5.2.3.9.1 | X |  |  |  | 4.2.5.2.3.9.1 |  |
| PD3035 | Electrical/electronic connectors to be used on LRUs shall be keyed to prevent incorrect application. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3036 | 3.2.5.2.3.9.2 | X |  |  |  | 4.2.5.2.3.9.2 |  |
| PD3037 | Electrical connectors shall be selected that provide for connection and tightening without the use of tools and with ease of installation and replacement of LRUs. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3038 | ###### 3.2.5.2.3.10 Automotive Filters. | X | X | X |  | 4.2.5.2.3.10 |  |
| PD3044 | 3.2.5.2.3.10.1 | X |  |  |  | 4.2.5.2.3.10.1 |  |
| PD3045 | All filters within the JLTV for water, fuel, oil, hydraulic, pneumatic, and air, shall be designed with the following additional features: |  |  |  |  |  | 3 |
| PD3046 | a. All filters shall be directly accessible by the operator/maintainer/crew (with or without B armor kit installed). | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3047 | b. Life of the filters shall be displayed through the Diagnostic system. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3048 | c. The filter elements shall be able to be changed in five minutes with the use of onboard tools(T) and one minute or less without the use of tools(O), by the operator or crew. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3042 | 3.2.5.2.3.10.2 |  |  | X |  | 4.2.5.2.3.10.2 |  |
| PD3043 | The engine air filter element (if applicable) shall meet MIL-PRF-46736 for a 200 hour service life capacity. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3051 | 3.2.5.2.3.10.3 Air Cleaner. |  |  |  |  | 4.2.5.2.3.10.3 |  |
| PD3052 | 3.2.5.2.3.10.3.1 |  |  | X |  | 4.2.5.2.3.10.3.1 |  |
| PD3053 | The vehicle shall incorporate an air cleaner system that complies with the requirements of MIL-PRF-62048, Air Cleaners, Automotive, Heavy Duty and Dry Type; at the 200 hour service life(T), 400 hour(O). |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3054 | 3.2.5.2.3.10.3.2 |  |  | X |  | 4.2.5.2.3.10.3.2 |  |
| PD3055 | The air filtration system shall achieve a minimum of 99.9%(T), 99.99%(O), filtration with SAE coarse test dust.. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3056 | 3.2.5.2.3.10.3.3 |  |  | X |  | 4.2.5.2.3.10.3.3 |  |
| PD3057 | The air filtration system shall achieve a minimum filtration of 99.5%(T), 99.99(O), with SAE fine test dust. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3060 | ###### 3.2.5.2.3.11 Interactive Electronic Technical Manuals (IETM) |  | X |  |  | 4.2.5.2.3.11 |  |
| PD3096 | 3.2.5.2.3.11.1 Interactive Electronic Technical Manuals (IETM). |  | X |  |  | 4.2.5.2.3.11.1 |  |
| PD3097 | Each JLTV VCS shall host and operate full IETM's that include operator and maintainer technical manuals (TMs) and Repair Parts and Special Tool Lists (RPSTL) for all onboard equipment, including GFE items. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3098 | 3.2.5.2.3.11.1.1 |  | X |  |  | 4.2.5.2.3.11.1.1 |  |
| PD3099 | The JLTV VCS shall host and operate an embedded training system to assist the mechanic/operator in performing maintenance tasks and diagnosis. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3102 | 3.2.5.2.3.11.2 I.E.T.M Embedded Video Maintenance Support. |  | X |  | X | 4.2.5.2.3.11.2 |  |
| PD3103 | The JLTV IETM software platform shall allow the operator/maintainer to view actual video coverage of Field and Sustainment Level Maintenance Tasks. (O) |  | X |  | X | If attempting to meet the objective requirement, an analysis shall be required to demonstrate compliance. The JLTV shall then be tested to verify compliance with the Section 3 objective requirement. | 3 |
| PD3104 | 3.2.5.2.3.11.2.1 |  | X |  | X | 4.2.5.2.3.11.2.1 |  |
| PD3105 | The operator/maintainer will use the IETM for standard maintenance fault isolation tasks, on-system video maintenance task demonstration, and on-system video instructional or refresher training. (O) |  | X |  | X | If attempting to meet the objective requirement, an analysis shall be required to demonstrate compliance. The JLTV shall then be tested to verify compliance with the Section 3 objective requirement. | 3 |
| PD3106 | 3.2.5.2.3.11.2.2 |  | X |  | X | 4.2.5.2.3.11.2.2 |  |
| PD3107 | The IETM will have a multi option capability allowing the maintainer to access various tasks and use links to access video instructions/demonstrations for the task. (O) |  | X |  | X | If attempting to meet the objective requirement, an analysis shall be required to demonstrate compliance. The JLTV shall then be tested to verify compliance with the Section 3 objective requirement. | 3 |
| PD3108 | 3.2.5.2.3.11.2.3 |  | X |  | X | 4.2.5.2.3.11.2.3 |  |
| PD3109 | The video function shall allow start, stop, pause, rewind, fast-forward, and return to the maintenance window. This will allow the maintainer the option of viewing a maintenance task on video and returning to the maintenance procedures to begin the task. (O) |  | X |  | X | If attempting to meet the objective requirement, an analysis shall be required to demonstrate compliance. The JLTV shall then be tested to verify compliance with the Section 3 objective requirement. | 3 |
| PD3110 | 3.2.5.2.3.11.2.4 |  | X |  | X | 4.2.5.2.3.11.2.4 |  |
| PD3111 | The video shall be formatted using memory-reduced compression, and can be viewed through the Display and Control Subsystem. (O) |  | X |  | X | If attempting to meet the objective requirement, an analysis shall be required to demonstrate compliance. The JLTV shall then be tested to verify compliance with the Section 3 objective requirement. | 3 |
| PD6543 | #### 3.2.5.3 SAFETY. | X | X | X | X | 4.2.5.3 |  |
| PD3161 | ##### 3.2.5.3.1 Cab Crush Protection. |  |  |  | X | 4.2.5.3.1 |  |
| PD3162 | The JLTV cab shall have a crush resistant roof capable of withstanding 150%(T), 200%(O) of the vehicles’ GVW (excluding GPK and RWS) placed on top of the cab in a vertical plane. Maximum vertical cab deformation into the occupant compartment shall be 3 inches or not greater than the headroom between the top of the helmet of a combat equipped 95% male occupant and the roof edge, whichever is less. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 1 |
| PD6549 | ###### 3.2.5.3.1.1 |  |  |  | X | 4.2.5.3.1.1 |  |
| PD6552 | Each representative JLTV variant type shall provide a roof crush resistant structure that shall pass the government’s inverted drop test at curb vehicle weight, 10 degrees pitch angle and 12.5 degrees roll angle, and vehicle drop height of 30 inches. Maximum vertical deformation in the occupant compartment is 3 inches or not greater than the headroom between the top of the helmet of a combat equipped 95% male occupant and the roof edge, whichever is less. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 1 |
| PD6917 | ##### 3.2.5.3.2 Side Impact Protection. |  |  |  | X | 4.2.5.3.2 |  |
| PD6918 | The JLTV shall have side impact protection that prevents serious or fatal injuries to the vehicle's occupants as described in FMVSS 571.214. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 2 |
| PD3176 | ##### 3.2.5.3.3 Crew Restraint System (seat belts). | X | X | X |  | 4.2.5.3.3 |  |
| PD3177 | Each occupant seat shall have a combined seat and restraint device. | X |  | X |  | Inspection and Certification shall be provided to verify compliance to the Section 3 requirement. | 2 |
| PD3178 | ###### 3.2.5.3.3.1 | X |  | X |  | 4.2.5.3.3.1 |  |
| PD6920 | The JLTV FoV seat and restraint device shall pass the specifications and testing listed in FMVSS 207,208, 209 and 210. | X |  | X |  | Inspection and Certification shall be provided to verify compliance to the Section 3 requirement. | 3 |
| PD6919 | ###### 3.2.5.3.3.2 | X |  |  |  | 4.2.5.3.3.2 |  |
| PD3179 | The restraints shall be a minimum of a 4 point system. The restraints shall not contain a “loop” that will snag an occupant's gear upon release of the restraint. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD3180 | ###### 3.2.5.3.3.3 |  | X |  |  | 4.2.5.3.3.3 |  |
| PD3181 | The restraints shall be able to be released by a single movement of the hand in less than one second. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3182 | ###### 3.2.5.3.3.4 |  | X |  |  | 4.2.5.3.3.4 |  |
| PD3183 | The vehicle shall sense an imminent vehicle rollover situation and respond by tightening the crew restraints to restrain the occupants in their seats (T) and by lowering the seat (O). The system shall be capable of multiple uses such that it does not have to be re-charged or re-set between events. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD3186 | ###### 3.2.5.3.3.5 |  | X |  |  | 4.2.5.3.3.5 |  |
| PD3187 | The restraint system shall allow vehicle and crew operation without hindrance or the need to loosen and/or remove the restraint. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3153 | ##### 3.2.5.3.4 Crew Ingress/Egress |  | X |  | X | 4.2.5.3.4 |  |
| PD6849 | ###### 3.2.5.3.4.1 Crew Ingress |  | X |  |  | 4.2.5.3.4.1 |  |
| PD6852 | 3.2.5.3.4.1.1 |  |  |  |  | 4.2.5.3.4.1.1 |  |
| PD6851 | Crew ingress time includes the time it takes for all occupants to open the door(s), step up into the vehicle, sit in assigned seat, close the door, and fully latch the seatbelt. |  |  |  |  |  | 3 |
| PD6853 | 3.2.5.3.4.1.2 |  | X |  |  | 4.2.5.3.4.1.2 |  |
| PD6854 | Total crew ingress time for a crew of six shall be 45 seconds or less, with occupants in combat gear. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6855 | 3.2.5.3.4.1.3 |  | X |  |  | 4.2.5.3.4.1.3 |  |
| PD6856 | Total crew ingress time for a crew of four shall be 30 seconds or less with occupants in combat gear. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6850 | ###### 3.2.5.3.4.2 Crew Egress |  | X |  | X | 4.2.5.3.4.2 |  |
| PD3152 | The JLTV, with or without B armor kit, shall provide provisions for the crew and passengers (in combat equipment) to quickly egress from the vehicle during combat or normal operations. Crew is defined as the number of occupants specified in the vehicle specific annexes for each sub-configuration. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD3154 | Crew egress times shall be as follows: |  | X |  | X |  | 2 |
| PD3155 | 1-2 personnel within 6 seconds(T), 4 seconds(O) |  | X |  | X | The JLTV shall be tested and analyzed to verify compliance with the Section 3 requirement. | 2 |
| PD3156 | 3 personnel within 9 seconds(T), 6 seconds(O) |  | X |  | X | The JLTV shall be tested and analyzed to verify compliance with the Section 3 requirement. | 2 |
| PD3157 | 4-6 personnel within 13 seconds(T), 9 seconds(O) |  | X |  | X | The JLTV shall be tested and analyzed to verify compliance with the Section 3 requirement. | 2 |
| PD3158 | 7 personnel(if applicable) within 18 seconds(T), 13 seconds(O) |  | X |  | X | The JLTV shall be tested and analyzed to verify compliance with the Section 3 requirement. | 2 |
| PD7236 | ##### 3.2.5.3.5 Toxic Gases. |  | X | X |  | 4.2.5.3.5 |  |
| PD7237 | ###### 3.2.5.3.5.1 Carbon Monoxide (CO). |  | X | X |  | 4.2.5.3.5.1 |  |
| PD7238 | Personnel, while occupying, operating, or maintaining the JLTV, shall not be exposed to CO concentrations emitted by the vehicle that result in Carboxyhemoglobin (COHB) blood levels greater than 10%. |  | X | X |  | Certification and Testing shall be required to support verification of the requirement. COHB levels shall be calculated from MIL-HDBK-759, using work effort level 3 for mission activities. Testing will be conducted per TOP 2-2-614. | 3 |
| PD7239 | ###### 3.2.5.3.5.2 Other Toxic Gases |  | X | X |  | 4.2.5.3.5.2 |  |
| PD7240 | Nitrogen dioxide, ether, ammonia, nitric oxide and sulfur dioxide emitted by the JLTV shall be limited to concentrations not to exceed those specified in the Threshold Limit Values for Chemical Substances in Work Air by the American Conference of Governmental Industrial Hygienists. |  | X | X |  | Certification and Testing shall be required to support verification of the requirement. Testing will be conducted per TOP 2-2-614. | 3 |
| PD3131 | #### 3.2.5.4 HUMAN SYSTEMS INTEGRATION (HSI)/MANPOWER & PERSONNEL INTEGRATION (MANPRINT). | X | X | X | X | 4.2.5.4 |  |
| PD3132 | ##### 3.2.5.4.1 Human Factor Engineering. |  | X | X | X | 4.2.5.4.1 |  |
| PD3133 | ###### 3.2.5.4.1.1 |  |  | X |  | 4.2.5.4.1.1 |  |
| PD3134 | The JLTV shall be in conformance with MIL-STD-1472. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3137 | 3.2.5.4.1.1.1 |  |  |  | X | 4.2.5.4.1.1.1 |  |
| PD3138 | All vehicle configurations shall ensure functionality, ease and safety of operation for all functions performed by operational and maintenance personnel based on 5th percentile female through 95th percentile male. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD1571 | ###### 3.2.5.4.1.2 Noise Limits. |  | X |  | X | 4.2.5.4.1.2 |  |
| PD1572 | The JLTV shall comply with the impulsive and steady state noise limits of MIL-STD-1474 at all crew locations, both inside and outside the vehicle. The vehicle shall comply with category C* interior steady state noise limits as outlined in MIL-STD-1474, Table 2, at all personnel occupied areas. |  | X |  | X | The JLTV shall be tested and analyzed to verify compliance with the Section 3 requirement. Noise limits shall be measured while the JLTV is moving at 2/3 top rated speed and the test conditions as stated in MIL-STD-1474. | 3 |
| PD913 | ###### 3.2.5.4.1.3 Heating, Ventilation, Air Conditioning (HVAC) and Defroster. |  | X | X | X | 4.2.5.4.1.3 |  |
| PD6570 | JLTV shall be capable of meeting the crew compartment HVAC and defrosting requirements with vehicle in A structure and B armor kit armor configurations. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD914 | 3.2.5.4.1.3.1 Heater |  | X |  | X | 4.2.5.4.1.3.1 |  |
| PD915 | 3.2.5.4.1.3.1.1 |  | X |  | X | 4.2.5.4.1.3.1.1 |  |
| PD916 | The heater shall be capable of raising the crew compartment temperature from -25° F to +41° F (-32 to +5 C) within 45 minutes (T) 20 minutes (O) after the vehicle has been started. |  | X |  | X | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD917 | 3.2.5.4.1.3.1.2 |  | X |  |  | 4.2.5.4.1.3.1.2 |  |
| PD918 | The vehicle’s heater shall be able to maintain a crew compartment temperature of no less than 68° F (20° C) at an ambient temperature of -25° F (-32 C). |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD921 | 3.2.5.4.1.3.1.3 |  | X |  |  | 4.2.5.4.1.3.1.3 |  |
| PD922 | The control of the air flow from blower shall operate independent of the heater's temperature controls. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD937 | 3.2.5.4.1.3.2 Ventilation. |  | X |  |  | 4.2.5.4.1.3.2 |  |
| PD6985 | 3.2.5.4.1.3.2.1 |  | X |  |  | 4.2.5.4.1.3.2.1 |  |
| PD938 | The JLTV vehicle cabs shall comply with the ventilation system performance requirements in MIL-STD-1472. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD919 | 3.2.5.4.1.3.2.2 |  | X |  |  | 4.2.5.4.1.3.2.2 |  |
| PD920 | Individual vents/ducts shall have hand moveable controls to adjust the amount of air output and position the air flow in a range from directly on occupants to completely off occupants. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6988 | 3.2.5.4.1.3.2.3 |  | X |  |  | 4.2.5.4.1.3.2.3 |  |
| PD6989 | The JLTV ventilation system shall have the capability to adjust the origin of air flow throughout the range of 100% fresh air to 100% recirculated air. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD927 | 3.2.5.4.1.3.3 Air Conditioning. |  | X | X |  | 4.2.5.4.1.3.3 |  |
| PD930 | Crew compartment cooling requirements shall be met with windows closed and under full radiant heat loading (I.E from the sun, crew, electronics, engine heat) in the operating range of the vehicle. |  |  |  |  |  | 3 |
| PD929 | 3.2.5.4.1.3.3.1 |  | X | X |  | 4.2.5.4.1.3.3.1 |  |
| PD928 | The JLTV air conditioning system shall be capable of maintaining a temperature of not greater than 85°F (29.5° C) (T); 76 F (24 ° C) (O) within the crew compartment. |  | X | X |  | Certification shall be provided, and testing conducted, to verify compliance with the Section 3 requirement. | 3 |
| PD6986 | 3.2.5.4.1.3.3.2 |  | X | X |  | 4.2.5.4.1.3.3.2 |  |
| PD6987 | The JLTV air conditioning system shall be capable of lowering the crew compartment temperature to 85°F (29.5° C) within 60 minutes (T) and 40 minutes (O) when the ambient temperature is 130°F. |  | X | X |  | Certification shall be provided, and testing conducted, to verify compliance with the Section 3 requirement. | 3 |
| PD931 | 3.2.5.4.1.3.3.3 |  |  | X |  | 4.2.5.4.1.3.3.3 |  |
| PD932 | Air conditioner shall operate using environmentally friendly cooling agent (e.g. CO2) Exceptions to this environmental requirement shall not be used unless approved in writing by the contracting officer. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD923 | 3.2.5.4.1.3.4 Defroster |  | X |  |  | 4.2.5.4.1.3.4 |  |
| PD924 | The windshield shall be capable of being defrosted within 30 minutes in accordance with SAE J381 (exception: ambient temperature shall be at -50° F (-46° C) with arctic heater kit and -25°F (-31° C) without arctic heater kit). |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3147 | ##### 3.2.5.4.2 Crew Compartment. | X | X | X | X | 4.2.5.4.2 |  |
| PD3148 | ###### 3.2.5.4.2.1 Cab. |  | X |  |  | 4.2.5.4.2.1 |  |
| PD3149 | 3.2.5.4.2.1.1 |  | X |  |  | 4.2.5.4.2.1.1 |  |
| PD3150 | Operator (aided or unaided) shall be able to see the ground or roadway when negotiating terrain at maximum break-over angles. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6921 | 3.2.5.4.2.1.2 Interior Occupant Protection. |  | X |  | X | 4.2.5.4.2.1.2 |  |
| PD6922 | 3.2.5.4.2.1.2.1 |  |  |  | X | 4.2.5.4.2.1.2.1 |  |
| PD6923 | The JLTV FoV shall provide interior impact protection for each occupant as described in FMVSS 571.201 |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3163 | 3.2.5.4.2.1.3 Seating. | X | X | X |  | 4.2.5.4.2.1.3 |  |
| PD3164 | 3.2.5.4.2.1.3.1 |  | X | X |  | 4.2.5.4.2.1.3.1 |  |
| PD3165 | Driver seat (T) all occupants seats (O) shall be individually adjustable fore and aft and up and down, to the occupant's height. |  | X | X |  | Certification shall be provided, and testing conducted, to verify compliance with the Section 3 requirement. | 3 |
| PD3166 | 3.2.5.4.2.1.3.2 |  |  |  |  | 4.2.5.4.2.1.3.2 |  |
| PD3167 | The JLTV design shall provide isolation and mitigation capabilities from all blast, IED, and ballistic events contained in Classified Annex E with all loads applied, including mine blast and shock from the vehicle landing back on the ground after the blast event and rollover. |  |  |  |  | See Annex E | 2 |
| PD3168 | 3.2.5.4.2.1.3.3 |  | X |  |  | 4.2.5.4.2.1.3.3 |  |
| PD3169 | Each seat and restraint system on the JLTV FoV shall be designed to accommodate a soldier/marine wearing CBRNE (MOPP IV), cold weather protective clothing and full combat Individual Body Armor (IBA) to include headgear and Load Bearing Equipment (LBE). Crew weights per MIL-STD-1366. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD7030 | 3.2.5.4.2.1.3.4 |  | X |  |  | 4.2.5.4.2.1.3.4 |  |
| PD7031 | All seats shall recline a sufficient amount such that an occupant wearing full body armor will not be seated in a position where his upper body leans forward beyond an upright, vertical position. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6924 | 3.2.5.4.2.1.3.5 Seat Head Restraints. | X |  | X |  | 4.2.5.4.2.1.3.5 |  |
| PD6925 | The JLTV FoV shall incorporate head restraints in accordance with FMVSS 571.202. | X |  | X |  | The JLTV shall be inspected and certified to verify compliance with the Section 3 requirement. | 3 |
| PD3170 | 3.2.5.4.2.1.3.6 Re-configurable/removable seats. | X |  |  |  | 4.2.5.4.2.1.3.6 |  |
| PD3172 | 3.2.5.4.2.1.3.6.1 | X |  |  |  | 4.2.5.4.2.1.3.6.1 |  |
| PD3171 | The JLTV shall be fitted with removable/re-configurable rear seats. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6675 | 3.2.5.4.2.1.3.6.1.1 |  | X |  |  | 4.2.5.4.2.1.3.6.1.1 |  |
| PD3173 | The seats shall be removable by the vehicle operator with on-board tools(T), without tools(O), within 5 minutes. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3174 | 3.2.5.4.2.1.3.6.2 Casualty Evacuation. |  | X |  |  | 4.2.5.4.2.1.3.6.2 |  |
| PD3175 | The JLTV seats shall be reconfigurable to allow for the carrying of a standard folding/collapsable litter (in unfolded/expanded position), NSN 6530-00-783-7905 and NSN 6530-00-783-7205, in support of non-standard casualty evacuation. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6990 | 3.2.5.4.2.1.3.6.3 |  | X |  |  | 4.2.5.4.2.1.3.6.3 |  |
| PD6991 | Additional JLTV seats shall be capable of being added for additional seating locations with the removal of mission specific equipment. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3188 | 3.2.5.4.2.1.4 Windshield & Windows. | X |  | X | X | 4.2.5.4.2.1.4 |  |
| PD3189 | 3.2.5.4.2.1.4.1 |  |  |  | X | 4.2.5.4.2.1.4.1 |  |
| PD3190 | Windshields and windows shall be configured to minimize solar glare. |  |  |  | X | Analysis shall be used to verify compliance with this requirement. | 3 |
| PD3191 | 3.2.5.4.2.1.4.2 Protection from sun glare. | X |  |  |  | 4.2.5.4.2.1.4.2 |  |
| PD3200 | Windshields or other transparent areas through which high acuity vision is required shall not be impaired due to sun glare protection. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD6736 | 3.2.5.4.2.1.4.2.1 | X |  |  |  | 4.2.5.4.2.1.4.2.1 |  |
| PD3192 | Visors or other means shall be used to preclude performance degradation due to glare from external sources such as sunlight or headlights. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD3193 | 3.2.5.4.2.1.4.2.2 | X |  |  |  | 4.2.5.4.2.1.4.2.2 |  |
| PD3194 | Visors or other means shall provide coverage across the entire width of the windshield and side windows to protect front seat occupants from glare. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD3195 | 3.2.5.4.2.1.4.2.3 | X |  |  |  | 4.2.5.4.2.1.4.2.3 |  |
| PD3196 | Visors or other means shall have a mechanical detent to prevent movement while in the stowed position. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6927 | 3.2.5.4.2.1.4.3 Automotive Glass. |  |  | X |  | 4.2.5.4.2.1.4.3 |  |
| PD6929 | 3.2.5.4.2.1.4.3.1 |  |  | X |  | 4.2.5.4.2.1.4.3.1 |  |
| PD6930 | The design of the standard automotive glass (windshields and windows) and replacement materials shall meet the glazing materials specifications called out in FMVSS 571.205 "Glazing Materials". |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6931 | 3.2.5.4.2.1.4.3.2 |  |  | X |  | 4.2.5.4.2.1.4.3.2 |  |
| PD6932 | Automotive glass windshields on the JLTV FoV shall pass the testing specified in FMVSS 571.219, Windshield zone intrusion. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3201 | 3.2.5.4.2.1.5 Windshield Wipers & Washers. | X | X | X |  | 4.2.5.4.2.1.5 |  |
| PD3202 | The cab shall be equipped with multi-speed windshield wipers with an adjustable, intermittent wiper setting, and windshield washing system. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3203 | 3.2.5.4.2.1.5.1 | X |  |  |  | 4.2.5.4.2.1.5.1 |  |
| PD3204 | A minimum size of 3-qt (2.8 l) washer reservoir compatible with cleaner and appropriate additives for the climatic conditions for destination shall be furnished. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3205 | 3.2.5.4.2.1.5.2 |  |  | X |  | 4.2.5.4.2.1.5.2 |  |
| PD3206 | Windshield wipers and washers shall conform to FMVSS 571.104 and SAE J198, and be compatible to all thicknesses of windshield armor protection, see Annex E. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3207 | 3.2.5.4.2.1.6 Vehicle Cab Interior. | X |  |  |  | 4.2.5.4.2.1.6 |  |
| PD3208 | The vehicle cab interior and upholstery shall be dark, non-reflective color. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3213 | 3.2.5.4.2.1.7 Cab Floor Drains. | X |  |  |  | 4.2.5.4.2.1.7 |  |
| PD3219 | 3.2.5.4.2.1.7.1 | X |  |  |  | 4.2.5.4.2.1.7.1 |  |
| PD3220 | The drainage system shall meet ballistic and blast protection levels called out in Annex E. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3231 | 3.2.5.4.2.1.8 M4/M16 Rifle Mounting. |  | X |  |  | 4.2.5.4.2.1.8 |  |
| PD3232 | The JLTV shall provide stowage capable of accepting all versions of the M4 and M16 rifles. The mounted rifles shall not interfere with vision, operation of vehicle controls, or vehicle ingress/egress, and shall be accessible to vehicle occupants without hindrance or the need to loosen and/or remove the seat restraint. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD7068 | 3.2.5.4.2.1.8.1 | X |  |  |  | 4.2.5.4.2.1.8.1 |  |
| PD7069 | The JLTV shall provide stowage of one rifle per crew member. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3235 | 3.2.5.4.2.1.9 Beverage holders. | X |  |  |  | 4.2.5.4.2.1.9 |  |
| PD3236 | The cab of the vehicle shall be equipped with rugged, cup holders for the driver and co-driver that are capable of holding containers in the range of a standard 12 ounce aluminum soda pop can to a 24 ounce plastic soda pop bottle (O). | X |  |  |  | The JLTV shall be inspected, if provided, to verify compliance with the Section 3 requirement. | 3 |
| PD3237 | 3.2.5.4.2.1.9.1 | X |  |  |  | 4.2.5.4.2.1.9.1 |  |
| PD3238 | The cup holders shall hold the containers securely without spilling during normal primary and secondary road operations (O). | X |  |  |  | The JLTV shall be inspected, if provided, to verify compliance with the Section 3 requirement. | 3 |
| PD3239 | 3.2.5.4.2.1.9.2 | X |  |  |  | 4.2.5.4.2.1.9.2 |  |
| PD3240 | The cup holders shall not interfere with combat operations (O). | X |  |  |  | The JLTV shall be inspected, if provided, to verify compliance with the Section 3 requirement. | 3 |
| PD3242 | 3.2.5.4.2.1.10 Rear View Mirrors. | X |  | X |  | 4.2.5.4.2.1.10 |  |
| PD3243 | Mirrors conforming to A-A-52432 and FMVSS 111 shall be provided on the left and right hand sides of the JLTV and be capable of folding toward the vehicle body sides in both directions. | X |  | X |  | Inspection and Certification shall be provided to verify compliance to the Section 3 requirement. | 3 |
| PD3244 | 3.2.5.4.2.1.11 Stowage. | X |  |  |  | 4.2.5.4.2.1.11 |  |
| PD3245 | Stowage space with latching device to utilize a standard military padlock shall be provided to accommodate Basic Issue Items (BII), publications (operator, hand receipt and warranty) and operator's Common Table of Allowances (CTA) 50-900 personal clothing and equipment. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6547 | 3.2.5.4.2.1.11.1 Additional Storage Space. | X |  |  |  | 4.2.5.4.2.1.11.1 |  |
| PD6548 | In addition to the storage of BII items and tools called out in this specification, the JLTV shall have a minimum of 60 cubic feet of additional, enclosed storage space that is protected from the elements, e.g. rain, snow, etc. that is suitable for the storage of personal gear, sleeping bags and other supplies. Minimum size of additional storage areas shall be no less than 4 cubic feet per storage area. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3246 | 3.2.5.4.2.1.11.2 | X |  |  |  | 4.2.5.4.2.1.11.2 |  |
| PD3247 | All stowage boxes and stowage spaces shall contain drain holes. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3250 | 3.2.5.4.2.1.11.3 | X |  |  |  | 4.2.5.4.2.1.11.3 |  |
| PD3251 | Provisions shall be included that inhibit contents of BII from obstructing the drain holes. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3252 | 3.2.5.4.2.1.12 Crew Compartment Integration. |  |  | X |  | 4.2.5.4.2.1.12 |  |
| PD3253 | Crew compartment integration shall consider population characteristics, crew task requirements, crew workload through the mission, functional relationships between controls and displays, crew compartment physical constraints, operating environment, assessments of available display technologies, and crew physical limitations. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3280 | 3.2.5.4.2.1.13 Door and Entry Point operation. | X |  | X |  | 4.2.5.4.2.1.13 |  |
| PD3281 | All JLTV doors and entry points shall latch securely in the closed position. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6937 | 3.2.5.4.2.1.13.1 |  |  | X |  | 4.2.5.4.2.1.13.1 |  |
| PD6938 | The JLTV FoV shall provide door lock and door retention component capabilities as specified in FMVSS 571.206 |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3282 | 3.2.5.4.2.1.13.2 Combat Lock | X |  | X |  | 4.2.5.4.2.1.13.2 |  |
| PD3283 | All JLTV doors and entry points shall be capable of being locked from the inside for combat missions and unlocked from the outside with rescue tool per drawing, 6437086, CAGE CODE, 6W728, per attachment J. | X |  | X |  | Inspection and Certification shall be provided to verify compliance to the Section 3 requirement. | 3 |
| PD3284 | 3.2.5.4.2.1.13.3 | X |  |  |  | 4.2.5.4.2.1.13.3 |  |
| PD3285 | Provision shall be made to prevent inadvertent actuation of internal door and entry point control handles while entering or leaving the platform, performing routine mission duties, or performing maintenance on the platform. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6909 | 3.2.5.4.2.1.13.4 | X |  |  |  | 4.2.5.4.2.1.13.4 |  |
| PD6910 | Doors (with & without B armor kit) shall be equipped with an external ring/eye, as a rescue provision for First Responders to remove/open any of the doors in the event of a combat emergency situation. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3286 | 3.2.5.4.2.1.13.5 |  | X |  |  | 4.2.5.4.2.1.13.5 |  |
| PD3287 | JLTV Doors, without B armor kit installed, shall be capable of being opened and closed by 5th-95th percentile military personnel, without injury or vehicle damage, on 60% slopes (facing up and down) and on 40% side slopes. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD6939 | 3.2.5.4.2.1.13.6 |  | X | X |  | 4.2.5.4.2.1.13.6 |  |
| PD6940 | JLTV Doors, when B armor kit is installed, shall be capable of being opened and closed by 5th-95th percentile military personnel, without injury or vehicle damage, on 30% (T), 40% (O), slopes (facing up and down) and side slopes. |  | X | X |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3288 | 3.2.5.4.2.1.13.7 |  | X |  |  | 4.2.5.4.2.1.13.7 |  |
| PD3289 | At least one door/escape portal shall be accessible to all on-board personnel, and be capable of being opened by 5th-95th percentile military personnel, even when the vehicle has rolled upside down in a narrow ditch, where normal door operation is not possible, or on either side. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3394 | #### 3.2.5.5 Vehicle Security. | X |  |  |  | 4.2.5.5 |  |
| PD3395 | The JLTV shall have a means to provide vehicle security (e.g., door locks, locking hatches and fuel tanks, etc.). | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD3396 | ##### 3.2.5.5.1 | X |  |  |  | 4.2.5.5.1 |  |
| PD3397 | The security system shall provide the capability to lock the entry points from inside the vehicle without inhibiting a quick exit from the vehicle. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3398 | ##### 3.2.5.5.2 | X |  |  |  | 4.2.5.5.2 |  |
| PD3399 | Each door shall be provided with a latching device that will permit the door to be pad locked from the outside with pad lock A-A-59487. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3379 | #### 3.2.5.6 Painting, & Corrosion. |  |  | X | X | 4.2.5.6 |  |
| PD6573 | ##### 3.2.5.6.1 Paint. |  |  | X |  | 4.2.5.6.1 |  |
| PD6574 | Unless otherwise specified, all external surfaces of the JLTV shall have a finish coat of Chemical Agent Resistant Coating (CARC) meeting MIL-DTL-64159, Type II. Color shall be Green 383, chip number 34094 per FED-STD-595. Camouflage pattern requirement of the system specification need not apply. CARC primer coat shall be MIL- P 53030, MIL-DTL-0053084, A-A-52474, or an ARL approved powder coat. Surface preparation, quality assurance and application of all CARC coatings shall be done IAW MIL-DTL 53072. If A-A-52474 primer is used, topcoat DFT shall be in the range of 2.0 to 2.5 mils. All interiors of vehicles shall receive a color determined by the PM office. The following items shall not be painted: terminal wiring connections, instruction diagrams and plates, instrumentation, rubber, lubrication fittings, hoses, nozzles and all other parts whose operation or function would be adversely affected by paint. Insulation material shall be painted unless the sound absorbing characteristics of the material are compromised. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3384 | ##### 3.2.5.6.2 Corrosion Resistance. |  |  |  | X | 4.2.5.6.2 |  |
| PD3385 | The vehicle shall be designed to operate for a minimum of 25 years(T), 30 years(O), corrosion service life, which will include varying or extended periods in corrosive environments involving one or more of the following: high humidity, salt spray, road de-icing agents, ground contact, gravel impingement, atmospheric contamination and temperature extremes (-50 to +160 F degrees). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3386 | ###### 3.2.5.6.2.1 |  |  |  | X | 4.2.5.6.2.1 |  |
| PD3387 | Only normal washing (includes high pressure), scheduled maintenance (exclusive of paint touch up) and repair of accidentally damaged areas (not a result of intended use, deficiency in design, materials, manufacturing or normal wear), shall be necessary to keep the corrosion prevention in effect. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD6941 | #### 3.2.5.7 Markings & Data Plates. | X |  |  |  | 4.2.5.7 |  |
| PD6942 | ##### 3.2.5.7.1 Markings. | X |  |  |  | 4.2.5.7.1 |  |
| PD6943 | Vehicle markings shall be IAW MIL-STD-642. Markings such as safety, fuel type, fill level of liquids, tire pressure, and instructional markings shall be letters or numerals one inch in height. Painted markings, letters and numerals shall be lusterless paint conforming to MI-C-46168 or MIL-C-53039. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6944 | ##### 3.2.5.7.2 Data Plates. | X |  |  |  | 4.2.5.7.2 |  |
| PD6945 | Instruction, caution, identification, operating and data plates shall be provided IAW A-A-50271 and installed in a readily visible location. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6950 | ###### 3.2.5.7.2.1 Instruction Plates. | X |  |  |  | 4.2.5.7.2.1 |  |
| PD6953 | The vehicle shall be equipped with instructions, plates or diagrams, including cautions and warnings describing any special or important procedures to be followed in assembling, operation or servicing. Instruction plate(s) shall completely describe and depict operating tasks. Shipping dataplate and identification, in accordance with (IAW) MIL-STD-209 shall be provided. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6949 | ###### 3.2.5.7.2.2 Identification Plate. | X |  |  |  | 4.2.5.7.2.2 |  |
| PD6954 | The JLTV shall have an identification (name) plate that contains the vehicle serial number, model number, description, national stock number, USA registration number, date of manufacture, contractor's name, and contact number for that vehicle. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6948 | ###### 3.2.5.7.2.3 Lubricant Plate. | X |  |  |  | 4.2.5.7.2.3 |  |
| PD6955 | Installed adjacent to the identification (name) plate shall be a lubricant plate IAW SAE J753 except that it shall reference military lubricants. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6947 | ###### 3.2.5.7.2.4 Vehicle Weight Classification Sign. | X |  |  |  | 4.2.5.7.2.4 |  |
| PD6951 | A vehicle weight classification sign as described in drawing no. 6-1-2248 (81337) shall be provided. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6946 | 3.2.5.7.2.4.1 Location. | X |  |  |  | 4.2.5.7.2.4.1 |  |
| PD6952 | The vehicle weight classification sign shall be installed in a location behind the front bumper and shall not interfere with other vehicle functions. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3416 | #### 3.2.5.8 Rear Reflective Signature. | X |  | X |  | 4.2.5.8 |  |
| PD3417 | Exterior safety markings meeting the intent of the requirements of FMVSS 571.108, shall be applied on the rear of the JLTV. | X |  | X |  | The JLTV shall be inspected and certified to verify compliance with the Section 3 requirement. | 3 |
| PD3440 | #### 3.2.5.9 Kits. | X | X |  | X | 4.2.5.9 |  |
| PD3441 | The vehicle shall operate IAW the specification requirements after installation of and use of the kits specified herein. The application and use of these kits shall in no way affect the life expectancy or performance of the vehicle or its components. The vehicle shall be furnished with the interface requirements for the kits, such as predrilled holes, electrical hook-up, hole accesses, coolant ports for heater and etc. The vehicle shall have space and power allocation to accept installation of all or any combination of the kits described herein. |  |  |  |  |  | 2 |
| PD3442 | ##### 3.2.5.9.1 |  |  |  | X | 4.2.5.9.1 |  |
| PD3443 | Each kit, shall not take longer than 4 man-hours to initially install at Field Level maintenance and subsequent installation shall be completed by the operator within 2 hours(T) and 0.5 hours(O). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3446 | ##### 3.2.5.9.2 Engine Arctic Kit. |  |  |  | X | 4.2.5.9.2 |  |
| PD3447 | The JLTV shall provide an engine arctic kit IAW the Operating Parameters of this ATPD. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3452 | ##### 3.2.5.9.3 Vehicle Medical Aid Kit. |  |  |  | X | 4.2.5.9.3 |  |
| PD3453 | Each JLTV shall be equipped with a Medical Aid kit (First aid kit) to treat injuries to personnel per NSN 6545-00-922-1200, PN 11677011 (19207). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD4052 | ##### 3.2.5.9.4 CBRN Detector Kit. |  |  |  | X | 4.2.5.9.4 |  |
| PD4056 | The JLTV shall be capable of accepting a chemical, nuclear detector IAW the requirements in the Detection section of this ATPD. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD3400 | ##### 3.2.5.9.5 Winch Kit. |  |  |  | X | 4.2.5.9.5 |  |
| PD3401 | The JLTV shall be capable of accepting a winch kit with cable, chain, shackle, and snatch block. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD3402 | ###### 3.2.5.9.5.1 |  |  |  | X | 4.2.5.9.5.1 |  |
| PD3403 | This winch shall be capable of operations to both the front and rear of the vehicle. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3404 | ###### 3.2.5.9.5.2 Location change. (if applicable) |  |  |  | X | 4.2.5.9.5.2 |  |
| PD3405 | The time to change winch location shall not exceed 30 minutes (T), 15 minutes or less (O) by two soldiers/marines (T), one soldier/marine (O) using JLTV BII tools. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3406 | ###### 3.2.5.9.5.3 |  |  |  | X | 4.2.5.9.5.3 |  |
| PD3407 | This winch shall be capable of being removed and installed on another vehicle in 30 minutes (T), 15 minutes or less (O) by two soldiers/marines (T), one soldier/marine (O) using JLTV BII tools. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3408 | ###### 3.2.5.9.5.4 |  |  |  | X | 4.2.5.9.5.4 |  |
| PD3409 | The winch, cable, and BII winch accessories shall be able to withstand and overcome loads equal to 1.5 times the GVW of the JLTV (T), 2 times GVW (O). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3410 | ###### 3.2.5.9.5.5 |  |  |  | X | 4.2.5.9.5.5 |  |
| PD3411 | The winch cable shall be long enough to reach an anchor 45 feet (T), 75 feet (O) from the JLTV and return (using the snatch block) to enable self-recovery with a 2:1 mechanical advantage. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3412 | ###### 3.2.5.9.5.6 |  |  |  | X | 4.2.5.9.5.6 |  |
| PD3413 | The winch shall incorporate an automatic brake that stops the cable from paying out when not under power. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3414 | ###### 3.2.5.9.5.7 |  |  |  | X | 4.2.5.9.5.7 |  |
| PD3415 | The JLTV winch kit shall provide control for outside-vehicle operations. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD2850 | ##### 3.2.5.9.6 Convoy Warning Lights Kit. | X |  |  |  | 4.2.5.9.6 |  |
| PD2851 | ###### 3.2.5.9.6.1 | X |  |  |  | 4.2.5.9.6.1 |  |
| PD2852 | There shall be provisions for readily mounting and connecting a commercial, yellow strobe type LED convey warning light, per NSN 2590-01-107-9696, on the vehicle suitable for operation in all locations. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD2855 | ###### 3.2.5.9.6.2 | X |  |  |  | 4.2.5.9.6.2 |  |
| PD2856 | The warning light shall be visible for 360 degrees and shall not be capable of being activated during the blackout mode. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD7279 | ##### 3.2.5.9.7 Exterior Work Lamps Kit. | X |  |  |  | 4.2.5.9.7 |  |
| PD7280 | The JLTV shall be capable of accepting a minimum of two (2) work lamps that meet the following requirements to facilitate low-light and maintenance operations: | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD7281 | ###### 3.2.5.9.7.1 | X |  |  |  | 4.2.5.9.7.1 |  |
| PD7282 | The work lamps shall be adjustable such that the lamps can be aimed at areas around the rear and sides of the vehicle. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD7283 | ###### 3.2.5.9.7.2 | X |  |  |  | 4.2.5.9.7.2 |  |
| PD7284 | The work lamps shall be provided with a 30 foot minimum length power cord and shall run off vehicle power. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD7285 | ###### 3.2.5.9.7.3 | X |  |  |  | 4.2.5.9.7.3 |  |
| PD7286 | The work lamps shall provide a minimum illumination of 1,000 lux over a minimum surface area of 50 square feet. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD7287 | ###### 3.2.5.9.7.4 | X |  |  |  | 4.2.5.9.7.4 |  |
| PD7288 | The work lamps shall be equipped with an individual on-off switch. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD7291 | ###### 3.2.5.9.7.5 | X |  |  |  | 4.2.5.9.7.5 |  |
| PD7292 | An on/off switch accessible from the driver's position shall be furnished and operate separately from the work lamps themselves. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD7289 | ###### 3.2.5.9.7.6 | X |  |  |  | 4.2.5.9.7.6 |  |
| PD7290 | The work lamps shall be robust enough to sustain a 3 foot drop onto the ground without damage. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. |  |
| PD6676 | ##### 3.2.5.9.8 Fording Kit. |  | X |  | X | 4.2.5.9.8 |  |
| PD6677 | The JLTV shall be capable of accepting a fording kit IAW the fording depth and venting requirements in the Fording section of this document. |  | X |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. Testing shall be conducted if kitted. | 2 |
| PD6678 | ##### 3.2.5.9.9 Exportable Power Kit. |  | X |  | X | 4.2.5.9.9 |  |
| PD6679 | The JLTV shall be capable of accepting a exportable power kit (if kitted ) IAW the Exportable Electrical Power section of this document. |  | X |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. Testing shall be conducted if kitted. | 2 |
| PD6737 | ##### 3.2.5.9.10 RPG Protection Kit. |  |  |  | X | 4.2.5.9.10 |  |
| PD6738 | The JLTV shall be capable of accepting an RPG protection kit IAW Annex G of the JLTV ATPD. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 1 |
| PD6739 | ##### 3.2.5.9.11 Silent Watch Energy Storage Kit. |  |  |  | X | 4.2.5.9.11 |  |
| PD6740 | The JLTV shall be capable of accepting a rechargeable silent watch energy storage kit (if kitted) IAW vehicle specific annexes. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD6741 | ##### 3.2.5.9.12 Vehicle Obscuration Smoke System Kit. |  |  |  | X | 4.2.5.9.12 |  |
| PD6742 | The JLTV shall be capable of accepting a vehicle obscuration smoke system kit IAW the Vehicle Obscuration Smoke System section of this document. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 2 |
| PD6992 | ##### 3.2.5.9.13 Materiel Handling Equipment Kit. |  |  |  | X | 4.2.5.9.13 |  |
| PD6993 | JLTV shall be capable of accepting a Material Handling Equipment (MHE) kit that meets the following requirements: |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD6994 | ###### 3.2.5.9.13.1 |  |  |  | X | 4.2.5.9.13.1 |  |
| PD6995 | The kit shall include a fully electric crane, remote control, and associated hardware. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD6996 | ###### 3.2.5.9.13.2 |  |  |  | X | 4.2.5.9.13.2 |  |
| PD6997 | The crane shall be fully powered by the vehicle electrical system. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD6998 | ###### 3.2.5.9.13.3 |  |  |  | X | 4.2.5.9.13.3 |  |
| PD6999 | The crane shall have the ability to be operated via remote control from outside the cab. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7000 | ###### 3.2.5.9.13.4 |  |  |  | X | 4.2.5.9.13.4 |  |
| PD7001 | The crane shall be fully operable without removal of other on-vehicle equipment, such as spare tire/carrier assemblies. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7002 | ###### 3.2.5.9.13.5 |  |  |  | X | 4.2.5.9.13.5 |  |
| PD7003 | The crane shall include a means to illuminate all areas within the span of the crane boom. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7010 | ###### 3.2.5.9.13.6 Lift Capability. |  |  |  | X | 4.2.5.9.13.6 |  |
| PD7011 | The MHE kit shall have lift capability of 500 lb (T) 1,000 lb (O). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7012 | ###### 3.2.5.9.13.7 Lift Rate. |  |  |  | X | 4.2.5.9.13.7 |  |
| PD7013 | The crane shall have the capacity of lowering or raising the load at a speed not less than 30 feet/min (9 m/min). |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7014 | ###### 3.2.5.9.13.8 Lift Traverse Angle. |  |  |  | X | 4.2.5.9.13.8 |  |
| PD7015 | The crane shall have a minimum working traverse of 370° with a rated capacity of 125% of static capacity at inner and outermost reach. The traverse overlap shall be toward the rear of the vehicle. The boom length shall be a minimum of 4 feet. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7016 | ###### 3.2.5.9.13.9 Installation. |  |  |  | X | 4.2.5.9.13.9 |  |
| PD7017 | The JLTV MHE shall be able to be installed or uninstalled by two operators, on the vehicle in 5 minutes. The JLTV MHE shall be able to be safely stored on board the vehicle during normal operating conditions. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7018 | ###### 3.2.5.9.13.10 |  |  |  | X | 4.2.5.9.13.10 |  |
| PD7019 | The crane shall be capable of being installed and removed from various locations necessary to assist in the removal/replacement of any damaged tire with a spare and installation/removal of B armor kit panels. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7020 | ###### 3.2.5.9.13.11 Vehicle Stability. |  |  |  | X | 4.2.5.9.13.11 |  |
| PD7021 | The MHE kit shall be designed so that when operating on an unloaded JLTV, with tires at highway pressure, and crane at maximum capacity, the system tipping moment shall not exceed 85% of the system righting moment when tested in accordance with SAE J765, except that the test shall occur on a 7% lateral slope. A safety switch shall be integrated with the system to preclude use of the crane on slopes larger than 7%. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7022 | ###### 3.2.5.9.13.12 |  |  |  | X | 4.2.5.9.13.12 |  |
| PD7023 | When installed on the vehicle and during operation, the stress imparted by the crane assembly (including mounting hardware) on any member shall not exceed the margin of safety provided in SAE J1063 when tested accordingly. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7024 | ###### 3.2.5.9.13.13 |  |  |  | X | 4.2.5.9.13.13 |  |
| PD7025 | A means shall be provided to lower any load to the ground in the event of an electrical system or control failure. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7026 | ###### 3.2.5.9.13.14 |  |  |  | X | 4.2.5.9.13.14 |  |
| PD7027 | A load capacity chart shall be visible to the crane operator. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD7028 | ###### 3.2.5.9.13.15 |  |  |  | X | 4.2.5.9.13.15 |  |
| PD7029 | The remote control box shall contain appropriate operating instructions. |  |  |  | X | Analysis shall be provided to demonstrate compliance to the Section 3 requirement. | 3 |
| PD3324 | #### 3.2.5.10 Fuels and Lubricants. | X | X | X | X | 4.2.5.10 |  |
| PD3326 | The JLTV shall be fully operational with standard military fuels, lubricants, and fluids as required by the climatic operating requirements without component degradation and adverse affect on the vehicle performance or warranty provisions. |  |  |  |  |  | 2 |
| PD3325 | ##### 3.2.5.10.1 Fuel. |  |  | X | X | 4.2.5.10.1 |  |
| PD3327 | ###### 3.2.5.10.1.1 JP-8 |  |  | X | X | 4.2.5.10.1.1 |  |
| PD3328 | The primary fuel for operation and testing will be JP-8 per MIL-DTL-83133, but the vehicle shall be capable of operating on DF2 diesel fuel per A-A-52557 or ASTM D975. |  |  | X | X | Certification shall be analyzed to verify compliance with this requirement. | 2 |
| PD3329 | ###### 3.2.5.10.1.2 |  |  | X | X | 4.2.5.10.1.2 |  |
| PD3330 | The JLTV shall be capable of operating with alternate fuels as defined by AR-70-12 with minimal operational impact. |  |  | X | X | Certification shall be analyzed to verify compliance with this requirement. | 3 |
| PD3331 | ###### 3.2.5.10.1.3 |  |  | X | X | 4.2.5.10.1.3 |  |
| PD3332 | The JLTV shall be capable of operating with emergency fuels, as defined by AR-70-12, for short periods of time not to exceed 24 hours, although significant degradation in performance will be anticipated. |  |  | X | X | Certification shall be analyzed to verify compliance with this requirement. | 3 |
| PD3335 | ###### 3.2.5.10.1.4 |  |  | X | X | 4.2.5.10.1.4 |  |
| PD3336 | If liquid cooled, the engine shall be serviced with a solution of propylene glycol conforming to A-A-52624 and water in equal parts by volume. If ethylene glycol is desired to be used, per A-A-52624, the contractor must submit a written justification on the need for its use, and the request must be approved in writing by the contracting officer. |  |  | X | X | Certification shall be analyzed to verify compliance with this requirement. | 3 |
| PD3337 | ###### 3.2.5.10.1.5 Fuel efficiency. |  | X |  | X | 4.2.5.10.1.5 |  |
| PD3338 | The JLTV shall meet a fuel efficiency of 60(T), 90(O), ton-miles per gallon based on maximum GVW, including armor. Fuel efficiency will be measured over the Munson Standard Fuel Consumption course per TOP 2.2.603. |  | X |  | X | The JLTV shall be tested and analyzed to verify compliance with the Section 3 requirement. Test shall be conducted IAW TOP 2-2-603. | 1 |
| PD6555 | ###### 3.2.5.10.1.6 Fuel Conservation. |  | X |  | X | 4.2.5.10.1.6 |  |
| PD6557 | The JLTV shall incorporate an automatic device that shuts off the vehicle after it has been at normal idle for more than 10 minutes without being moved/used. The shut off device shall cut off all power to the engine/powerpack, except for the engine cooling system, such that no power drains to the batteries/energy storage device occurs after shut-off. The device shall have a “combat override” switch so that the driver/operator can turn off this automatic shut off feature in a combat or silent watch situation. The override switch shall return to its active, default, position every time the engine is turned off. |  | X |  | X | The JLTV shall be tested and analyzed to verify compliance with the Section 3 requirement. | 2 |
| PD3343 | ###### 3.2.5.10.1.7 Fuel Access Connection. | X |  |  |  | 4.2.5.10.1.7 |  |
| PD3344 | The JLTV shall have a fuel access connection to allow access to JLTV fuel for auxiliary power units. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3345 | ##### 3.2.5.10.2 Lubricants |  |  | X |  | 4.2.5.10.2 |  |
| PD3346 | ###### 3.2.5.10.2.1 Vehicle Lubrication. |  |  | X |  | 4.2.5.10.2.1 |  |
| PD3347 | Two fluids (T) and a single fluid (O) (excluding grease and gear oils) shall be used for all lubrication and hydraulic applications on the JLTV in each operating environment, e.g. -25 F to 130 F and -26 F and below. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3358 | 3.2.5.10.2.1.1 |  |  | X |  | 4.2.5.10.2.1.1 |  |
| PD3359 | Grease lubrication fittings shall conform to SAE J534. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3472 | #### 3.2.5.11 Engine/Drive Train. | X | X | X | X | 4.2.5.11 |  |
| PD3473 | ##### 3.2.5.11.1 Engine Cooling System. | X |  | X | X | 4.2.5.11.1 |  |
| PD3474 | The cooling system shall meet the requirements of SAE J1436 except for the following: Inspection of fluid fill levels shall be accomplished without removal of caps from coolers or surge tanks. | X |  | X |  | Inspection and Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3477 | ###### 3.2.5.11.1.1 | X |  |  |  | 4.2.5.11.1.1 |  |
| PD3478 | JLTV FoV shall have front-end / underbody protection and ruggedized external components to prevent damage to vehicle lights, body, and engine components, from brush, tree limbs or other entangling material and to reduce/prevent water/mud ingestion into the radiator area while traveling cross-country at cross-country speeds. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3481 | ###### 3.2.5.11.1.2 |  |  |  | X | 4.2.5.11.1.2 |  |
| PD3484 | Given clean heat exchanger(s), the required cooling shall be provided continuously at all ambient conditions between -50 Deg F and 130 Deg F, full radiant heat load and at 750mm HG: |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3485 | a. Under all operating conditions within the maximum tractive effort (TE) to weight vs. speed defined by: |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3486 | i. TE/projected vehicle combat loaded weight = 0.6 to maximum forward speed. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3487 | ii. Gear engaged idle and high idle operations |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3488 | b. For all conditions specified under the Braking and Speed on Grade Section of this ATPD. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3489 | c. Including the capability of cooling the ancillary vehicle power (hydraulic, electrical, electronics, pneumatic, etc.) losses for the JLTV application as a minimum, while satisfying (a) and (b). Integration of the cooling requirements of the vehicle power system is optional, and is dependent upon due consideration to the volume, weight, and other penalties imposed on the propulsion system by such cooling requirements. |  |  |  | X | Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3490 | ##### 3.2.5.11.2 Fan Clutch. | X |  |  | X | 4.2.5.11.2 |  |
| PD3491 | If a fan clutch is used, a positive lockup shall be provided in case of a clutch or a control system failure. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD3492 | ###### 3.2.5.11.2.1 |  |  |  | X | 4.2.5.11.2.1 |  |
| PD3493 | The cooling fan shall be designed so that it will not experience aerodynamic stall with a 30% cooler face area blockage. |  |  |  | X | Analysis shall be used to verify compliance with this requirement. | 3 |
| PD3494 | ###### 3.2.5.11.2.2 | X |  |  |  | 4.2.5.11.2.2 |  |
| PD3495 | The fan shall be equipped with a thermostatic control so that fan use is minimized when not required for cooling. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3496 | ##### 3.2.5.11.3 Oil Filtration. | X |  |  |  | 4.2.5.11.3 |  |
| PD3499 | ###### 3.2.5.11.3.1 | X |  |  |  | 4.2.5.11.3.1 |  |
| PD3501 | A full flow type oil filter system with integral emergency bypass, IAW the engine manufacturer's specification, shall be furnished to ensure maximum engine protection. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6761 | ###### 3.2.5.11.3.2 | X |  |  |  | 4.2.5.11.3.2 |  |
| PD6762 | Spin-on type oil filters shall be used for engine oil filtration. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3504 | ##### 3.2.5.11.4 Engine Speed Control. | X | X | X |  | 4.2.5.11.4 |  |
| PD3506 | ###### 3.2.5.11.4.1 |  | X |  |  | 4.2.5.11.4.1 |  |
| PD3507 | The tactical idle control shall operate only when the vehicle is in park or neutral and automatically disengage when the vehicle is placed in gear. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD3508 | ###### 3.2.5.11.4.2 | X |  |  |  | 4.2.5.11.4.2 |  |
| PD3509 | Tamper resistant means shall be provided to limit the maximum engine speed to the engine manufacturer's maximum recommended operating speed. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 2 |
| PD3510 | ###### 3.2.5.11.4.3 |  |  | X |  | 4.2.5.11.4.3 |  |
| PD3511 | The accelerator control system shall conform to FMVSS 571.124. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3526 | ##### 3.2.5.11.5 Exhaust System. |  |  | X | X | 4.2.5.11.5 |  |
| PD3527 | The exhaust system shall conform to FMCSR 393.83. The exhaust system as installed shall be gas tight to prevent the accumulation of exhaust gas in the occupied areas in accordance with best commercial practice. The exhaust system shall be configured to prevent entry of water. Exhaust mufflers and exhaust pipes shall be made of corrosion resistant stainless steel and shall be furnished with adequate guards/shielding to prevent personnel contact. The exhaust system shall be located to minimize the danger of ignition of fuel from leaks or overflow of fuel. The contractor shall minimize visible profile of the exhaust system from thermal detection devices to the maximum extent possible. |  |  | X | X | Certification and Analysis shall be provided to demonstrate compliance with the Section 3 requirement. | 3 |
| PD3530 | ##### 3.2.5.11.6 Gear Train. |  | X | X |  | 4.2.5.11.6 |  |
| PD3531 | The gear train shall be lubricated with standard military products as indicated in the Fuels and Lubricants section of this document. When utilizing permanently sealed components, they shall have a life of not less than 100,000 miles of operation over OMS/MP. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3532 | ###### 3.2.5.11.6.1 Transmission (If Applicable). |  | X |  |  | 4.2.5.11.6.1 |  |
| PD3533 | The transmission shall be automatic and shall have a gear range capable of meeting the performance specified in this Purchase Description |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3534 | 3.2.5.11.6.1.1 |  | X |  |  | 4.2.5.11.6.1.1 |  |
| PD3535 | The transmission, shall include the following: |  |  |  |  |  | 3 |
| PD3536 | a. A downshift inhibitor system that prevents driver shift control action from over-speeding or damaging engine, transmission, or drive train components. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3537 | b. Starter Interlock. The engine starter shall be inoperative when the transmission shift lever is in a forward or reverse drive position. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD3538 | c. A means to manually select and identify the gear range. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD3539 | d. A neutral interlock shall be provided which shall allow the truck to start only in park or neutral. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3540 | ###### 3.2.5.11.6.2 Transfer Case (If Applicable). |  | X |  |  | 4.2.5.11.6.2 |  |
| PD3541 | If utilized, the transfer case shall be installed which has the ability to provide torque proportioning full time all-wheel drive. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3542 | 3.2.5.11.6.2.1 |  | X |  |  | 4.2.5.11.6.2.1 |  |
| PD3543 | If a single speed transfer case is used, it shall contain a planetary differential that shall provide full time all-wheel drive. A multi-speed transfer case, if used, must possess a low range speed of at least 20 mph. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 2 |
| PD3544 | ##### 3.2.5.11.7 Steering. | X | X | X |  | 4.2.5.11.7 |  |
| PD3545 | Power steering shall be furnished and have full limit steer when the vehicle is stationary while at GVW and GCVW. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3546 | ###### 3.2.5.11.7.1 |  | X |  |  | 4.2.5.11.7.1 |  |
| PD3547 | In the event power assist is lost, the system shall be manually steerable and capable of being brought to a safe stop. |  | X |  |  | The JLTV shall be tested IAW AVTP 03-30W to verify compliance with the Section 3 requirement. | 2 |
| PD3548 | ###### 3.2.5.11.7.2 |  | X |  |  | 4.2.5.11.7.2 |  |
| PD3549 | The system at all payload conditions shall meet the requirement of the TECOM/CSTA test methodology for Dead Engine Steering Test, "Y-Course", derived from the Allied Vehicle Testing Publication (AVTP), No. 03-30WT. Throughout its entire steering arc (lock-to-lock) and including maximum tire side wall deflection, no components of the steering system shall contact or bind. |  | X |  |  | The JLTV shall be tested to verify compliance with the Section 3 requirement. | 3 |
| PD3552 | ###### 3.2.5.11.7.3 | X |  |  |  | 4.2.5.11.7.3 |  |
| PD3553 | The steering wheel shall be capable of being locked in a neutral position with either a standard padlock A-A-59487 (Part Identification Number AA59487-1BC) or chain. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6933 | ###### 3.2.5.11.7.4 |  |  | X |  | 4.2.5.11.7.4 |  |
| PD6934 | The JLTV FoV steering columns and controls shall be designed in accordance with FMVSS 203/204 to minimize chest, head, neck and facial injuries to the driver and any occupants of the passenger compartment. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3583 | ##### 3.2.5.11.8 Engine EPA Emissions Requirements. |  | X | X |  | 4.2.5.11.8 |  |
| PD3584 | The JLTV FOV is not subject to EPA Motor Vehicle Heavy Duty Diesel Exhaust emission standards or the EPA Non-road exhaust emission standards since the vehicle will contain permanent armor protection. This determination is IAW 40 CFR, Sections 85.1703, 89.908 and 1068.225. |  |  |  |  |  | 3 |
| PD3587 | ###### 3.2.5.11.8.1 |  |  | X |  | 4.2.5.11.8.1 |  |
| PD3588 | The JLTV shall meet National Security Exemption labeling requirements IAW EPA regulations. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3589 | ###### 3.2.5.11.8.2 |  |  | X |  | 4.2.5.11.8.2 |  |
| PD3590 | The engine provided shall be of the latest engine technology that shall meet all performance requirements specified in this PD while operating with turbine fuel IAW with MIL-DTL-83133(JP-8) as the primary fuel (or MIL-DTL-5624, JP-5), which may have up to 3,000 ppm sulfur. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3591 | ###### 3.2.5.11.8.3 |  |  | X |  | 4.2.5.11.8.3 |  |
| PD3592 | Pollution control technologies that are affected by the sulfur level of the JP-8 fuel either in maintenance or life expectancy shall not be used, e.g., Exhaust Gas Re-circulation (EGR), NOX traps, particulate traps, catalytic converters, etc. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3593 | ###### 3.2.5.11.8.4 |  | X | X |  | 4.2.5.11.8.4 |  |
| PD3594 | The engine shall also be capable of running and meeting performance requirements using DF-2 fuel per ASTM D 975 or CID A-A-52557. |  | X | X |  | The JLTV shall be tested and certified to verify compliance with the Section 3 requirement. | 3 |
| PD6744 | #### 3.2.5.12 Fuel System. | X |  | X | X | 4.2.5.12 |  |
| PD6745 | ##### 3.2.5.12.1 |  |  | X |  | 4.2.5.12.1 |  |
| PD6747 | The fuel system shall meet the requirements of FMCSR 393 and incorporate the Standard Army Refueling System (SARS) components. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6935 | ###### 3.2.5.12.1.1 |  |  | X |  | 4.2.5.12.1.1 |  |
| PD6936 | The fuel system shall meet the requirements of FMVSS 301-Fuel System Integrity. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6746 | ##### 3.2.5.12.2 | X |  |  |  | 4.2.5.12.2 |  |
| PD6748 | The fuel system shall include an automatic water separator. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6749 | ##### 3.2.5.12.3 Fuel Tanks. | X |  | X | X | 4.2.5.12.3 |  |
| PD6750 | The vehicle shall be equipped with corrosion resistant fuel tank (s) of sufficient volume to permit vehicle operations for the range specified in the vehicle specific annex's. |  |  |  | X | Analysis shall be provided to demonstrate compliance with this requirement. | 3 |
| PD6751 | ###### 3.2.5.12.3.1 | X |  |  |  | 4.2.5.12.3.1 |  |
| PD6752 | A shut-off valve between the tanks, if more than one tank, shall be furnished. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6753 | ###### 3.2.5.12.3.2 |  |  | X |  | 4.2.5.12.3.2 |  |
| PD6754 | Metal fuel lines shall be galvanically insulated from adjacent surfaces. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD6755 | ###### 3.2.5.12.3.3 | X |  |  |  | 4.2.5.12.3.3 |  |
| PD6756 | Mounting straps used to secure metal tanks to fream/chassis shall be insulated. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6757 | ###### 3.2.5.12.3.4 | X |  |  |  | 4.2.5.12.3.4 |  |
| PD6759 | Fuel tank(s) shall be provided with drain plug(s) and safety type tank filler caps, captive chained to filler neck strainers, which are accessible and removable by personnel wearing arctic mittens. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD6758 | ###### 3.2.5.12.3.5 | X |  |  |  | 4.2.5.12.3.5 |  |
| PD6760 | Fuel tank ports must be a minimum of 2.25 inches (5.7 cm) diameter, and shall be compatible with NATO dispensing nozzles having a nominal outside diameter of 2 inches (51 mm). | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD7309 | ###### 3.2.5.12.3.6 | X |  |  |  | 4.2.5.12.3.6 |  |
| PD7310 | External fuel covers shall visually blend in with the body of the vehicle to preclude the covers being identified as potential targets. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3554 | #### 3.2.5.13 Hydraulics System (if applicable). | X |  | X | X | 4.2.5.13 |  |
| PD3555 | The hydraulic system shall have provisions for operating hydraulic equipment external to the vehicle and include such provisions for future use (O). | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3556 | ##### 3.2.5.13.1 | X |  |  | X | 4.2.5.13.1 |  |
| PD3557 | Removable caps or plugs shall cover “quick disconnect” hydraulic fittings for attachment of the external hydraulic system to prevent dirt or other foreign objects from contaminating the system. | X |  |  |  | Inspection shall be conducted to verify compliance with the Section 3 requirement. | 3 |
| PD3558 | ##### 3.2.5.13.2 Hydraulic Reservoir. | X |  |  | X | 4.2.5.13.2 |  |
| PD3559 | Vehicle shall have a hydraulic reservoir of sufficient capacity to operate vehicle systems and auxiliary equipment for all mission types. | X |  |  | X | The JLTV shall be inspected and analyzed to verify compliance with the Section 3 requirement. | 3 |
| PD3560 | ###### 3.2.5.13.2.1 | X |  |  | X | 4.2.5.13.2.1 |  |
| PD3561 | Reservoir shall be provided with at least the following: |  |  |  |  |  |  |
| PD3562 | a. Filter(s) shall be readily accessible for cleaning or replacement without draining the reservoir in all hydraulic circuits. Bypasses shall be furnished where necessary, to protect filters and ensure components are adequately lubricated during cold temperature operation. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD3563 | b. Baffles to preclude foaming. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD3564 | c. Dip stick, sight gage, and pressure vented type filler cap of no less than 5 psi. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD3565 | d. Access size to allow manual cleaning of the reservoir. | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD3566 | e. Reservoir shall allow for hydraulic maintenance without draining the systems (objective). | X |  |  |  | The JLTV shall be inspected to verify compliance with the Section 3 requirement. | 3 |
| PD3567 | f. Hydraulic system cooler-if required. | X |  |  | X | The JLTV shall be analyzed and inspected to verify compliance with the Section 3 requirement, if cooler is required. | 3 |
| PD3568 | ##### 3.2.5.13.3 Hydraulic Hoses & Fittings. |  |  | X |  | 4.2.5.13.3 |  |
| PD3569 | High-pressure hoses and fittings shall conform to the requirements of SAE J516, SAE J517 and SAE J343. Self-sealing quick disconnect hydraulic couplings shall be provided for all hydraulic system connectors required to be removed for engine, transmission or transfer case removal/replacement. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3574 | #### 3.2.5.14 Environmental Quality. |  |  | X |  | 4.2.5.14 |  |
| PD3575 | ##### 3.2.5.14.1 |  |  | X |  | 4.2.5.14.1 |  |
| PD3576 | Hazardous Materials such as asbestos, beryllium, radioactive materials, hexavalent chromium, (electroplating and coatings), cadmium (electroplating), mercury, or other highly toxic or carcinogenic materials, as defined in 29 CFR 1910.1200, shall not be used in the manufacture, assembly, maintenance or sustainment of the JLTV without prior approval from the government. The JLTV shall be designed in such a way that the use of benzene, N-butyl alcohol, toluene, dichloromethane, and xylene are not required during maintenance or sustainment of the vehicle. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3577 | ##### 3.2.5.14.2 |  |  | X |  | 4.2.5.14.2 |  |
| PD3578 | Class I and Class II Ozone Depleting Substances shall not be used. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3579 | ##### 3.2.5.14.3 Lead. |  |  | X |  | 4.2.5.14.3 |  |
| PD3580 | Lead shall not be used without prior approval of the government. The use of lead solder may be approved for electrical components where a suitable alternative is not available. Lead-acid batteries may be used without approval from the government. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3581 | ##### 3.2.5.14.4 Applicability. |  |  | X |  | 4.2.5.14.4 |  |
| PD3582 | Environmental requirements shall apply to any components/parts purchased through a subcontractor/vendor or OEM parts, as well as manufactured parts. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |
| PD3597 | #### 3.2.5.15 DISPOSAL. |  |  | X |  | 4.2.5.15 |  |
| PD3598 | The system shall be designed such that the user shall have the ability to dispose of the system in full compliance with applicable U.S., foreign and international environmental quality laws and regulations. |  |  | X |  | Certification shall be provided to verify compliance with the Section 3 requirement. | 3 |

ACRONYMS

| Ao | Operational Availability |
| --- | --- |
| ABS | Anti-lock Braking System |
| AC | Alternating Current |
| ADU | Auxiliary Display Unit |
| AFES | Automatic Fire Extinguishing Systems |
| AFU | Arming/Firing Unit |
| ALGL | Advanced Lightweight Grenade Launcher |
| AR | Army Regulation |
| ATPD | Automotive Tank Purchase Description |
| BC | Battle Command |
| BCT | Brigade Combat Team |
| BII | Basic Issue Items |
| BIT | Built In Test |
| BITE | Built In Test Equipment |
| C2 | Command and Control |
| C2OTM | Command and Control On The Move |
| C4I | Command, Control, Communications and Computers Intelligence |
| C4ISR | Command, Control, Communications, Computers, Intelligence, Surveillance, and Reconnaissance |
| C4ISR/EW | Command, Control, Communications, Computers, Intelligence, Surveillance, and Reconnaissance/ Electronic Warfare |
| CAISI | Combat Service Support Automated Information System |
| CAMC2 | Combined Army and Marine Command and Control |
| CARC | Chemical Agent Resistant Coating |
| CBM+ | Condition Based Maintenance Plus |
| CBRN | Chemical, Biological, Radiological, and Nuclear |
| CC | Crew Compartment |
| CCSI | Common CBRN Sensor Interface |
| CDD | Capability Development Document |
| CECOM | Communications Electronics Command |
| CG | Center of Gravity |
| CID | Combat Identification |
| CLOE | Common Logistics Environment |
| CNR | Combat Net Radio |
| COMSEC | Communication Security |
| CONDOR | Command and Control On-the-Move Network, Digital Over the Horizon Relay |
| CONUS | Continental United States |
| COTS | Commercial Off the Shelf |
| CREW | Counter-Radio Controlled Improvised Explosive Device-Electronic Warfare |
| CROWS | Common Remotely Operated Weapons Station |
| CRT | Combat Repair Team |
| CRT | Cathode Ray Tube |
| CT | Companion Trailer |
| CTA | Common Table of Allowances |
| CTIS | Central Tire Inflation System |
| CW | Curb Weight |
| CWS | Collision Warning System |
| DAGR | Defense Advanced GPS Receiver |
| DC | Direct Current |
| DEW | Directed Energy Weapon |
| DIACAP | DOD Information Assurance Certification and Accreditation Process |
| DODISS | Department of Defense Index of Specifications and Standards |
| DOT | Department of Transportation |
| DVE | Drivers Vision Enhancement |
| E3 | Electromagnetic/Electronic Environmental Effects |
| ECC | Essential Combat Configuration |
| ECU | Electronic Control Unit |
| EFF | Essential Function Failure |
| EKMS | Key Management System/Electronic Key Management System |
| EM | Electromagnetic |
| EMC | Electromagnetic Compatibility |
| EMI | Electromagnetic Interference |
| EMP | Electromagnetic Pulse |
| EPA | Environmental Protection Agency |
| EPLRS | Enhanced Position Location and Reporting System |
| ESC | Electronic Stability Control |
| EW | Electronic Warfare |
| FBCB2 | Force XXI Battle Command Brigade and Below |
| FCS | Future Combat Systems |
| FDSC | Failure Definition and Scoring Criteria |
| FLIR | Forward-Looking Infrared |
| FMC | Fully Mission Capable |
| FMCSR | Federal Motor Carriers Safety Regulations |
| FMTV | Family of Medium Tactical Vehicles |
| FMVSS | Federal Motor Vehicle Safety Standards |
| FoS | Follow On Support |
| FoV | Family of Vehicles |
| GB-GRAM | Ground-Based GPS Receiver Applications Module |
| GCCS-A | Global Command and Control System - Army |
| GCVW | Gross Combined Vehicle Weight |
| GFE | Government Furnished Equipment |
| GFI | Government Furnished Information |
| GIC | Gavarit Internation de Chargement (guage shown in mil-std-1366) |
| GPK | Gunner Protection Kit |
| GPM | Gallons per Minute |
| GPS | Global Positioning System |
| GUI | Graphical User Interface |
| GVW | Gross Vehicle Weight |
| HAZMAT | Hazardous Materiel |
| HEMP | High-Altitude Electromagnetic Pulse |
| HEMTT | Heavy Expanded Mobility Tactical Truck |
| HF | High Frequency |
| HMI | Human Machine Interface |
| HW | Hardware |
| IA | Information Assurance |
| IAD | Immune at Any Distance |
| IAW | In Accordance With |
| IBA | Individual Body Armor |
| ICD | Interface Control Documents |
| ICS | Integrated Computer System |
| IED | Improvised Explosive Device |
| IETM | Interactive Electronic Technical Manuals |
| IFF | Identifying Friend or Foe |
| INLS | Improved Navy Ligherage System |
| ISO | International Organization for Standardization |
| ITAS | Improved Target Acquisition System |
| JBC-P | Joint Battle Command - Platform |
| JCR | Joint Capability Release |
| JHSV | Joint High Speed Vessel |
| JLTV | Joint Light Tactical Vehicle |
| JTF | Joint Task Force |
| JTRS | Joint Tactical Radio System |
| KPH | Kilometers Per Hour |
| LASH | Lighter Aboard Ship |
| LCAC | Landing Craft Air-Cushioned |
| LCM | Landing Craft Medium |
| LCU | Landing Craft Utility |
| LDSS | Logistics Decision Support System |
| LED | Light Emitting Diode |
| LMR | Land Mobile Radio |
| LMS | Lightweight Multipurpose Shelter |
| LPD | Low Probability of Detection |
| LPI | Low Probability of Intercept |
| LRAS3 | Long Range Advanced Scout Surveillance System |
| LRM | Line Replaceable Module |
| LRU | Line-Replaceable Unit |
| LSV | Logistic Support Vehicle |
| LVAD | Low Velocity Aerial Delivery |
| LVOSS | Light Vehicle Obscuration Smoke System |
| LW | Land Warrior |
| MANPRINT | Manpower and Personal Integration |
| MaxTTR | Maximum Time to Repair |
| MBCOTM | Mounted Battle Command On the Move |
| MC4 | Medical Communications for Combat Casualty Care |
| MCMT | Mean Corrective Maintenance Time |
| MHE | Material Handling Equipment |
| MIL-HDBK | Military Handbook |
| MIL-STD | Military Standard |
| MMBHMF | Mean Miles Between Hardware Mission Failure |
| MMBOMF | Mean Miles Between Operational Mission Failure |
| MOPP | Mission Oriented Protective Posture |
| MP | Mission Profile |
| MPF | Maritime Pre-positioned Fleet |
| MPH | Miles Per Hour |
| MRE | Meals Ready to Eat |
| MRV | Mission Role Variant |
| MTMC-TEA | Military Traffic Management Command Transportation Engineering Agency |
| MTS | Movement Tracking System |
| MTS-ES | Movement Tracking System - Enhanced Software |
| MTTR | Mean Time to Repair |
| MTVR | Medium Tactical Vehicle Replacement |
| MW | Mounted Warrior |
| NATO | North American Treaty Organization |
| NBC | Nuclear, Biological, and Chemical |
| NDI | Non-Developmental Item |
| NM | Nautical Miles |
| NRMM | NATO Reference Mobility Model |
| NSA | National Security Agency |
| NSN | National Stock Number |
| NTSC | National Television Systems Committee |
| NVG | Night Vision Goggles |
| OCONUS | Outside the Continental United States |
| ODU | Operator Display Unit |
| OEM | Original Equipment Manufacturer |
| OMS | Operational Mode Summary |
| OMS/MP | Operational Mode Summary/Mission Profile |
| OPTEMPO | Operating/Operations Tempo |
| OS | Operating System |
| OVM | On-Vehicle Maintenance |
| PKI | Public Key Infrastructure |
| PLC | Power Line Carrier |
| PMC | Preventative Maintenance Checks |
| PMCS | Preventative Maintenance Checks and Services |
| POL | Petroleum, Oil and Lubricant |
| psi | Pounds per square inch |
| PS-MRS | Platform Soldier-Mission Readiness System |
| RAM | Reliability, Availability and Maintainability |
| RAMD | Reliability, Availability, Maintainability, and Durability |
| RCU | Remote Control Unit |
| RF | Radio Frequency |
| RFID | Radio Frequency Identification |
| RMS | Root Mean Squared |
| RORO | Roll-on/Roll-off |
| RPG | Rocket Propelled Grenade |
| RPSTL | Repair Parts and Special Tools Lists |
| RWS | Remote Weapons Station |
| SA | Situational Awareness |
| SA | System Abort |
| SAE | Society of Automotive Engineers |
| SAM-E | Standard Army Maintenance Systems - Enhanced |
| SATCOM | Satellite Communications |
| SBU | Sensitive but Unclassified |
| SEABEE | Construction Battalion (US Navy) |
| SEP | Spherical Error Probable |
| SINCGAR | Single Channel Ground to Air Radio |
| SICPS | Standard Integrated Command Post Shelter |
| SLI | Starting, Lighting, and Ignition |
| SOC | State of Charge |
| STAMIS | Standard Army Management Information System |
| STANAG | NATO Standardization Agreement |
| SW | Software |
| TIC | Toxic Industrial Chemical |
| TIM | Toxic Industrial Material |
| TM | Technical Manual |
| TMDE | Test Maintenance and Diagnostic Equipment |
| TRADOC | Training and Doctrine Command |
| USMC | United States Marine Corp |
| VAC | Volts Alternating Current |
| VAP | Virtual Access Point |
| VCI | Vehicle Cone Index |
| VCS | Vehicle Computing System |
| VDC | Volts Direct Current |
| VoIP | Voice Over Internet Protocol |